Specification for: ______________ Fire Department Pumper 


INTENT OF SPECIFICATIONS
It shall be the intent of these specifications to provide a complete apparatus equipped as hereinafter and as specified. With a view to obtaining the best results and the most acceptable apparatus for service in the __________________ Fire Department, these specifications cover only the general requirements as to the type of construction and tests to which the apparatus must conform, together with certain details as to finish, equipment and appliances with which the successful bidder shall conform. Minor details of construction and materials where not otherwise specified are left to the discretion of the contractor, who shall be solely responsible for the design and construction for all features. The manufacturer shall provide loose equipment only when specified by the customer. The National Fire Protection Association (NFPA) Standard 1901, current edition, unless otherwise specified as requested by the customer in these specifications, shall prevail.

The apparatus must meet all NFPA, DOT, ICC, AE, TRA, FMVSS and local state Motor Vehicle Requirements.

It is required that the apparatus be manufactured to current NFPA edition standards, all NFPA equipment (loose equipment) not specified in the specifications will not be provided by the contractor.

Bids shall only be considered from companies that have an established reputation in the field of fire apparatus construction that have been in business and construction for a minimum of twenty-five (25) years.

The bidder of the apparatus herein specified; shall be wholly owned (100%) and managed by a Company, Corporation, and/or Parent Company that is wholly based, and permanently resides in the United States of America.

The Company, Corporation, and/or Parent Company and all assets belonging to such; shall be wholly owned and managed (100%) by the entities specified above.

The bidder shall state the location of the manufacturing facility where the apparatus is to be built and the location of the parent company if a subsidiary of a manufacturer.

The bidder shall provide satisfactory evidence of their ability to construct the apparatus specified in the bidders manufacturing facilities.

The bidder’s representation shall state the length of time representing the manufacturer of specified apparatus.

Due to the severe service requirements the department will impose on the apparatus as specified, each bidder shall provide a list of at least six (6) departments in which similar apparatus utilizing the brand of chassis proposed have been in service for over one year. This list shall include contact names and phone numbers.
The bid shall be accompanied by a set of “Contractor’s Specifications” consisting of a detailed description of the apparatus being furnished under this contract which conform. Computer runoff sheets are not acceptable as “Contractor’s Specifications”. Item compliance shall be indicated in the “Yes/No” column of each item by all Bidders. Note: Each bidder shall submit their bid in the same sequence as these specifications to allow the department to easily compare. {No Exceptions}
These specifications shall indicate size, type, model and make of all component parts and equipment. {No Exceptions}
QUALITY AND WORKMANSHIP
The design of the Apparatus shall embody the latest approved automotive engineering practices.

The workmanship must be of the highest quality in its respective field. Special consideration will be given to the following points: Accessibility of the various units, which require periodic maintenance, ease of operation (including both pumping and driving) and symmetrical proportions.

Construction shall be rugged and ample safety factors shall be provided to carry loads as specified and to meet both on and off-road requirements and to speed conditions as set forth under “Performance tests and requirements”.

Welding shall be employed in the assembly of the apparatus in a manner that will not prevent the ready removal of any component part for service or repair, with apparatus bodies of bolt together design not being acceptable.

All steel welding shall follow American Welding Society requirements for AWS D1.1:2012 Structural Welding Code for welding steel structural assemblies. All aluminum welding shall follow American Welding Society requirements for AWS D1.2/D1.2M:2003 Structural Welding Code for any type structure made from aluminum structural alloys. All sheet metal welding shall follow American Welding Society AWS D9.1M/D9.1:2006 Structural Welding code for Arc/Braze requirements of non-structural materials. All pressure pipe welding shall follow American Society of Mechanical Engineers ASME IX/ ASME B31:2010 requirements to the qualification of procedures in welding and brazing, in accordance with the ASME Boiler and Pressure Vessel Code and the ASME B31 Code for Pressure Piping. Flux core arc welding to use alloy rods, type 7000, American Welding Society AWS standards A5.20-E70T1. The manufacturer shall be required to have an American Welding Society certified welding inspector in plant during testing operations within working hours to monitor weld quality.

Employees classified as welders shall be tested and certified to meet American Welding Society and American Society of Mechanical Engineers welding codes.

DELIVERY
The bidder shall provide the number of calendar days from the date the bid is awarded to the delivery of the completed unit. If a range of days is quoted, there shall be no more than 20 days between the quoted number of days. No exceptions.
A qualified delivery engineer representing the contractor shall deliver the apparatus and instruct the Fire Department personnel in the proper operation, care and maintenance of the equipment delivered.

PERFORMANCE TESTS AND REQUIREMENTS

A road test shall be conducted with the apparatus fully loaded to its estimated in-service weight and shall be capable of the following performance while on dry paved roads that are in good condition and for a continuous run of ten (10) miles or more, during which time the apparatus shall show no loss of power or overheating. The transmission drive shaft or shafts and rear axles shall run quietly and be free from abnormal vibration or noise throughout the operating range of the apparatus. The successful bidder shall furnish a Weight Certificate showing weights on front axle, rear axles and total weight for the completed apparatus at time of delivery.

· The apparatus shall be capable of accelerating to 35 MPH from a standing start within 25 seconds on a level concrete highway without exceeding the maximum governed RPM of the engine.

· The apparatus, fully loaded, shall be capable of obtaining a minimum top speed of 50 MPH on a level dry concrete highway with the engine not exceeding its governed RPM (fully loaded).

· The service brakes shall be capable of stopping a fully loaded vehicle in 35ft at 20 mph on a level concrete highway. The air brake system shall conform to Federal Motor Vehicle Safety Standards (FMVSS) 121.

· The apparatus, when fully loaded, shall have not less than 25 percent or more than 50 percent of the weight on the front axle, and not less than 50 percent nor more than 75 percent on the rear axle.

· The contractor shall have the Underwriter’s Laboratories Incorporated conduct the tests of the apparatus as in accordance with standard practices required by the Underwriter Laboratories Incorporated (Guide for the Certification of Fire Department Pumper latest edition). A copy of all tests shall accompany the Apparatus. (For apparatus sold within Canadian ULC S515 latest revision shall prevail).

· The contractor shall furnish copies of the Pump Manufacturer’s Certification of hydrostatic test, the Engine Manufacturer current certified brake horsepower curve, and the Manufacturer’s record of pumper construction details when delivered.

INFORMATION REQUIRED
The manufacturer shall supply at time of delivery, a complete operation and maintenance manual covering the completed apparatus as delivered.

A Fire Apparatus Safety Guide published by Fire Apparatus Manufacturer's Association shall be provided with the apparatus upon delivery. This manual includes essential safety information for fire fighters, fire chiefs, apparatus mechanics, and fire department safety officers. The guide is applicable to municipal, wildland, and airport firefighting apparatus manufactured on either custom or commercial chassis. 
A permanent plate shall be mounted in the driver's compartment to specify the quantity and type of the following fluids used in the vehicle: Engine oil, engine coolant, and chassis transmission fluid, pump transmission lubrication fluid and drive axle lubrication fluid.

The manufacture shall supply the final certification of GVWR and GAWR on a nameplate affixed to the vehicle.

A permanent plate in the driver's compartment shall be installed, specifying the seating capacity of the enclosed cab.

Signs that state "OCCUPANTS MUST BE SEATED AND BELTED WHEN APPARATUS IS IN MOTION" shall be provided and will be visible from each seated position. An accident prevention sign shall be located at the rear step area of the apparatus. It shall warn all personnel that standing on the step while apparatus is in motion shall be prohibited.

A nameplate indicating the chassis transmission shift selector position to be used when pumping shall be provided in the driving compartment and located so that it can be easily read from the driver's position.

LIABILITY
The bidder, if their bid is accepted, shall defend any and all suits and assume all liability for the use of any patented device or article forming part of the apparatus or any appliance provided under the contract.

GENERAL CONSTRUCTION

The apparatus shall be designed with due consideration to distribution of load between the front and rear axles, so that all specified equipment, including filled water tank, a full complement of personnel and fire hose will be carried without injury to the apparatus. Weight balance and distribution shall be in accordance with the recommendations of the National Fire Protection Agency (NFPA) 1901, current edition.

The apparatus shall be designed so that all recommended daily maintenance checks can be performed easily by the operator without the need for hand tools. Apparatus components that interfere with repair or removal of other major components must be attached with fasteners (cap, screws, nuts, etc.) so that the components can be removed and installed with normal hand tools. These components must not be welded or otherwise permanently secured into place.

The GAWR and GVWR of the chassis shall be adequate to carry the fully equipped apparatus including all tanks filled, the specified hose load, unequipped personnel weight, ground ladders and a miscellaneous equipment allowance per NFPA criteria. It shall be the responsibility of the purchaser to provide the contractor with the weight of equipment to be carried if it is in excess of the allowance as set forth by NFPA.

The unequipped personnel weight shall be calculated at 250 lbs. per person times the maximum number of persons to ride on the apparatus.

The height of the fully loaded vehicle's center of gravity shall not exceed the chassis manufacturer's maximum limit.

The front to rear weight distribution of the fully loaded vehicle shall be within the limits set by the chassis manufacturer. The front axle loads shall not be less than the minimum axle loads specified by the chassis manufacturer, under full loads and all other loading conditions.

The difference in weight on the end of each axle, from side to side, when the vehicle is fully loaded and equipped shall not exceed 7 percent.

The apparatus shall be so designed that the various parts are readily accessible for lubrication, inspection, adjustment and repair.

Where special tools manufactured or designed by the contractor and are required to provide routine service on any component of the apparatus built or supplied by the contractor, such tools shall be provided with the apparatus.

EXCEPTIONS TO SPECIFICATIONS

The following specifications shall be strictly adhered to. Exceptions shall be allowed if they are equal to or superior to that as specified and provided they are listed and entirely explained on a separate page entitled "Exceptions to Specifications". The exceptions list to refer to specification page number and paragraph.

Proposals taking total exception to specifications or total exception to certain parts of the specifications such as Electrical Systems, Chassis, Body or Pump, will not be accepted.

Prototype units will not be acceptable. Apparatus shall be inspected upon completion for compliance with specifications.

Deviations will not be tolerated and will be cause for rejection of apparatus unless they were originally listed in bidder’s proposal and accepted in writing by the department.

If the bidder takes an exception, on the exception page, the bidder must state an option price to bring their specifications into full compliance with the Department specifications.

Failure to provide this information shall be cause to reject the proposal as being non-responsive. {No Exceptions}
Copied or run off sheets of these specifications shall be unacceptable and the bid will be rejected no exceptions.

PURCHASER'S RIGHTS
The Purchaser reserves the right to accept or reject any or all bids as it deemed in their best interests.

BID/PROPOSAL DRAWINGS

For purposes of evaluation, the bidder shall provide a drawing illustrating, but not limited to, the overall dimensions, wheelbase, and overall length of the actual proposed apparatus and other specified equipment, shall be required to be included with the bidder's proposal package. No exceptions.
The drawings shall be large "D" size (minimum 24.00 inches x 36.00 inches) and shall be in color.

APPROVAL/PRE-CONSTRUCTION DRAWINGS
After the award of the bid, the contractor shall provide detailed colored engineering drawings including, but not limited to, the overall dimensions, wheelbase, and overall length of the proposed apparatus for use during the pre-construction conference.

The drawings shall include, but shall not be limited to the right, left, top, front and rear views of the apparatus.

The Fire Department shall sign the final approval drawing.

AS BUILT DRAWING PDF

The contractor shall provide to the Fire Department a “PDF” type, as built sales drawing.
BID SECURITY
Each bidder must submit a bid bond or a cashier's check with his or her proposal for the amount of ten percent (10%) of the bid price of the proposal submitted.
SERVICE REQUIREMENTS

Each bidder shall supply, with their proposal, detailed information on the bidder's ability to perform routine and emergency service on the apparatus after delivery. 

Detailed information and photos shall be provided on the bidder’s service facilities, EPA approved paint facilities, number of EVT certified mechanics, mobile service vehicles, and the type and nature of repair work the bidder is able to provide at their dealership. 

Bidder shall state the number of miles from the __________________ Fire Department’s facility to the nearest fully staffed repair facility operated by the bidder. Sub-contracting to independent third-party service providers shall not constitute compliance with this requirement. It is the intent of the __________________ Fire District to assure that parts and service are readily available for the equipment specified. Dealership service capabilities shall be one of the criteria for award of this contract.

SINGLE SOURCE MANUFACTURER
Bids shall only be accepted from a single source apparatus manufacturer.
The definition of single source manufacturer is company that designs and manufactures their products utilizing an approach that includes complete product integration, including the apparatus chassis, cab, and body modules being constructed, assembled, and tested on company premises only.

Warranties qualified to the chassis and body design construction (excluding vender component warranties such as engine, axles, transmission, and pumps, etc.) will be from a single source manufacturer and not separated between manufacturers (i.e. body and chassis). The bidder shall provide evidence of maintaining compliance to this requirement.

TAG-ON ORDERS-COOPERATIVE PURCHASING
Other fire departments, metropolitan regions, or municipalities, may purchase apparatus and equipment similar to the Apparatus and Equipment that is the subject of this Contract. The following terms shall apply to any such tag-on orders:

(a) Changes - The Manufacturer's intention is to make available to others, tag-on orders utilizing the same specification as the Apparatus and Equipment that is the subject of this Contract in order to provide favorable pricing and lead-times to other buyers due to having such specification fully engineered. The Manufacturer recognizes however that each additional buyer may have unique requirements that must be accommodated; and in this regard, limited changes will be permitted. Such changes will be captured in the pre-construction meeting and the price of any tag-on unit adjusted accordingly.

(b) Term – Tag-on orders may be placed for a term of one year after the Effective Date of this Contract.

(c) Escalation - The Manufacturer reserves the right to adjust the price of any tag-on order if material costs escalate during the term of this Contract, changes in regulations become effective (for example EPA, NFPA or other), or the tag-on order would cross a model year.

(d) Acceptance – The Manufacturer reserves the right to accept or reject any tag-on orders under this Contract.

FINITE ELEMENT ANALYSIS AND TESTING
Finite Element Analysis (FEA) shall be provided by the manufacturer. Evidence of this testing being conducted shall be included with the bid. No exceptions.
Prototype bodies have been subjected to rigorous testing over varied terrains simulating different environmental conditions.

The purpose of such complex engineering methods of analysis shall be to ensure the longevity of the design by analyzing stress levels throughout the body and incorporating the structural supports wherever necessary.

There shall have been a minimum of three (3) different load cases (per DOT, FHWA, and TTMA recommended practice) applied and analyzed to properly display the different areas and levels of stresses that will be present under the various operating conditions of the apparatus.

In addition to the FEA analysis, the core product design shall be strain gauged instrumental to ensure validation of FEA results and “Real World” drive/apparatus driving conditions.

Analysis shall also have been conducted on the mounting system for the apparatus body and pump house. Exceptions to this statement may be cause for immediate rejection and/or be considered non-compliant.

SUPPLIED INFORMATION & EXTRAS
The apparatus manufacturer shall supply two (2) copies of apparatus manuals with all manufactured apparatus.

The manuals shall include, but not be limited to: all component warranties, users’ manuals and information for supplied products, apparatus engineering information including drawings and build prints, and whatever other pertinent information the manufacturer can supply to its customer regarding the said apparatus.
Included in the delivery of the unit, the manufacturer shall also include spare hardware and extra fasteners, paint for touch-up, information regarding washing and care procedures, as well as other recommendations for care and maintenance of the general apparatus.
The manufacturer shall also supply a manufacturer's record of apparatus construction details, including the following information:

· Owner name and address
· Apparatus manufacturer, model, and serial number
· Chassis make, model, and serial number
· GAWR of front and rear axles
· Front tire size and total rated capacity in pounds
· Rear tire size and total rated capacity in pounds
· Chassis weight distribution in pounds with water and manufacturer mounted equipment (front and rear)
· Engine make, model, serial number, rated horsepower, related speed and no load governed speed
· Type of fuel and fuel tank capacity
· Electrical system voltage and alternator output in amps
· Battery make and model, capacity in CCA
· Paint numbers
· Weight documents from a certified scale showing actual loading on the front axle, rear 

· axle, and overall vehicle (with the water tank full) but without personnel, equipment, 


and hose)
· Written load analysis and results of the electrical system performance tests
· Transmission make, model, and type
· Pump to drive through the transmission (yes or no)
· Engine to pump gear ratio and transmission gear ratio used
· Pump make and model, rated capacity in gallons per minute, serial number, and number 
of stages
· Pump manufacturer's certification of suction capability
· Pump manufacturer's certification of hydrostatic test
· Pump manufacturer's certification of inspection and test for the fire pump
· Copy of the apparatus manufacturer's approval for stationary pumping applications
· Pump transmission make, model and serial number
· Priming device type
· Type of pump pressure control system
· The engine manufacturer's certified brake horsepower curve for the engine furnished, 


showing the maximum no load governed speed
· Certification of the water tank capacity
ELECTRICAL SCHEMATICS
The apparatus manufacturer shall supply one (1) set of as-built wiring schematics, to include all line voltage schematics, with each apparatus.

WARNING AND INFORMATION LABELS
All warning and informational labels (non-vendor specific) shall be provided in appropriate locations to alert the operator of potential hazards and operating instructions.

ON-LINE CUSTOMER INTERACTION

The manufacture shall provide the capability for online access through the manufacture’s website. The customer shall be able to view digital photos of their apparatus in the specified phases of construction. The following phases will be captured and displayed on the manufacture’s website:

(1) Chassis when available at manufacturing facility

(2) Body – Prior to Paint

(3) Body – Painted

(4) Pump and Plumbing

(5) Assembly – 80% Complete

Due to the complex nature of fire apparatus and the importance of communication between the manufacture and customer, this line item is considered a critical requirement.  

LIABILITY INSURANCE COVERAGE
In order to protect the department and its personnel, the bidder shall show proof that it has no less than $20 million in liability insurance in force. A certificate of coverage shall be included in the bid package. Failure to carry liability insurance of at least this amount or failure to include proof of coverage shall be cause to reject the bidder's proposal.
GENERAL WARRANTY
The manufacturer shall provide a two (2) year bumper-to-bumper warranty from the date of delivery.

The warranty on the chassis, engine, transmission, tires, storage batteries, generators, electrical lamps and other devices subject to deterioration is limited to the warranty of the manufacturer thereof and adjustments for the same are to be made directly with the manufacturer by the customer.
APPARATUS TEST BY UNDERWRITERS’ LABORATORIES
The following Apparatus shall comply with all NFPA 1901, current edition, applicable regulations in effect as of the contract signing date. There shall be multiple tests performed by the contractor and Underwriter's Laboratories when the apparatus has been completed. The manufacturer shall furnish the completed Test Certificates to the purchaser at time of delivery. Since the inspection services of Underwriters Laboratories are available to all bidders on an equal basis, no other third-party testing service shall be acceptable. The tests conducted on the apparatus shall include, but not be limited to:
PUMP & PLUMBING PERFORMANCE TEST 
The apparatus pump and plumbing system shall be tested and certified.
12 VOLT ELECTRICAL TEST 
The apparatus low voltage electrical system shall be tested and certified.
FIRE STATION PRE-CONSTRUCTION CONFERENCE
The factory authorized Distributor shall be required, prior to manufacturing, to have a pre-construction conference at the fire departments station with individuals from the __________________ Fire Department to finalize all construction details.
FINAL INSPECTION CONFERENCE
The factory authorized Distributor shall be required, during manufacturing, to have a final completion inspection conference at the site of the manufacturing facility with up to four (4) individuals from the __________________ Fire Department to inspect the apparatus after construction.
The purchaser shall, at their expense, provide transportation, lodging, and meals. Any distance greater than 100 miles shall be by commercial air travel. The Dealership’s representative shall accompany the purchaser on the final inspection trip.
PUMP & APPARATUS TRAINING
The successful bidder shall provide a minimum three (3) hour structured training course to personnel assigned to operate the apparatus, covering nomenclature of components, proper operation of the apparatus, daily operational maintenance checks, and other information necessary for a firefighter/driver/engineer to properly operate and maintain the apparatus.
The firefighter/operator training shall be conducted within one week after the vehicle is fully accepted and readied for service by the "Purchaser" or at a time mutually agreed upon by the "Purchaser" and "Supplier".
CHASSIS MODEL

The chassis shall be a Metro Star model. The cab and chassis shall include design considerations for multiple emergency vehicle applications, rapid transit and maneuverability. The chassis shall be manufactured for heavy duty service with the strength and capacity to support a fully laden apparatus, one hundred (100) percent of the time.
MODEL YEAR
The chassis shall have a vehicle identification number that reflects a 2026 model year.

COUNTRY OF SERVICE
The chassis shall be put in service in the country of United States of America (USA).  

The chassis will meet applicable U.S.A. federal motor vehicle safety standards per CFR Title 49 Chapter V Part 571 as clarified in the incomplete vehicle book per CFR Title 49 Chapter V Part 568 Section 4 which accompanies each chassis.  The chassis manufacturer is not responsible for compliance to state, regional, or local regulations.  Dealers should identify those regulations and order any necessary optional equipment from the chassis manufacturer or their OEM needed to be in compliance with those regulations.
CAB AND CHASSIS LABELING LANGUAGE
The cab and chassis shall include the applicable caution, warning, and safety notice labels with text to be written in English. All applicable caution, warning, and safety notice labels shall be Innovative Controls brand. Where applicable to the location within the specific layout and label package of the cab and chassis, the labels shall include decorative chrome bezels. Designs shall include bezels that fit individual labels or packaged configurations of labels in certain common locations.
APPARATUS TYPE 
The apparatus shall be a pumper vehicle designed for emergency service use which shall be equipped with a permanently mounted fire pump which has a rated capacity of 1500 gallons per minute.  The apparatus shall include a water tank and hose body whose primary purpose is to combat structural and associated fires.

VEHICLE TYPE 
The chassis shall be manufactured for use as a straight truck type vehicle and designed for the installation of a permanently mounted apparatus behind the cab.  The apparatus of the vehicle shall be supplied and installed by the apparatus manufacturer.

VEHICLE ANGLE OF APPROACH PACKAGE
The angle of approach of the apparatus shall be a minimum of 8.00 degrees.   

NFPA1901 Angle of Approach definition:

“To determine the angle of approach, place a thin steel strip against the front of the tires where they touch the ground or stretch a tight string from one front tire to the other at the front where they touch the ground. Determine the lowest point (component or equipment) on the vehicle forward of the front tire that would make the smallest angle of approach. Hang a plumb bob from the lowest point and mark the point on the ground where the point of the plumb bob touches. Measure the vertical distance from the ground to the point where the plumb bob was hung (distance V). Measure the horizontal distance from the plumb bob point to the steel strip or string running from front tire to front tire (distance H).  Divide the vertical distance by the horizontal distance. The ratio of V/H is the tangent of the angle of approach.  If the ratio is known, the angle of approach can be determined from a table of trigonometric functions of angles or from a math calculator.  The standard requires a minimum angle of approach of 8.00 degrees: since the tangent of 8.00 degrees is 0.1405, if V divided by H is 0.1405 or larger, the angle of approach is 8.00 degrees or greater.”
AXLE CONFIGURATION
The chassis shall feature a 4 x 2 axle configuration consisting of a single rear drive axle with a single front steer axle.
GROSS AXLE WEIGHT RATINGS FRONT

The front gross axle weight rating (GAWR) of the chassis shall be 20,000 pounds.

This front gross axle weight rating shall be adequate to carry the weight of the completed apparatus including all equipment and personnel.
GROSS AXLE WEIGHT RATINGS REAR 
The rear gross axle weight rating (GAWR) of the chassis shall be 26,000 pounds.

This rear gross axle weight rating shall be adequate to carry the weight of the completed apparatus including all equipment and personnel.
PUMP PROVISION
The chassis shall include provisions to mount a drive line pump in the middle of the chassis, behind the cab, more commonly known as the midship location. Chassis driveline pump provisions shall include an interlock feature for automatic setting of the park brake when the vehicle is shifted into pump mode while the transmission is in neutral and the transmission output speed translates to less than 1 mph. When the conditions are met the driver side parking brake valve shall activate. Once shifted to road mode the condition for electric automatic brake engagement is no longer present and the driver’s parking brake control valve shall function normally.
WATER & FOAM TANK CAPACITY 
The chassis shall include a carrying capacity of 1020 gallons.  The water and foam tank shall be supplied and installed by the apparatus manufacturer.
WARRANTY

Purchaser shall receive a Custom Chassis Two (2) Years or 36,000 Miles limited warranty in accordance with, and subject to, warranty certificate RFW0102. The warranty certificate is incorporated by reference into this proposal, and included with this proposal or available upon request.
CAB STYLE 

The cab shall be a custom, fully enclosed, EMFD model with a 10.00 inch raised roof over the driver, officer, and crew area, designed and built specifically for use as an emergency response vehicle by a company specializing in cab and chassis design for all emergency response applications. The cab shall be designed for heavy-duty service utilizing superior strength and capacity for the application of protecting the occupants of the vehicle.  This style of cab shall offer up to eight (8) seating positions.

The cab shall incorporate a fully enclosed design with side wall roof supports, allowing for a spacious cab area with no partition between the front and rear sections of the cab. To provide a superior finish by reducing welds that fatigue cab metal; the roof, the rear wall and side wall panels shall be assembled using a combination of welds and proven industrial adhesives designed specifically for aluminum fabrication for construction.

The cab shall be constructed using multiple aluminum extrusions in conjunction with aluminum plate, which shall provide proven strength and the truest, flattest body surfaces ensuring less expensive paint repairs if needed.  All aluminum welding shall be completed to the American Welding Society and ANSI D1.2-96 requirements for structural welding of aluminum.  

All interior and exterior seams shall be sealed for optimum noise reduction and to provide the most favorable efficiency for heating and cooling retention. 

The cab shall be constructed of 5052-H32 corrosion resistant aluminum plate. The cab shall incorporate tongue and groove fitted 6061-T6 0.13- & 0.19-inch-thick aluminum extrusions for extreme duty situations.  A single formed, one (1) piece extrusion shall be used for the “A” pillar, adding strength and rigidity to the cab as well as additional roll-over protection. The cab side walls and lower roof skin shall be 0.13 inch thick; the rear wall and raised roof skins shall be 0.09 inch thick; the front cab structure shall be 0.19 inch thick.

The exterior width of the cab shall be 94.00 inches wide with a minimum interior width of 88.00 inches. The overall cab length shall be 137.10 inches with 60.00 inches from the centerline of the front of the axle to the back of the cab.  

The cab interior shall be designed to afford the maximum usable interior space and attention to ergonomics with hip and legroom while seated which exceeds industry standards. The crew cab floor shall be flat across the entire walking area for ease of movement inside the cab.
The cab shall offer an interior height of 57.50 inches from the front floor to the headliner and a rear floor to headliner height of 65.00 inches in the raised roof area, at a minimum.  The cab shall offer an interior measurement at the floor level from the rear of the engine tunnel to the rear wall of the cab of 57.88 inches. All interior measurements shall include the area within the interior trimmed surfaces and not to any unfinished surface.  

The cab shall include a driver and officer area with two (2) cab doors large enough for personnel in full firefighting gear.  The front doors shall offer a clear opening of 40.25 inches wide X 53.50 inches high, from the cab floor to the top of the door opening.  The cab shall also include a crew area with up to two (2) cab doors, also large enough for personnel in full firefighting gear. The rear doors shall offer a clear opening of 32.25 inches wide X 61.00 inches high, from the cab floor to the top of the door opening.  

The cab shall incorporate a progressive two (2) step configuration from the ground to the cab floor at each door opening.  The progressive steps are vertically staggered and extend the full width of each step well allowing personnel in full firefighting gear to enter and exit the cab easily and safely. 

The first step for the driver and officer area shall measure approximately 11.50 inches deep X 31.13 inches wide.  The intermediate step shall measure approximately 8.50 inches deep X 32.50 inches wide.  The height from the first step to the intermediate step and the intermediate step to the cab floor shall not exceed 11.00 inches.  

The first step for the crew area shall measure approximately 11.50 inches deep X 20.44 inches wide.  The intermediate step shall measure approximately 10.25 inches deep X 22.75 inches wide.  The height from the first step to the intermediate step and the intermediate step to the cab floor shall not exceed 12.80 inches.
OCCUPANT PROTECTION

An IMMI 4Front® occupant protection system shall be installed in the vehicle’s cab.  The system shall inflate three (3) air bags in the following locations:

· Steering wheel air bag to protect the head and neck of the driver 

· Knee bolster air bag to protect the driver’s legs 

· Knee bolster air bag to protect the officer’s legs

The air bags shall use a combination of high-pressure stored argon and oxygen with a pyrotechnic charge for initiation to inflate the bags remain inflated for several seconds.

The system shall be connected to the crash detection sensor that will also activate the driver and first officer integrated belt pretensioners if it detects a frontal crash.

A RollTek™ rollover occupant protection system shall be installed in the apparatus cab. The system shall include an integrated roll sensor (IRS) master module and a slave sensor in applicable configurations. 

The IRS shall be a microprocessor-controlled solid-state sensing device that utilizes vehicle-specific calibrations to detect rollovers. The IRS shall be equipped with pyrotechnic loops for connection to the protective countermeasures which shall include seat integrated side roll airbags (SRA), integrated seat belt pretensioners, and air seat pull-downs (S4S), in applicable occupant seat positions.

The IRS shall continuously monitor the truck`s acceleration and angle, and upon detection of an imminent roll-over, shall activate protective countermeasures in a pre-programmed sequence.  In addition, the IRS shall also act as a data recorder to record crash events for post-crash evaluation.
CAB FRONT FASCIA
The front cab fascia shall be constructed of 5052-H32 Marine Grade, 0.13 of an inch thick aluminum plate which shall be an integral part of the cab.

The cab fascia will encompass the entire front of the aluminum cab structure from the bottom of the windshield to the bottom of the cab and shall be the “Classic” design.  

The front cab fascia shall include two (2) molded plastic modules on each side accommodating four (4) Hi/Low beam headlights and four (4) warning lights. A chrome plated molded plastic bezel shall be provided on each side around each set of four lamps.
FRONT GRILLE

The front fascia shall include a box style, 304 stainless steel front grille 44.45 inches wide X 33.50 inches high X 1.50 inches deep.  The grille shall include a minimum free air intake of 732.00 square inches.  The upper portion of the grille shall be hinged to provide service access behind the grille. 
CAB UNDERCOAT

There shall be undercoating applied to the underside of the cab which provides an abrasion resistant coating for protection against corrosion caused by moisture, salt, alkalis and galvanic reaction.
CAB SIDE DRIP RAIL

There shall be a drip rail along the top radius of each cab side.  The drip rails shall help prevent water from the cab roof running down the cab side.
CAB PAINT EXTERIOR 

The cab exterior shall be painted a single color per customers specified paint color following the RFG-SR-001 paint standards.
CAB PAINT PROCESS/MANUFACTURER

The cab shall be painted with PPG Industries paint prior to the installation of glass accessories and all other cab trim to ensure complete paint coverage and the maximum in corrosion protection of all metal surfaces.

All metal surfaces on the cab shall be mechanically etched by sanding disc to remove any surface oxidation or surface debris which may hinder the paint adhesion. Once all imperfections on the exterior surfaces are removed and sanded smooth, body fillers shall be applied to the cab on all surfaces that require a critically aesthetic finish and sanded smooth.

The entire cab shall then be coated with a high-quality base primer that is designed to fill any minor surface defects, provide an adhesive bond between the primer and the paint and improve the color and gloss retention of the color. The finish to this procedure shall be sanding the cab to a smooth finish followed by sealing the seams with an automotive seam sealer. The minimum thickness of the primer coat after sanding shall be 2.50 mils with a maximum thickness of 5.00 mils.

The cab shall then be painted the specific color designated by the customer with an acrylic urethane type system designed to retain color and resist acid rain and most atmospheric chemicals found on an emergency scene. The paint shall have a minimum thickness of 1.00 mils with a maximum of 4 mills, followed by a clear top coat with a minimum of 2.5 mils and a maximum of 3.5 mils. The entire cab shall then be baked to speed the curing process of the coatings.
CAB PAINT PRIMARY/LOWER COLOR

The primary/lower paint color shall be PPG FBCH 75042 ALT red.
CAB PAINT WARRANTY

Purchaser shall receive a Paint and Finish (Exterior Clear coated) Ten (10) Years limited warranty in accordance with, and subject to, warranty certificate RFW0710. The warranty certificate is incorporated by reference into this proposal, and included with this proposal or available upon request.
CAB PAINT INTERIOR
The visible interior cab structure surfaces shall be painted with a multi-tone onyx black texture finish.
CAB ENTRY DOORS
The cab shall include four (4) entry doors, two (2) front doors and two (2) crew doors designed for ease of entering and egress when outfitted with an SCBA. The doors shall be constructed of extruded aluminum with a nominal thickness of 0.13 inch.  The exterior skins shall be constructed of 0.13-inch aluminum plate.  

The doors shall include a double rolled style automotive rubber seal around the perimeter of each door frame and door edge which ensures a weather tight fit.

All door hinges shall be hidden within flush mounted cab doors for a pleasing smooth appearance and perfect fit along each side of the cab.  Each door hinge shall be piano style with a 0.38-inch pin and shall be constructed of stainless steel.  
CAB ENTRY DOOR TYPE
All cab entry doors shall be barrier clear design resulting in exposed lower cab steps. The doors shall provide approximately 32.00 inches of clearance from the ground to the bottom of the door so cab doors may be opened un-hindered by most obstacles encountered, such as guard rails along interstate highways.

Entry doors shall include Pollak mechanical plunger style switches for electrical component activation.
CAB INSULATION
The cab ceiling and walls shall include a nonwoven polyester fiber insulation. The insulation shall act as a barrier absorbing noise as well as assisting in sustaining the desired climate within the cab interior.
LH MID EMS COMPARTMENT

The cab shall include a compartment located in the middle of the wall above the left side wheel well. This compartment shall measure 17.00 inches wide X 43.00 inches high X 23.00 inches deep.
The cab shall include a hinged box pan door featuring a full-length stainless steel piano style hinge and a bright aluminum tread plate inner panel located in the middle of the wall above the left side wheel well. The compartment shall have a minimum clear door opening of 15.00 inches wide X 40.00 inches high. 
The cab compartment located in the middle of the wall above the left side wheel well shall include solid aluminum walls with an interior access point rear facing. This compartment shall be finished to customer specification.
The left-hand EMS compartment shall include access from inside the cab.  The compartment shall be accessible from the inside of the cab via an aluminum framed aluminum hinged door with one (1) non-locking latch.  The interior access door shall feature a clear door opening of 14.50 inches wide and as tall as possible in the available customer specified left EMS compartment height and access point. The interior access door shall include a finish to match the EMS compartment exterior finish.
The left-hand mid EMS compartment located in crew area of the cab shall include one (1) aluminum shelf which shall be secured using Unistrut channel on two (2) sides of the interior walls of the compartment. The shelf shall include a 1.00-inch lip around the edges. The shelf shall be finished the same as the interior of the compartment.
The left side EMS compartment door shall include a locking bent D-ring slam latch installed in the lower latch side of the door. There shall be a switch to activate the open compartment warning light in the cab in the event the door is left ajar.
The cab shall include a polished stainless steel scuff plate wrapped around the bottom edge of the compartment opening.  The scuff plate shall extend approximately 2.00 inches onto the painted cab surface below the left side compartment door.
RH MID EMS COMPARTMENT

The cab shall include a compartment located in the middle of the wall above the right-side wheel well. This compartment shall measure 17.00 inches wide X 43.00 inches high X 23.00 inches deep.
The cab shall include a hinged box pan door featuring a full-length stainless steel piano style hinge and a bright aluminum tread plate inner panel located in the middle of the wall above the right-side wheel well. The compartment shall have a minimum clear door opening of 15.00 inches wide X 40.00 inches high. 
The cab compartment located in the middle of the wall above the right-side wheel well shall include solid aluminum walls with an interior access point rear facing. This compartment shall be finished to customer specification.
The right hand EMS compartment shall include access from inside the cab.  The compartment shall be accessible from the inside of the cab via an aluminum framed aluminum hinged door with one (1) non-locking latch.  The interior access door shall feature a clear door opening of 14.50 inches wide and as tall as possible in the available customer specified right EMS compartment height and access point. The interior access door shall include a finish to match the EMS compartment exterior finish.
The right-hand mid EMS compartment located in crew area of the cab shall include one (1) aluminum shelf which shall be secured using Unistrut channel on two (2) sides of the interior walls of the compartment. The shelf shall include a 1.00-inch lip around the edges. The shelf shall be finished the same as the interior of the compartment.
The right-side EMS compartment door shall include a locking bent D-ring slam latch installed in the lower latch side of the door. There shall be a switch to activate the open compartment warning light in the cab in the event the door is left ajar.
The cab shall include a polished stainless steel scuff plate wrapped around the bottom edge of the compartment opening.  The scuff plate shall extend approximately 2.00 inches onto the painted cab surface below the right-side compartment door.
MID EMS COMPARTMENT LIGHTING

The interior portion of each of the mid EMS compartments shall include compartment door activated LED lighting to illuminate all usable surfaces within each compartment. 
MID EMS COMPARTMENT EXTERIOR FINISH

The mid EMS compartment surfaces that are exposed to the interior of the cab shall be painted with a multi-tone onyx black texture finish.
MID EMS COMPARTMENT INTERIOR FINISH

The EMS compartment interior shall feature a DA sanded finish.
CAB STRUCTURAL WARRANTY

Purchaser shall receive an Aluminum Cab Structure Ten (10) Years or 100,000 Miles limited warranty in accordance with, and subject to, warranty certificate RFW0602.  The warranty certificate is incorporated by reference into this proposal, and included with this proposal or available upon request.
CAB TEST INFORMATION 
The cab shall have successfully completed the preload side impact, static roof load application and frontal impact without encroachment to the occupant survival space when tested in accordance with Section 4 of SAE J2420 COE Frontal Strength Evaluation Dynamic Loading Heavy Trucks, Section 5 of SAE J2422 Cab Roof Strength Evaluation Quasi –Static Loading Heavy Trucks and ECE R29 Uniform Provisions Concerning the Approval of Vehicles with regard to the Protection of the Occupants of the Cab of a Commercial Vehicles Annex 3 Paragraph 5. 

The above tests have been witnessed by and attested to by an independent third party.  The test results were recorded using cameras, high speed imagers, accelerometers and strain gauges. Documentation of the testing shall be provided upon request.
ELECTRICAL SYSTEM

The chassis shall include a single starting electrical system which shall include a 12-volt direct current multiplexing system, suppressed per SAE J551.  The wiring shall be appropriate gauge cross link with 311-degree Fahrenheit insulation.  All SAE wires in the chassis shall be color coded and shall include the circuit number and function where possible. The wiring shall be protected by 275-degree Fahrenheit minimum high temperature flame retardant loom.  All nodes and sealed Deutsch connectors shall be waterproof.
MULTIPLEX DISPLAY

The multiplex electrical system shall include an UltraView 780 display with an interactive touchscreen display and fourteen (14) tactile push buttons. The display shall be located on the left side of the dash in the switch panel. The display shall feature a full color 7.00-inch LCD display screen which shall include a message bar displaying the time of day and important messages requiring acknowledgement by the user. The display screen shall be video ready for back-up cameras, thermal cameras, and 360 camera systems.

The display shall offer varying fonts and background colors. The display shall be fully programmable to the needs of the customer and shall offer virtually infinite flexibility for screen configuration options.
MULTIPLEX DISPLAY SPECIAL LAYOUT

The Vista display and control screen shall be configured specifically for a virtual button on the control screen to override the park brake interlocked deactivation of the wiper system. This will reset when the park brake is cycled.
LOAD MANAGEMENT SYSTEM

The apparatus load management shall be performed by the included multiplex system. The multiplex system shall also feature the priority of sequences and shall shed electrical loads based on the priority list specifically programmed. 
DATA RECORDING SYSTEM
The chassis shall have a Weldon Vehicle Data Recorder (VDR) system installed. The system shall be designed to meet NFPA 1901 and shall be integrated with the Weldon Multiplex electrical system. The following information shall be recorded:

· Vehicle Speed

· Acceleration

· Deceleration

· Engine Speed

· Engine Throttle Position

· ABS Event

· Seat Occupied Status

· Seat Belt Status

· Master Optical Warning Device Switch Position

· Time

· Date

Each portion of the data shall be recorded at the specified intervals and stored for the specified length of time to meet NFPA 1901 guidelines and shall be retrievable by connecting a laptop computer to the VDR system. The laptop connection shall be a panel mounted female type B USB connection point, remotely mounted in the left side foot well.
ACCESSORY POWER #1
The electrical distribution panel shall include two (2) power studs. The studs shall be size #10 and each of the power studs shall be circuit protected with a fuse of the specified amperage.   One (1) power stud shall be capable of carrying up to a 40-amp battery direct load.  One (1) power stud shall be capable of carrying up to a 15-amp ignition switched load. The two (2) power studs shall share one (1) #10 ground stud. 

An OEM body connections bracket shall be installed on the chassis near the left-hand battery box.  The bracket shall include one (1) set each of 200-amp master power switched and 300-amp battery direct fused power and ground studs.
ACCESSORY POWER #2
A six (6) position Blue Sea Systems 5025 blade type fuse panel shall be installed behind the switch panel.  The fuse panel shall be protected by a 40-amp fuse. The panel shall be capable of carrying up to a maximum 40-amp battery direct load.
ACCESSORY POWER #3
A six (6) position Blue Sea Systems 5025 blade type fuse panel shall be installed on the side wall of the engine tunnel behind the officer's seat.  The fuse panel shall be protected by a 40-amp fuse. The panel shall be capable of carrying up to a maximum 40-amp battery direct load. An additional 4.00 feet of wire shall be provided for use by the apparatus builder.
ACCESSORY POWER #4
A six (6) position Blue Sea Systems 5025 blade type fuse panel shall be provided and installed in the forward-facing center seat box.  The fuse panel shall be protected by a 40-amp fuse. The panel shall be capable of carrying up to a maximum 40-amp battery direct load.
EXTERIOR ELECTRICAL TERMINAL COATING

All terminals exposed to the elements will be sprayed with a high visibility protective rubberized coating to prevent corrosion.
ELECTRICAL SYSTEM WARRANTY

Purchaser shall receive an Electrical System Two (2) Years or 36,000 Miles limited warranty in accordance with, and subject to, warranty certificate RFW0202.  The warranty certificate is incorporated by reference into this proposal, and included with this proposal or available upon request.
ENGINE 

The chassis engine shall be a Cummins L9 engine. The L9 engine shall be an in-line six (6) cylinder, four-cycle diesel-powered engine.  The engine shall offer a rating of 450 horsepower at 2100 RPM and shall be governed at 2200 RPM.  The torque rating shall feature 1250-foot pounds of torque at 1200 RPM with 543 cubic inches (8.9 liters) of displacement.  

The L9 engine shall feature a VGT™ Turbocharger, a high-pressure common rail fuel system, fully integrated electronic controls with an electronic governor, and shall be EPA certified to meet the 2021-26 emissions standards using cooled exhaust gas recirculation and selective catalytic reduction technology. 

The engine shall include an engine mounted combination full flow/by-pass oil filter with replaceable spin on cartridge for use with the engine lubrication system.  The engine shall include Citgo brand Citgard 500, or equivalent SAE 15W40 CK-4 low ash engine oil which shall be utilized for proper engine lubrication. 

A wiring harness shall be supplied ending at the back of the cab.  The harness shall include a connector which shall allow an optional harness for the pump panel.  The included circuits shall be provided for a tachometer, oil pressure, engine temperature, hand throttle, high idle and a PSG system.  A circuit for J1939 data link shall also be provided at the back of the cab.

If an L9 engine is NOT available or cannot be provided for that specific quote or build slot at time of production, you will automatically be upgraded and charged for an X12 (or the X10 engine) with all costs associated with the upgrade being passed on to the end user. No exceptions. 

If a pre-2027 emission engine is NOT available at the time of build (starting production on January 1, 2026) your order will automatically be upgraded and charged for either the 2027 engine compliant Cummins X-10 or X-15, with all associated costs being passed on to the end user. No exceptions.
CAB ENGINE TUNNEL 

The cab interior shall include an integrated engine tunnel constructed of 5052-H32 Marine Grade, 0.19 of an inch thick aluminum.  The tunnel shall be a maximum of 41.50 inches wide X 25.50 inches high.
DIESEL PARTICULATE FILTER CONTROLS

There shall be two (2) controls for the diesel particulate filter. One (1) control shall be for regeneration and one (1) control shall be for regeneration inhibit.
ENGINE PROGRAMMING HIGH IDLE SPEED

The engine high idle control shall maintain the engine idle at approximately 1250 RPM when engaged.
ENGINE HIGH IDLE CONTROL

The vehicle shall be equipped with an automatic high-idle speed control which shall be pre-set to operate the engine at a specified RPM to increase alternator output if the system voltage drops to 12.5 volts.  This device shall automatically operate only when the engine is running, the transmission is in neutral, and with the parking brake set.  The automatic high idle will stay engaged for a minimum of ten (10) minutes and until the system, voltage has reached 13.0 volts. Application of the service brake will override the automatic high idle and reset timer. The vehicle shall be equipped with a high-idle speed virtual button on the vehicle display and control screen to activate/deactivate manual control only. It shall be pre-set so when activated, it will operate the engine at the specified RPM to increase alternator output. This device shall operate only when the engine is running, the transmission is in neutral, and with the parking brake set. When automatically engaged the high idle shall disengage when the operator depresses the brake pedal, or the transmission is placed in gear, and shall be available to manually or automatically re-engage when the brake pedal is released, or when the transmission is placed in neutral. Virtual control screen shall not override automatic high idle between voltage parameters during timed cycle. Display shall indicate when high idle is disabled, enabled, or active. 
ENGINE PROGRAMMING ROAD SPEED GOVERNOR

The engine shall include programming which will govern the top speed of the vehicle.
AUXILIARY ENGINE BRAKE

A compression brake, for the six (6) cylinder engine shall be provided. A cutout relay shall be installed to disable the compression brake when in pump mode or when an ABS event occurs.  The engine compression brake shall activate upon 0% accelerator when in operation mode and actuate the vehicle’s brake lights.

The engine shall utilize a variable geometry turbo (VGT) as an integrated auxiliary engine brake to offer a variable rate of exhaust flow, which when activated in conjunction with the compression brake shall enhance the engine's compression braking capabilities.
An engine compression brake control device shall be included. The electronic control device shall monitor various conditions and shall activate the engine brake only if all of the following conditions are simultaneously detected: 

· A valid gear ratio is detected.

· The driver has requested or enabled engine compression brake operation.

· The throttle is at a minimum engine speed position.

· The electronic controller is not presently attempting to execute an electronically controlled final drive gear shift.  

The compression brake shall be controlled through an on/off switch and a low/medium/high selector switch.
ELECTRONIC ENGINE OIL LEVEL INDICATOR

The engine oil shall be monitored electronically and shall send a signal to activate a warning in the instrument panel when levels fall below normal. The warning shall activate in a low oil situation upon turning on the master battery and ignition switches without the engine running.
FLUID FILLS

The front of the chassis shall accommodate fluid fill for the engine oil through the grille.  This area shall also accommodate a check for the engine oil. The transmission, power steering, and coolant fluid fills and checks shall be under the cab.  The windshield washer fill shall be accessible through the front left side mid step. 
ENGINE DRAIN PLUG

The engine shall include an original equipment manufacturer installed oil drain plug. 
ENGINE WARRANTY 
The Cummins engine shall be warranted for a period of five (5) years or 100,000 miles, whichever occurs first.
REMOTE THROTTLE HARNESS

An apparatus interface wiring harness for the engine and transmission pump interlocks shall be supplied with the chassis.  The harness shall include a connector for connection to a chassis pump panel harness supplied by the body builder and shall terminate in the left frame rail behind the cab for connection by the body builder.  The harness shall include circuits deemed for a pump panel and shall contain circuits for a hand throttle, and a multiplexed gauge.  Separate circuits shall also be included for a pump control switch, “Pump Engaged” and “OK to Pump” indicator lights, open compartment ground, start signal, park brake ground, ignition signal, master power, clean power, customer ignition, air horn solenoid switch, high idle switch and high idle indicator light. The harness shall contain interlocks that will prevent shifting to road or pump mode unless the transmission output speed translates to less than 1 mph and the transmission is in neutral. The shift to pump mode shall also require the park brake be set.
ENGINE PROGRAMMING REMOTE THROTTLE

The engine ECM (Electronic Control Module) discreet wire remote throttle circuit shall be turned off for use with a J1939 based pump controller or when the discreet wire remote throttle controls are not required.
ENGINE PROGRAMMING IDLE SPEED

The engine low idle speed will be programmed at 750 rpm.
ENGINE AIR INTAKE 
The engine air intake system shall include an ember separator.  This ember separator shall be designed to protect the downstream air filter from embers using a combination of unique flat and crimped metal screens packaged in a heavy-duty galvanized steel frame.  This multilayered screen shall trap embers and allow them to burn out before passing through the pack.

The engine air intake system shall also include an air cleaner mounted above the radiator. This air cleaner shall utilize a replaceable dry type filter element designed to prevent dust and debris from being ingested into the engine. A service cover shall be provided on the housing, reducing the chance of contaminating the air intake system during air filter service.

The air intake system shall include a restriction indicator light in the warning light cluster on the instrument panel, which shall activate when the air cleaner element requires replacement.
ENGINE FAN DRIVE
The engine cooling system fan shall incorporate a thermostatically controlled, Horton fully variable type fan drive with SmartClutch J-1939 CAN controller. 
The variable speed fan clutch only engages at the amount needed for proper cooling to facilitate improved vehicle performance, cab heating in cold climates, and fuel economy.  The fan clutch design shall be fail-safe so that if the clutch drive fails the fan shall engage to prevent engine overheating due to the fan clutch failure. The fan speed shall include a J-1939 CAN clutch controller to receive signal from the engine control module to activate at variable rates of speed. Variable speeds shall be set through thermostatic and engine speed signals to run as efficiently and quietly as required to maintain temperature. 
ENGINE COOLING SYSTEM
There shall be a heavy-duty aluminum cooling system designed to meet the demands of the emergency response industry.  The cooling system shall have the capacity to keep the engine properly cooled under all conditions of road and pumping operations.  The cooling system shall be designed and tested to meet or exceed the requirements specified by the engine and transmission manufacturer and all EPA requirements.  The complete cooling system shall be mounted to isolate the entire system from vibration or stress. The individual cores of the cooling system shall be mounted in a manner to allow expansion and contraction at various rates without inducing stress into the adjoining cores.

The cooling system shall be comprised of a charge air cooler to radiator serial flow package that provides the maximum cooling capacity for the specified engine as well as serviceability.  The main components shall include a surge tank, a charge air cooler bolted to the front of the radiator, recirculation shields, a shroud, a fan, and required tubing.  

The radiator shall be a down-flow design constructed with aluminum cores, plastic end tanks, and a steel frame.  The radiator shall be equipped with a drain cock to drain the coolant for serviceability.  

The cooling system shall include a one piece injected molded polymer fan with a three (3) piece fiberglass fan shroud. 

The cooling system shall be equipped with a surge tank that is capable of removing entrained air from the system. The surge tank shall be equipped with a low coolant probe and rearward oriented sight glass to observe coolant in the system. A cold fill and observation line shall be included within the frame mounted translucent recovery bottle to monitor the level of the coolant. The surge tank shall have a dual seal cap that meets the engine manufacturer's pressure requirements and allows for expansion and recovery of coolant into a separate integral expansion chamber.

All radiator tubes shall be formed from aluminized steel tubing.  Recirculation shields shall be installed where required to prevent heated air from reentering the cooling package and affecting performance.  

The charge air cooler shall be a cross-flow design constructed completely of aluminum with cast tanks. All charge air cooler tubes shall be formed from aluminized steel tubing and installed with silicone hump hoses and stainless steel “constant torque” style clamps meeting the engine manufacturer's requirements.

The radiator and charge air cooler shall be removable through the bottom of the chassis.
ENGINE COOLING SYSTEM PROTECTION
The engine cooling system shall include a recirculation shield designed to act as a light duty skid plate below the radiator to provide additional protection for the engine cooling system from light impacts, stones, and road debris. The skid plate shall be painted to match the frame components.
ENGINE COOLANT 
The cooling package shall include Extended Life Coolant (ELC).  The use of ELC provides longer intervals between coolant changes over standard coolants providing improved performance.  The coolant shall contain a 50/50 mix of ethylene glycol and de-ionized water to keep the coolant from freezing to a temperature of -34 degrees Fahrenheit.

Proposals offering supplemental coolant additives (SCA) shall not be considered, as this is part of the extended life coolant makeup.
ENGINE COOLANT FILTER 
An engine coolant filter with a shut-off valve for the inlet and outlet shall be installed on the chassis. The location of the filter shall allow for easy maintenance.  

Proposals offering engines equipped with coolant filters shall be supplied with standard non-chemical type particulate filters.
ELECTRONIC COOLANT LEVEL INDICATOR 

The instrument panel shall feature a low engine coolant indicator light which shall be located in the center of the instrument panel.  An audible tone alarm shall also be provided to warn of a low coolant incident.
COOLANT HOSES
The cooling systems hose shall be formed silicone hose and formed aluminized steel tubing and include stainless steel constant torque band clamps.
ENGINE COOLANT OVERFLOW BOTTLE

A remote engine coolant overflow expansion bottle shall be provided in the case of over filling the coolant system. The overflow bottle shall capture the expansion fluid or overfill rather than allow the fluid to drain on the ground. 
ENGINE PUMP HEAT EXCHANGER 
A single bundle type coolant to water heat exchanger shall be installed between the engine and the radiator.  The heat exchanger shall be designed to prohibit water from the pump from coming in contact with the engine coolant.  This shall allow the use of water from the discharge side of the pump to assist in cooling the engine.
ENGINE EXHAUST SYSTEM 
The exhaust system shall include an end-in end-out horizontally mounted single module after treatment device, and downpipe from the charge air cooled turbo.  The single module shall include four temperature sensors, diesel particulate filter (DPF), urea dosing module (UL2), and a selective catalytic reduction (SCR) catalyst to meet current EPA standards.  The selective catalytic reduction catalyst utilizes a diesel exhaust fluid solution consisting of urea and purified water to convert NOx into nitrogen, water, and trace amounts of carbon dioxide. The solution shall be mixed and injected into the system through the DPF and SCR.
The system shall utilize 0.07-inch-thick stainless steel exhaust tubing between the engine turbo and the DPF.  Zero leak clamps seal all system joints between the turbo and DPF. 

The single module after treatment through the end of the tailpipe shall be connected with zero leak clamps. The discharge shall terminate horizontally on the right side of the vehicle ahead of the rear tires.  

The exhaust system after treatment module shall be mounted below the frame in the outboard position. 
DIESEL EXHAUST FLUID TANK 
The exhaust system shall include a molded cross linked polyethylene tank for Diesel Exhaust Fluid (DEF).  The tank shall have a capacity of six (6) usable gallons and shall be mounted on the left hand side of the chassis frame behind the batteries below the frame.  

The DEF tank shall be designed with capacity for expansion in case of fluid freezing. Engine coolant, which shall be thermostatically controlled, shall be run through lines in the tank to help prevent the DEF from freezing and to provide a means of thawing the fluid if it should become frozen.

The tank fill tube shall be routed under the rear of the cab with the fill neck and splash guard accessible in the top rear step.
ENGINE EXHAUST ACCESSORIES
An exhaust temperature mitigation device shall be shipped loose for installation by the body manufacturer on the vehicle.  The temperature mitigation device shall lower the temperature of the exhaust by combining ambient air with the exhaust gasses at the exhaust outlet.

The tail pipe shall have a drop in it to allow additional clearance from the body. 
ENGINE EXHAUST WRAP 
The exhaust tubing between the engine turbo and the diesel particulate filter (DPF) shall be wrapped with a thermal cover in order to retain the necessary heat for DPF regeneration. The exhaust wrap shall also help protect surrounding components from radiant heat which can be transferred from the exhaust.

The exhaust flex joint shall not include the thermal exhaust wrap.  
EMISSIONS SYSTEMS WARRANTY

Purchaser shall receive a Regulated Emissions Systems Five (5) Years or 100,000 Miles limited warranty in accordance with, and subject to, warranty certificate RFW0140. The warranty certificate is incorporated by reference into this proposal, and included with this proposal or available upon request.
TRANSMISSION 
The drive train shall include an Allison model EVS 3000 torque converting, automatic transmission which shall include electronic controls.  The transmission shall feature two (2) 10-bolt PTO pads located on the converter housing.   
The transmission shall include two (2) internal oil filters which shall offer Allison formulated Castrol TranSynd™ synthetic transmission fluid which shall be utilized in the lubrication of the EVS transmission.  An electronic oil level sensor shall be included with the readout located in the shift selector.

The transmission gear ratios shall be:  

1st 
3.49:1 

2nd 
1.86:1 

3rd 
1.41:1 

4th 
1.00:1 

5th 
0.75:1

Rev 
5.03:1
TRANSMISSION MODE PROGRAMMING 
The transmission, upon start-up, will select the fifth speed operation without the need to press the mode button.
TRANSMISSION FEATURE PROGRAMMING
The Allison Gen V/VI-E transmission EVS group package number 127 shall contain the 198 vocational package in consideration of the duty of this apparatus as a pumper.  This package shall incorporate an automatic neutral with selector override. This feature commands the transmission to neutral when the park brake is applied, regardless of drive range requested on the shift selector. This requires re-selecting drive range to shift out of neutral for the override. 

This package shall be coupled with the use of a split shaft PTO and incorporate pumping circuits.  These circuits shall be used allowing the vehicle to operate in the fourth range lockup while operating the pump mode due to the 1 to 1 ratio through the transmission, therefore the output speed of the engine is the input speed to the pump.  The pump output can be easily calculated by using this input speed and the drive ratio of the pump itself to rate the gallons of water the pump can provide.

A transmission interface connector shall be provided in the cab. This package shall contain the following input/output circuits to the transmission control module. The Gen V/VI-E transmission shall include prognostic diagnostic capabilities. These capabilities shall include the monitoring of the fluid life, filter change indication, and transmission clutch maintenance.

Function ID
Description




Wire assignment
Inputs

C 


PTO Request 




142

J  


Fire Truck Pump Mode (4th Lockup) 

122 / 123

Outputs

C 


Range Indicator 



145 (4th)

G 

PTO Enable Output



130




Signal Return




103
TRANSMISSION SHIFT SELECTOR 
An Allison pressure sensitive range selector touch pad shall be provided and located to the right of the driver within clear view and easy reach. The shift selector shall have a graphical Vacuum Florescent Display (VFD) capable of displaying two lines of text.  The shift selector shall provide mode indication and a prognostic indicator (wrench symbol) on the digital display.  The prognostics monitor various operating parameters and shall alert you when a specific maintenance function is required. 
ELECTRONIC TRANSMISSION OIL LEVEL INDICATOR 

The transmission fluid shall be monitored electronically.
TRANSMISSION PRE-SELECT WITH AUXILIARY BRAKE 
When the auxiliary brake is engaged, the transmission shall automatically shift to second gear to decrease the rate of speed assisting the secondary braking system and slowing the vehicle.
TRANSMISSION COOLING SYSTEM 
The transmission shall include a water to oil cooler system located in the cooling loop between the radiator and the engine.  The transmission cooling system shall meet all transmission manufacturer requirements.  The transmission cooling system shall feature continuous flow of engine bypass water to maintain uninterrupted transmission cooling. 
TRANSMISSION DRAIN PLUG

The transmission shall include an original equipment manufacturer installed magnetic transmission fluid drain plug.
TRANSMISSION WARRANTY 
The Allison EVS series transmission shall be warranted for a period of five (5) years with unlimited mileage. Parts and labor shall be included in the warranty.
PTO LOCATION
The transmission shall have two (2) power take off (PTO) mounting locations, one (1) in the 8:00 o’clock position and one (1) in the 4:00 o’clock position.
DRIVELINE 
All drivelines shall be heavy duty metal tube and equipped with MSI 1710 series universal joints.  The shafts shall be dynamically balanced prior to installation to alleviate future vibration.  In areas of the driveline where a slip shaft is required, the splined slip joint shall be coated with Glide Coat®. The drivelines shall include Meritor brand u-joints with thrust washers.
MIDSHIP PUMP / GEARBOX
A temporary jackshaft driveline shall be installed by the chassis manufacturer to accommodate the mid-ship split shaft pump as specified by the apparatus manufacturer. Holes shall be provided as specified by the OEM for mounting a customer installed pump module.

The midship pump/gearbox provisions shall be for a Waterous CSUC20 pump.
The Waterous pump gearbox shall have a “C” (medium length) drop length.
The ratio for the midship pump shall be 2.27:1.
The midship pump shall be located so the dimension from the centerline of the suction to the centerline of the rear axle is 99.50 inches.
PUMP SHIFT CONTROLS

One (1) air pump shift control panel shall be located on the left-hand side of the engine tunnel, integrated with the shifter pod. The following shall be provided on the panel: a three (3) position control lever; an engraved PUMP ENGAGED identification light; and an engraved OK TO PUMP identification light. The pump shift control panel shall be black with a yellow border outline and shall include pump instructions. An instruction plate describing the transmission shift selector position used for pumping shall be provided and located so it can be read from the driver’s position per NFPA 16.10.1.3. The road mode shall be selected when the control lever is in the forward position and pump mode shall be selected when the control lever is in the rearward position. 

The control lever center position shall exhaust air from both pump and road sides of the pump gear box shift cylinder.
PUMP SHIFT CONTROL PLUMBING

Air connections shall be provided from the air supply tank to the pump shift control valve and from the pump shift control valve to the frame mounted bracket. The frame mounted bracket shall include labeling identifying the pump and road connection points with threaded 0.25-inch NPT fittings on the solenoid for attaching the customer installed pump.  The air supply shall be pressure protected from service brake system.
FUEL FILTER/WATER SEPARATOR
The fuel system shall have a Fleetguard FS20121 fuel filter/water separator as a primary filter. The fuel filter shall have a drain valve. 

A water in fuel sensor shall be provided and wired to an instrument panel lamp and audible alarm to indicate when water is present in the fuel/water separator.

A secondary fuel filter shall be included as approved by the engine manufacturer.
FUEL LINES
The fuel system supply and return lines installed from the fuel tank to the engine shall be black textile braided lines which are reinforced with braided high tensile steel wire. The fuel lines shall be connected with reusable steel fittings.
FUEL SHUTOFF VALVES 
There shall be two (2) fuel shutoff valves which shall be installed, one (1) in the fuel draw line at the primary fuel filter and one (1) in the fuel outlet line at the primary fuel filter to allow the fuel filters to be changed without loss of fuel to the fuel pump.

A third fuel shutoff valve shall be installed in the fuel draw line, near the fuel tank to allow maintenance to be performed with minimal loss of fuel.
ELECTRIC FUEL PRIMER 
Integral to the engine assembly is an electric lift pump that serves the purpose of pre-filter fuel priming. 
FUEL TANK 
The fuel tank shall have a capacity of sixty-eight (68) gallons and shall measure 35.00 inches in width X 17.00 inches in height X 29.00 inches in length.  

The baffled tank shall have a vent port to facilitate venting to the top of the fill neck for rapid filling without "blow-back" and a roll over ball check vent for temperature related fuel expansion and draw. 
The tank is designed with dual draw tubes and sender flanges. The tank shall have 2.00-inch NPT fill ports for right or left-hand fill.  A 0.50-inch NPT drain plug shall be centered in the bottom of the tank.
The fuel tank shall be mounted below the frame, behind the rear axle.  Two (2) three-piece strap hanger assemblies with “U” straps bolted midway on the fuel tank front and rear shall be utilized to allow the tank to be easily lowered and removed for service purposes. Rubber isolating pads shall be provided between the tank and the upper tank mounting brackets.  Strap mounting studs through the rail, hidden behind the body shall not be acceptable.
FUEL TANK MATERIAL AND FINISH
The fuel tank shall be constructed of 12-gauge aluminized steel. The exterior of the tank shall be powder coated black and then shall feature a spray on bedliner coating.

All powder coatings, primers and paint shall be compatible with all metals, pretreatments and primers used.  The cross-hatch adhesion test per ASTM D3359 Method B, results to be 5B minimum.  The pencil hardness test per ASTM D3363 shall have a final post-curved pencil hardness of H-2H.  The direct impact resistance test per ASTM D2794, results to be 5B minimum.   

Any proposals offering painted fuel tanks with variations from the above process shall not be accepted.  The film thickness of vendor supplied parts shall also be sufficient to meet the performance standards as stated above.
FUEL TANK STRAP MATERIAL

The fuel tank straps shall be constructed of ASTM A-36 hot-dip galvanized steel. The fuel tank straps shall include a natural galvanized finish.
FUEL TANK FILL PORT 
The fuel tank fill ports shall be provided with two (2) left fill ports located one (1) in the forward position and one (1) in the middle position and the right fill port located in the middle position of the fuel tank.

A 1.50-inch diameter hole shall be provided in the left and right frame rails for vent hose routing provisions. The holes shall be located adjacent to the fuel tank and 5.13 inches up from the bottom of each rail.
FUEL TANK SERVICEABILTY PROVISIONS
The chassis fuel lines shall have additional length provided so the tank can be easily lowered and removed for service purposes. The additional 8.00 feet of length shall be located above the fuel tank and shall be coiled and secured. The fuel line fittings shall be pointed towards the right side (curbside) of the chassis.
FUEL TANK DRAIN PLUG
A 0.5-inch NPT magnetic drain plug shall be centered in the bottom of the fuel tank.
FRONT AXLE

The front axle shall be a Meritor Easy Steer Non drive front axle, model number MFS-20.  The axle shall include a 3.74-inch drop and a 71.00-inch king pin intersection (KPI). The axle shall include a conventional style hub with a standard knuckle. 
FRONT AXLE WARRANTY

The front axle shall be warranted by Meritor for five (5) years with unlimited miles under the general service application. Details of the Meritor warranty are provided on the PDF document attached to this option.
FRONT WHEEL BEARING LUBRICATION 
The front axle wheel bearings shall be lubricated with oil.  The oil level can be visually checked via clear inspection windows in the front axle hubs.
FRONT SHOCK ABSORBERS 
Two (2) Bilstein inert, nitrogen gas filled shock absorbers shall be provided and installed as part of the front suspension system.  The shocks shall be a monotubular design and fabricated using a special extrusion method, utilizing a single blank of steel without a welded seam, achieving an extremely tight peak-to-valley tolerance and maintains consistent wall thickness.  The monotubular design shall provide superior strength while maximizing heat dissipation and shock life.  

The ride afforded through the use of a gas shock is more consistent and shall not deteriorate with heat, the same way a conventional oil filled hydraulic shock would.

The Bilstein front shocks shall include a digressive working piston assembly allowing independent tuning of the compression and rebound damping forces to provide optimum ride and comfort without compromise.  The working piston design shall feature fewer parts than most conventional twin tube and “road sensing” shock designs and shall contribute to the durability and long life of the Bilstein shock absorbers. 

Proposals offering the use of conventional twin tube or “road sensing” designed shocks shall not be considered.
FRONT SUSPENSION 
The front suspension shall include a ten (10) leaf spring pack in which the longest leaf measures 54.00 inch long and 4.00 inches wide and shall include a military double wrapped front eye.  Both spring eyes shall have a case-hardened threaded bushing installed with lubrication counter bore and lubrication land off cross bore with grease fitting.  The spring capacity shall be rated at 21,500 pounds.
STEERING COLUMN/ WHEEL 
The cab shall include a Douglas Autotech steering column which shall include a seven (7) position tilt, a 2.25 inch telescopic adjustment, and an 18.00 inch, four (4) spoke steering wheel located at the driver’s position. The steering wheel shall be covered with black polyurethane foam padding. 
The steering column shall contain a horn button, self-canceling turn signal switch, four-way hazard switch and headlamp dimmer switch.
ELECTRONIC POWER STEERING FLUID LEVEL INDICATOR 

The power steering fluid shall be monitored electronically and shall send a signal to activate an audible alarm and visual warning in the instrument panel when fluid level falls below normal.
POWER STEERING PUMP
The hydraulic power steering pump shall be a TRW PS and shall be gear driven from the engine.  The pump shall be a balanced, positive displacement, sliding vane type. The power steering system shall include an oil to air passive cooler.
FRONT AXLE CRAMP ANGLE 
The chassis shall have a front axle cramp angle of 48-degrees to the left and 44-degrees to the right.
POWER STEERING GEAR
The power steering gear shall be a TRW model TAS 65 with an assist cylinder.
CHASSIS ALIGNMENT 

The chassis frame rails shall be measured to ensure the length is correct and cross checked to make sure they run parallel and are square to each other.  The front and rear axles shall be laser aligned. The front tires and wheels shall be aligned and toe-in set on the front tires by the chassis manufacturer.
REAR AXLE 
The rear axle shall be a Meritor model RS-25-160 single drive axle. The axle shall include precision forged, single reduction differential gearing, and shall have a fire service rated capacity of 27,000 pounds.

The axle shall be built of superior construction and quality components to provide the rugged dependability needed to stand up to the fire industry’s demands.  The axle shall include rectangular shaped, hot-formed housing with a standard wall thickness of 0.63 of an inch for extra strength and rigidity and a rigid differential case for high axle strength and reduced maintenance.

The axle shall have heavy-duty Hypoid gearing for longer life, greater strength and quieter operation.  Industry-standard wheel ends for compatibility with both disc and drum brakes, and unitized oil seal technology to keep lubricant in and help prevent contaminant damage will be used.
REAR AXLE DIFFERENTIAL LUBRICATION 
The rear axle differential shall be lubricated with oil. 
REAR AXLE WARRANTY

The rear axle shall be warranted by Meritor for five (5) years with unlimited miles under the general service application. Details of the Meritor warranty are provided on the PDF document attached to this option.
REAR WHEEL BEARING LUBRICATION 
The rear axle wheel bearings shall be lubricated with oil.
VEHICLE TOP SPEED 
The top speed of the vehicle shall be approximately 68 MPH +/-2 MPH at governed engine RPM.
REAR SUSPENSION
The single rear axle shall feature a Reyco 79KB vari-rate, self-leveling captive slipper type conventional multi-leaf spring suspension, with 57.50-inch X 3.00-inch springs.  One (1) adjustable and one (1) fixed torque rod shall be provided. 
The rear suspension capacity shall be rated at 26,000 pounds. 
REAR SHOCK ABSORBERS 
Two (2) Bilstein inert, nitrogen gas filled shock absorbers shall be provided and installed as part of the rear suspension system.  The shocks shall be a monotubular design and fabricated using a special extrusion method, utilizing a single blank of steel without a welded seam, achieving an extremely tight peak-to-valley tolerance and maintains consistent wall thickness.  The monotubular design shall provide superior strength while maximizing heat dissipation and shock life.  

The ride afforded through the use of a gas shock is more consistent and shall not deteriorate with heat, the same way a conventional oil filled hydraulic shock would.

The Bilstein front shocks shall include a digressive working piston assembly allowing independent tuning of the compression and rebound damping forces to provide optimum ride and comfort without compromise.  The working piston design shall feature fewer parts than most conventional twin tube and “road sensing” shock designs and shall contribute to the durability and long life of the Bilstein shock absorbers. 

Proposals offering the use of conventional twin tube or “road sensing” designed shocks shall not be considered.
TIRE INTERMITTENT SERVICE RATING

The chassis shall be rated using Intermittent Service ratings provided to the emergency vehicle market by the tire manufacturers as the basis for determining the maximum vehicle load and speed.
FRONT TIRES
The front tires shall be Michelin 315/80R-22.5 20PR "L" tubeless radial XZUS 2 regional tread. 

The front tire stamped load capacity shall be 20,000 pounds per axle with a nominal speed rating of 65 miles per hour when properly inflated to 130 pounds per square inch.

REAR TIRES
The rear tires shall be Michelin 12R-22.5 16PR "H" tubeless radial XDN2 all-weather tread. 

The rear tire stamped load capacity shall be 27,120 pounds per axle with a nominal speed rating of 75 miles per hour when properly inflated to 120 pounds per square inch.

REAR AXLE RATIO 
The rear axle ratio shall be 5.13:1.
TIRE PRESSURE INDICATORS 
There shall be electronic chrome LED valve caps shipped loose for installation by the OEM which shall illuminate with a red LED when tire pressure drops 8psi provided. The valve caps are self-calibrating and set to the pressure of the tire upon installation. 
FRONT WHEELS
The front wheels shall be Alcoa hub piloted, 22.50-inch X 9.00-inch aluminum wheels. The outer face of the wheels shall feature Alcoa’s Dura-Bright® finish as an integral part of the wheel surface. Alcoa Dura-Bright® wheels keep their shine without polishing. Brake dust, grime and road debris are easily removed by simply cleaning the wheels with soap and water. The hub piloted mounting system shall provide easy installation and shall include two-piece flange nuts.
REAR WHEELS
The rear wheels shall be Alcoa hub piloted, 22.50-inch X 8.25-inch aluminum wheels with a polished outer surface and Alcoa Dura-Bright® wheel treatment as an integral part of the wheel surface. The inner rear wheels shall be Alcoa hub piloted, 22.50-inch X 8.25-inch aluminum wheels with a polished inner and outer surface and Alcoa Dura-Bright® wheel treatment as an integral part of the wheel surface. The hub piloted mounting system shall provide easy installation and shall include two-piece flange nuts.

BALANCE WHEELS AND TIRES 
All of the wheels and tires, including any spare wheels and tire assemblies, shall be dynamically balanced.
WHEEL TRIM
The front wheels shall include stainless steel lug nut covers and stainless-steel baby moons shipped loose with the chassis for installation by the apparatus builder. The baby moons shall have cutouts for oil seal viewing when applicable. 

The rear wheels shall include stainless steel lug nut covers and band mounted spring clip stainless steel high hats shipped loose with the chassis for installation by the apparatus builder.
The lug nut covers, baby moons, and high hats shall be RealWheels® brand constructed of 304L grade, non-corrosive stainless steel with a mirror finish. Each wheel trim component shall meet D.O.T. certification.
WHEEL GUARDS 
The rear dual wheels shall include a plastic isolator approximately 0.04” installed between the inner and outer wheel hub to help prevent corrosion caused by metal-to-metal contact. There shall also be a plastic isolator between the axle hub and the wheels on both front and rear axles.
BRAKE SYSTEM 
A rapid build-up air brake system shall be provided.  The air brakes shall include, at a minimum, a two (2) air tank, three (3) reservoir system with a total of 4152 cubic inch of air capacity.  A floor mounted treadle valve shall be mounted inside the cab for graduated control of applying and releasing the brakes.  An inversion valve shall be installed to provide a service brake application in the unlikely event of primary air supply loss. All air reservoirs provided on the chassis shall be labeled for identification.

The rear axle spring brakes shall automatically apply in any situation when the air pressure falls below 25 PSI and shall include a mechanical means for releasing the spring brakes when necessary.  An audible alarm shall designate when the system air pressure is below 60 PSI.

A four (4) sensor, four (4) modulator Anti-lock Braking System (ABS) shall be installed on the front and rear axles in order to prevent the brakes from locking or skidding while braking during hard stops or on icy or wet surfaces.  This in turn shall allow the driver to maintain steering control under heavy braking and in most instances, shorten the braking distance.  The electronic monitoring system shall incorporate diagonal circuitry which shall monitor wheel speed during braking through a sensor and tone ring on each wheel.  A dash mounted ABS lamp shall be provided to notify the driver of a system malfunction. The ABS system shall automatically disengage the auxiliary braking system device when required.  The speedometer screen shall be capable of reporting all active defaults using PID/SID and FMI standards.
Additional safety shall be accommodated through Automatic Traction Control (ATC) which shall be installed on the single rear axle.  The ATC system shall apply the ABS when the drive wheels lose traction.    The system shall scale the electronic engine throttle back to prevent wheel spin while accelerating on ice or wet surfaces.
A virtual button on the vehicle display and control screen shall be provided and properly labeled “mud/snow”.  When the switch is pressed once, the system shall allow a momentary wheel slip to obtain traction under extreme mud and snow conditions.  During this condition the ATC light shall blink continuously notifying the driver of activation.  Pressing the switch again shall deactivate the mud/snow feature.

The Electronic Stability Control (ESC) unit is a functional extension of the electronic braking system. It is able to detect any skidding of the vehicle about its vertical axis as well as any rollover tendency. The control unit comprises an angular-speed sensor that measures the vehicle’s motion about the vertical axis, caused, for instance, by cornering or by skidding on a slippery road surface. An acceleration sensor measures the vehicle’s lateral acceleration. The Controller Area Network (CAN) bus provides information on the steering angle. On the basis of lateral acceleration and steering angle, an integrated microcontroller calculates a theoretical angular speed for the stable vehicle condition.
FRONT BRAKES 
The front brakes shall be Meritor EX225 Disc Plus disc brakes with 17.00-inch vented rotors.
REAR BRAKES 
The rear brakes shall be Meritor 16.50-inch X 8.63-inch S-cam drum type. The brakes shall feature a cast iron shoe.
PARK BRAKE

Upon application of the push-pull valve in the cab, the rear brakes will engage via mechanical spring force.  This is accomplished by dual chamber rear brakes, satisfying the FMVSS parking brake requirements.
PARK BRAKE CONTROL

A Meritor-Wabco manual hand control push-pull style valve shall operate the parking brake.

The parking brake actuation valve shall be mounted to the left side of the engine tunnel integrated into the transmission shift pod console within easy access of the driver. 
REAR BRAKE SLACK ADJUSTERS 
The rear brakes shall include Meritor automatic slack adjusters installed on the axle which features a simple, durable design offering reduced weight.  The automatic slack adjusters shall feature a manual adjusting nut which cannot inadvertently be backed off and threaded grease fittings for easy serviceability.
REAR BRAKE DUST SHIELDS 
The rear brakes shall be equipped with brake dust shields.
AIR DRYER 
The brake system shall include a Wabco System Saver 1200 air dryer with an integral 100-watt heater with a Metri-Pack sealed connector.  The air dryer incorporates an internal turbo cutoff valve that closes the path between the air compressor and air dryer purge valve during the compressor "unload" cycle. The turbo cutoff valve allows purging of moisture and contaminants without the loss of turbo boost pressure.  The air dryer shall be located on the right-hand frame rail forward of the front wheel behind the right-hand cab step.
FRONT BRAKE CHAMBERS
The front brakes shall be provided with MGM type 24 long stroke brake chambers.
REAR BRAKE CHAMBERS

The rear axle shall include TSE 30/36 brake chambers which shall convert the energy of compressed air into mechanical force and motion.  This shall actuate the brake camshaft, which in turn shall operate the foundational brake mechanism forcing the brake shoes against the brake drum.  The TSE Type 36 brake chamber has a 36.00 square inch effective area.
AIR COMPRESSOR 
The air compressor provided for the engine shall be a Wabco® SS318 single cylinder pass-through drive type compressor which shall be capable of producing 18.7 CFM at 1200 engine RPMs.  The air compressor shall feature a higher delivery efficiency translating to more air delivery per horsepower absorbed.  The compressor shall include an aluminum cylinder head which shall improve cooling, reduce weight and decrease carbon formation. Superior piston and bore finishing technology shall reduce oil consumption and significantly increasing the system component life.
AIR GOVERNOR 
An air governor shall be provided to control the cut-in and cut-out pressures of the engine mounted air compressor.  The governor shall be calibrated to meet FMVSS requirements. The air governor shall be located on the air dryer bracket. 
MOISTURE EJECTORS 
Heated, automatic moisture ejectors with a manual drain provision shall be installed on all reservoirs of the air supply system. The manual drain provision shall include an actuation pull cable coiled and tied at each drain valve.  The supplied cables when extended shall be sufficient in length to allow each drain to be activated from the side of the apparatus.
AIR SUPPLY LINES 
The air system on the chassis shall be plumbed with color coded reinforced nylon tubing air lines.  The primary (rear) brake line shall be green, the secondary (front) brake line red, the parking brake line orange and the auxiliary (outlet) will be blue.   
Push to connect type fittings shall be used on the nylon tubing.  All drop hoses shall include fiber reinforced neoprene covered hoses.
AIR INLET CONNECTION 
An air connection for the shoreline air inlet shall be supplied.
The air inlet shall be installed in the left-hand side lower front step in the forward position.
A quick release air outlet female connector shall be installed in the mid-section of the left lower cab step towards the front of the cab for the use of auxiliary air tools.  The air outlet connector shall be compatible with a Milton 787, Parker Hannifin B13 or Meyers 54-410 connector.
The cab mounted air outlet connection shall be plumbed to the chassis auxiliary air system reservoir.
The air connector supplied shall be a 0.25-inch size Tru-Flate Interchange style manual connection which is compatible with Milton ‘T’ style, Myers 0.25-inch Automotive style and Parker 0.25-inch 10 Series connectors.
AIR TANK SPACERS
The air tanks shall be moved 3.00 inches inward towards the center of the chassis from the standard position when mounted in the frame rail. This shall provide clearance between the air tanks and the frame for body U-bolt clearance.
WHEELBASE 
The chassis wheelbase shall be 184.00 inches.
REAR OVERHANG
The chassis rear overhang shall be 47.00 inches.

FRAME 
The frame shall consist of double rails running parallel to each other with cross members forming a ladder style frame.  The frame rails shall be formed in the shape of a "C" channel, with the outer rail measuring 10.25 inches high X 3.50 inches deep upper and lower flanges X 0.38 inches thick with an inner channel of 9.44 inches high X 3.13 inches deep and 0.38 inches thick. Each rail shall be constructed of 110,000 psi minimum yield high strength low alloy steel.  Each double rail section shall be rated by a Resistance Bending Moment (RBM) minimum of 3,213,100-inch pounds and have a minimum section modulus of 29.21 cubic inches. The frame shall measure 35.00 inches in width.

Proposals calculating the frame strength using the “box method” shall not be considered.

Proposals including heat treated rails shall not be considered.  Heat treating frame rails produces rails that are not uniform in their mechanical properties throughout the length of the rail.  Rails made of high strength, low alloy steel are already at the required yield strength prior to forming the rail.

A minimum of seven (7) fully gusseted 0.25-inch-thick cross members shall be installed.  The inclusion of the body mounting, or bumper mounting shall not be considered as a cross member. The cross members shall be attached using zinc coated grade 8 fasteners. The bolt heads shall be flanged type, held in place by distorted thread flanged lock nuts.  Each cross member shall be mounted to the frame rails utilizing a minimum of 0.25-inch-thick gusset reinforcement plates at all corners balancing the area of force throughout the entire frame.  

.  

Any proposals not including additional reinforcement for each cross member shall not be considered.  

All relief areas shall be cut in with a minimum 2.00-inch radius at intersection points with the edges ground to a smooth finish to prevent a stress concentration point.

REAR TOW DEVICE
The frame rails shall contain (6) holes per frame in a pattern specified by the OEM for mounting Spartan ERV tow eyes at the rear of the frame at a location defined by the OEM.
FRAME FINISH   
The frame shall be hot dip galvanized prior to assembly and attachment of any components.  The components that shall be galvanized shall include:

· Main frame “C” channels

· Front splayed rails and fish plates

· Cross members (excluding suspension cross members)

· Cross member gussets

· Fuel tank mounting brackets

· Fuel tank straps 

· Air tank mounting brackets 

· Exhaust mounting brackets

· Air dryer bracket

· Radiator skid plate

· Battery supports

· Battery trays 

· Battery covers 

The frame parts which are not galvanized shall be powder coated prior to any attachment of components.  Parts which shall be powder coated shall include but are not limited to:

· Bumper extensions

· Steering gear bracket

· Air tanks 

Other non-galvanized under carriage components which are received from the suppliers with coatings already applied shall include but are not limited to:

· Suspension components

· Front and rear axles

All powder coatings, primers and paint used on the non-galvanized components shall be compatible with all metals, pretreatments and primers used.  The cross-hatch adhesion test per ASTM D3359 shall not have a failure of more than ten (10) squares.  The pencil hardness test per ASTM D3363 shall have a final post-curved pencil hardness of H-2H.  The direct impact resistance test per ASTM D2794 shall have an impact resistance of 120.00 inches per pound at 2 mils.
FRAME SEALANT
There shall be an RTV type sealant applied to the seams between the frame rail and the frame liner to help prevent water intrusion between the frame rails.  The sealant shall be applied to all seams along the length of the frame and at the top, front, and rear ends of the liner.  The sealant shall be applied after the frame rails have been assembled and painted.
FRAME ASSEMBLY STRUCTURAL WARRANTY
Purchaser shall receive a Frame Assembly Structural Fifty (50) Years or 250,000 Miles limited warranty in accordance with, and subject to, warranty certificate RFW0305. The warranty certificate is incorporated by reference into this proposal, and included with this proposal or available upon request.
FRAME RAIL CORROSION WARRANTY
Purchaser shall receive a Frame Rail Corrosion (Zinc Plate and Powder Coat) Twenty-Five (25) Years or 150,000 Miles limited warranty in accordance with, and subject to, warranty certificate RFW0316.  The warranty certificate is incorporated by reference into this proposal, and included with this proposal or available upon request.
FRAME COMPONENTS CORROSION WARRANTY
Purchaser shall receive a Frame Components Corrosion (Zinc Plate) Twenty (20) Years or 132,000 Miles limited warranty in accordance with, and subject to, warranty certificate RFW0314.  The warranty certificate is incorporated by reference into this proposal, and included with this proposal or available upon request.
FRONT BUMPER 
The chassis shall be equipped with a severe duty front bumper constructed from structural steel channel. The bumper material shall be 0.38 thick ASTM A36 steel which shall measure 12.00 inches high with a 3.05-inch flange and shall be 99.00 inches wide with angled front corners.

The front bumper shall be extended approximately 21.00 inches ahead of the cab.
The front bumper shall be painted the same as the lower cab color.
FRONT BUMPER APRON 
The 21.00 inch extended front bumper shall include an apron constructed of 0.19-inch-thick embossed aluminum tread plate.  

The apron shall be installed between the bumper and the front face of the cab affixed using stainless steel bolts attaching the apron to the top bumper flange.
FRONT BUMPER COMPARTMENT CENTER
The front bumper shall include a compartment in the bumper apron located in the center between the frame rails which may be used as a hose well. The compartment shall be constructed of 0.13 inch 5052-H32 grade aluminum and shall include drain holes in the bottom corners to allow excess moisture to escape. The compartment shall be the full size of available space in the apron from the cab fascia to the bumper and 38.00 inches wide X 12.00 inches deep. The clear opening shall be 37.75 inches wide. The front edge of the compartment shall include a rolled edge and angled deflector to prevent hose and couplings from catching along the front edge of the compartment. The compartment shall also include a cover constructed of 0.19-inch-thick bright embossed aluminum tread plate.
The front bumper compartment cover shall include gas cylinder stays which shall hold the cover open.  Each cover shall be held in the closed position via a D-ring style latch.
MECHANICAL SIREN
The front bumper shall include an electro mechanical Federal Q2B™ siren, which shall be streamlined, chrome-plated and shall produce 123 decibels of sound at 10.00 feet.   The Q2B™ siren produces a distinctive warning sound that is recognizable at long distances. A unique clutch design provides a longer coast down sound while reducing the amp draw to 100 amps.  The siren shall include a pedestal mount to surface mount on a horizontal surface.  
The siren shall be pedestal mounted on the bumper apron on the furthest outboard section of the bumper on the driver side.
AIR HORNS
The front bumper shall include two (2) Hadley brand E-Tone air horns which shall measure 21.00 inches long with a 6.00-inch round flare.  The air horns shall be trumpet style with a chrome finish on the exterior and a painted finish deep inside the trumpet.
The air horns shall be recess mounted in the front bumper face, one (1) on the right side of the bumper in the inboard position relative to the right-hand frame rail and one (1) on the left side of the bumper in the inboard position relative to the left-hand frame rail.
AIR HORN RESERVOIR 
One (1) air reservoir, with a 2084 cubic inch capacity, shall be installed on the chassis to act as a supply tank for operating air horns.  The reservoir shall be isolated with a 90 PSI pressure protection valve on the reservoir supply side to prevent depletion of the air to the air brake system.
ELECTRONIC SIREN SPEAKERS
There shall be two (2) Federal Signal Inc. Dynamax® model ES100C, 100-watt speakers provided.  Each speaker shall measure 5.90 inches tall X 5.50 inches wide X 2.30 inches deep.  Each speaker shall include a Federal Signal “Electric F” style grille which shall measure 6.61 inches tall X 6.78 inches wide.
The two (2) electronic siren speakers shall be located on the front bumper face outboard of the frame rails with one (1) on the right side and one (1) on the left side in the outboard positions.
FRONT BUMPER TOW HOOKS 
Two (2) heavy duty tow hooks, painted to match the frame components, shall be installed below the front bumper in the forward position, bolted directly to the underside of each chassis frame rail with grade 8 bolts.
CAB TILT SYSTEM 
The entire cab shall be capable of tilting approximately 45-degrees to allow for easy maintenance of the engine and transmission.   The cab tilt pump assembly shall be located on the right side of the chassis above the battery box.

 

The electric-over-hydraulic lift system shall include an ignition interlock and red cab lock down indicator lamp on the tilt control which shall illuminate when holding the “Down” button to indicate safe road operation.  

It shall be necessary to activate the master battery switch and set the parking brake in order to tilt the cab. As a third precaution the ignition switch must be turned off to complete the cab tilt interlock safety circuit.

Two (2) spring-loaded hydraulic hold down hooks located outboard of the frame shall be installed to hold the cab securely to the frame.  Once the hold-down hooks are set in place, it shall take the application of pressure from the hydraulic cab tilt lift pump to release the hooks.  

Two (2) cab tilt cylinders shall be provided with velocity fuses in each cylinder port.  The cab tilt pivots shall be 1.90-inch ball and be anchored to frame brackets with 1.25-inch diameter studs. 

 

A steel safety channel assembly, painted safety yellow shall be installed on the right-side cab lift cylinder to prevent accidental cab lowering.  The safety channel assembly shall fall over the lift cylinder when the cab is in the fully tilted position.  A cable release system shall also be provided to retract the safety channel assembly from the lift cylinder to allow the lowering of the cab.
CAB TILT AUXILIARY PUMP 
A manual cab tilt pump module shall be attached to the cab tilt pump housing/power distribution box.  
CAB TILT LIMIT SWITCH
A cab tilt limit switch shall be installed.  The switch will effectively limit the travel of the cab when being tilted. The limit adjustment of the switch shall be preset by the chassis manufacturer to prevent damage to the cab or any bumper mounted option mounted in the cab tilt arc. Further adjustment to the limit by the apparatus manufacturer shall be available to accommodate additional equipment. 
CAB TILT CONTROL RECEPTACLE
A 25.00-foot cab tilt control harness shall be provided on the right side of frame just behind the cab. This harness shall consist of an 8.00-foot harness connected to the tilt pump and a 17.00-foot extension harness with a six (6) pin Deutsch connector with cap for mounting in a compartment in the body.

The remote-control pendant shall include 20.00 feet of cable with a mating Deutsch connector. The remote-control pendant shall be shipped loose with the chassis.
CAB TILT LOCK DOWN INDICATOR
The cab dash shall include a message located within the dual air pressure gauge which shall alert the driver when the cab is unlocked and ajar. The alert message shall cease to be displayed when the cab is in the fully lowered position and the hold down hooks are secured and locked to the cab mounts.

In addition to the alert message an audible alarm shall sound when the cab is unlocked and ajar with the parking brake released. 
CAB WINDSHIELD
The cab windshield shall have a surface area of 2825.00 square inches and be of a two (2) piece wraparound design for maximum visibility. 

The glass utilized for the windshield shall include standard automotive tint. The left and right windshield shall be fully interchangeable thereby minimizing stocking and replacement costs. 

Each windshield shall be installed using black self-locking window rubber. 
GLASS FRONT DOORS
The front cab doors shall include a window which is 27.00 inches in width X 26.00 inches in height.  These windows shall have the capability to roll down completely into the door housing.  This shall be accomplished using electric actuation.  The left and right front door windows shall be controlled using a switch on each respective side inner door panel. The driver’s door shall include a switch for each powered door window in the cab.
There shall be an irregular shaped fixed window which shall measure 2.50 inches wide at the top, 8.00 inches wide at the bottom X 26.00 inches in height, more commonly known as “cozy glass” ahead of the front door roll down windows.

The windows shall be mounted within the frame of the front doors trimmed with a black ring on the exterior.
GLASS REAR DOORS
The rear crew doors shall include a window which is 27.00 inches in width X 26.00 inches in height.  The window shall be a powered type and shall be controlled by a switch on the door panel ledge and on the driver’s control panel.
GLASS TINT
The windows located in the cab shall include a dark gray automotive tint which shall allow forty-five percent (45%) light transmittance. The dark tint shall aid in cab cooling and help protect passengers from radiant solar energy.
CABIN AIR FILTRATION SYSTEM

An Active Air Purification system will be installed in the cab. The system utilizes RGF’s Photohydroionization® Cell (PHI-Cell®) technology which produces hydro-peroxides and hydroxide ions, reducing airborne mold, bacteria, viruses, and odors up to 99%. 

The system shall include a stainless-steel housing approximately 7.50 inches high X 16.13 inches wide X 6.6 inches deep in a trapezoid shape and shall be located at the upper portion of the rear wall mounted in a horizontal orientation. The system shall be 12V DC and shall be active either when the ignition power is on, or when the shoreline is connected.
CLIMATE CONTROL
The cab shall include a 57,500 BTU @ 425 CFM front overhead heater/defroster which shall be provided and installed above the windshield between the sun visors.

The cab shall also include a combination heater air-conditioning unit mounted on the engine tunnel. This unit shall offer eight (8) adjustable louvers, four (4) forward facing and four (4) rearward facing, a temperature control valve and two (2) blowers offering three (3) speeds which shall be capable of circulating 550 cubic feet of air per minute. The unit shall be rated for 42,500 BTU/Hr. of cooling and 36,000 BTU/Hr. of heating. 

All defrost/heating systems shall be plumbed with one (1) seasonal shut-off valve at the front corner on the right side of the cab.

The air conditioner lines shall be a mixture of custom bend zinc coated steel fittings and Aeroquip flexible hose with Aeroquip EZ clip fittings.
CLIMATE CONTROL DRAIN
The climate control system shall include a gravity drain for water management.  The gravity drain shall remove condensation from the air conditioning system without additional mechanical assistance.
CLIMATE CONTROL ACTIVATION

The heating and defrosting controls shall be located on the front overhead climate control unit.  There shall be additional heating and air conditioning controls located on the engine tunnel mounted climate control unit.
A/C CONDENSER LOCATION

A roof mounted A/C condenser shall be installed centered on the cab forward of the raised roof against the slope rise.
A/C COMPRESSOR

The air-conditioning compressor shall be a belt driven, engine mounted, open type compressor that shall be capable of producing a minimum of 32,000 BTU at 1500 engine RPMs. The compressor shall utilize R-134A refrigerant and PAG oil.
CAB CIRCULATION FANS FRONT

The cab shall include two (2) all metal 6.00-inch air circulation fans installed in the outer front cab corners. Each fan shall be controlled by an individual virtual button on the vehicle display and control screen. The fans can be used to help defog the windshield or to increase air circulation for passenger comfort.
UNDER CAB INSULATION

The underside of the cab tunnel surrounding the engine shall be lined with multi-layer insulation, engineered for application inside diesel engine compartments.

The insulation shall act as a noise barrier, absorbing noise thus keeping the decibel level in the cab well within NFPA recommendations. As an additional benefit, the insulation shall assist in sustaining the desired temperature within the cab interior.

The engine tunnel insulation shall measure approximately 0.30 inch thick including a multi-layer foil faced glass cloth and polyester fiber layer. The foil surface acts as protection against heat, moisture and other contaminants. The insulation shall meet or exceed FMVSS 302 flammability test.

The insulation shall be cut precisely to fit each section and sealed for additional heat and sound deflection. The insulation shall be held in place by acrylic pressure sensitive adhesive.
INTERIOR TRIM FLOOR

The floor of the cab shall be covered with a multi-layer mat consisting of 0.25-inch-thick sound absorbing closed cell foam with a 0.06-inch-thick non-slip vinyl surface with a pebble grain finish. The covering shall be held in place by a pressure sensitive adhesive and a cast aluminum trim piece at each cab door opening. All exposed seams shall be sealed with silicone caulk matching the color of the floor mat to reduce the chance of moisture and debris retention.
INTERIOR TRIM 

The cab interior shall include trim on the front ceiling, rear crew ceiling, and the cab walls.  It shall be easily removable to assist in maintenance.  The trim shall be constructed of insulated vinyl over a hard board backing.  
REAR WALL INTERIOR TRIM

The rear wall of the cab shall be trimmed with aluminum sheet metal coated with a customer specified interior paint or protective coating.
HEADER TRIM

The cab interior shall feature header trim over the driver and officer dash constructed of 5052-H32 Marine Grade, 0.13-inch-thick aluminum.
TRIM CENTER DASH 

The main center dash area shall be constructed of 5052-H32 Marine Grade, 0.13-inch-thick aluminum plate. There shall be four (4) holes located on the top of the dash near each outer edge of the electrical access cover for ventilation. The center dash electrical access cover shall include a gas cylinder stay which shall hold the cover open during maintenance.
TRIM LH DASH

The left-hand dash shall be constructed of 5052-H32 Marine Grade, 0.13-inch-thick aluminum plate for a perfect fit around the instrument panel. For increased occupant protection the extreme duty left hand dash utilizes patent pending break away technology to reduce rigidity in the event of a frontal crash. The left-hand dash shall offer lower vertical surface area to the left and right of the steering column to accommodate control panels.
TRIM RH DASH

The right-hand dash shall be constructed of 5052-H32 Marine Grade, 0.13 of an inch thick aluminum plate and shall include a glove compartment with a hinged door and a Mobile Data Terminal (MDT) provision. The glove compartment size will measure 14.00 inches wide X 4.50 inches high X 5.88 inches deep. The MDT provision shall be provided above the glove compartment.
ENGINE TUNNEL TRIM

The cab engine tunnel shall be covered with a multi-layer mat consisting of 0.25-inch closed cell foam with a 0.06-inch-thick non-slip vinyl surface with a pebble grain finish. The mat shall be held in place by pressure sensitive adhesive. The engine tunnel mat shall be trimmed with anodized aluminum stair nosing trim for an aesthetically pleasing appearance.
POWER POINT DASH MOUNT
The cab interior shall include (2) Blue Sea dual universal serial bus (USB) charging receptacles in the cab dash switch panel to provide a power source for USB chargeable electrical equipment.  The USB port shall be capable of a 5 Volt-4.8-amp total output. The receptacles shall be wired battery direct.
STEP TRIM

Each cab entry door shall include a three-step entry. The first step closest to the ground shall be constructed of SAE 304 stainless steel with embossed perforations and diamond shaped cutout.  The perforations and cutouts shall allow water and other debris to flow through rather than becoming trapped within the stepping surface. The stainless-steel material shall have a number 8 mirror finish.  The lower step shall be mounted to a frame which is integral with the construction of the cab for rigidity and strength.  The middle step shall be integral with the cab construction and shall be trimmed in 0.08 inch thick 3003-H22 embossed aluminum tread plate.
UNDER CAB ACCESS DOOR

The cab shall include an aluminum access door in the left crew step riser painted to match the cab interior paint with a push and turn latch. The under-cab access door shall provide access to the diesel exhaust fluid fill.   
INTERIOR DOOR TRIM

The interior trim on the doors of the cab shall consist of an aluminum panel constructed of Marine Grade 5052-H32 0.13 of an inch thick aluminum plate.  The door panels shall include a painted finish.
DOOR TRIM CUSTOMER NAMEPLATE

The interior door trim on the front doors shall include a customer nameplate which states the vehicle was custom built for their department, city, township, or county. 
CAB DOOR TRIM REFLECTIVE
The interior of each door shall include high visibility reflective tape. A white reflective tape shall be provided vertically along the rear outer edge of the door. The lowest portion of each door skin shall include a reflective tape chevron with red and white stripes and a Spartan logo. The chevron tape shall measure 6.00 inches in height. 
INTERIOR GRAB HANDLE "A" PILLAR

There shall be two (2) rubber covered 11.00-inch grab handles installed inside the cab, one on each “A” post at the left and right door openings. The left handle shall be located 7.88 inches above the bottom of the door window opening and the right handle shall be located 2.88 inches above the bottom of the door window opening. The handles shall assist personnel in entering and exiting the cab.
INTERIOR GRAB HANDLE FRONT DOOR

Each front door shall include one (1) ergonomically contoured 9.00-inch cast aluminum horizontal grab handle which shall be located at the upper-most center of the door panel, and one (1) 9.00-inch vertical grab handle which shall be located outboard of the paddle latch at the center of the door panel.  Each handle shall feature a DA sand finish and assist personnel entering and exiting the cab.
INTERIOR GRAB HANDLE REAR DOOR
A black powder coated cast aluminum assist handle shall be provided on the inside of each rear crew door. A 30.00-inch-long handle shall extend horizontally the width of the window just above the window sill. The handle shall assist personnel in exiting and entering the cab.
INTERIOR SOFT TRIM COLOR
The cab interior soft trim surfaces shall be black in color.
INTERIOR TRIM SUN VISOR 

The header shall include two (2) sun visors, one (1) on each side forward of the driver and officer seating positions above the windshield. The sun visors shall be constructed of impact resistant, transparent acrylic polycarbonate sun visors with a smoke gray tint. 

The see thru visors are designed for maximum flexibility of positioning utilizing an arm with virtually unlimited adjustability with lateral travel of the tinted visor at the end of the arm which can be locked in place by a thumbscrew.

The visors are easily adjusted and can be placed into a chosen position with one hand.  The sun visors will help protect vehicle occupants from solar glare without obscuring their vision.
INTERIOR FLOOR MAT COLOR

The cab interior floor mat shall be black in color.
CAB PAINT INTERIOR DOOR TRIM
The inner door panel surfaces shall be painted with multi-tone onyx black texture finish.
HEADER TRIM INTERIOR PAINT

The metal surfaces in the header area shall be coated with multi-tone onyx black texture finish.
TRIM CENTER DASH INTERIOR PAINT

The entire center dash shall be coated with multi-tone onyx black texture finish. Any accessory pods attached to the dash shall also be painted this color.
TRIM LH DASH INTERIOR PAINT

The left-hand dash shall be painted with a multi-tone onyx black texture finish.
TRIM RIGHT HAND DASH INTERIOR PAINT

The right-hand dash shall be painted with multi-tone onyx black texture finish.
REAR WALL INTERIOR PAINT

The rear wall of the cab shall be trimmed with aluminum sheet metal coated with a multi-tone onyx black texture finish.
DASH PANEL GROUP
The main center dash area shall include three (3) removable panels located one (1) to the right of the driver position, one (1) in the center of the dash and one (1) to the left of the officer position. The center panel shall be within comfortable reach of both the driver and officer.
SWITCHES CENTER PANEL

The center dash panel shall include no rocker switches or legends.
SWITCHES LEFT PANEL
The left dash panel shall include five (5) switches. There shall be three (3) across the top of the panel with two (2) below. Two (2) of the top row of switches shall be rocker type and the left one (1) shall be the windshield wiper/washer control switch. The lower switches shall be a rocker type switch.

A rocker switch with a blank legend installed directly above shall be provided for any position not designated by a specific option. The non-designated switches shall be two-position, black switches with a green indicator light. Each blank switch legend can be custom engraved by the body manufacturer. All switch legends shall have backlighting provided.
SWITCHES RIGHT PANEL
The right dash panel shall include no rocker switches or legends.
SEAT BELT WARNING 

A seat belt warning system, integrated with the Vehicle Data Recorder system, shall be installed for each seat within the cab.  The system shall provide a visual warning indicator in the vehicle display and control screen. 

The warning system shall activate when any seat is occupied with a minimum of 60 pounds, the corresponding seat belt remains unfastened, and the park brake is released. The warning system shall also activate when any seat is occupied, the corresponding seat belt was fastened in an incorrect sequence, and the park brake is released. Once activated, the visual indicators and applicable audible alarm shall remain active until all occupied seats have the seat belts fastened.
SEAT MATERIAL
The Bostrom Firefighter seats shall include a covering of extra high strength, wear resistant fabric made of durable low seam Durawear Plus™ ballistic polyester.  A PVC coating shall be bonded to the back side of the material to help protect the seats from UV rays and from being saturated or contaminated by fluids. Durawear Plus™ meets or exceeds specification of the common trade name Imperial 1800. The material meets FMVSS 302 flammability requirements.

SEAT COLOR
All seats supplied with the chassis shall be black in color. All seats shall include red seat belts.
SEAT BACK LOGO
The seat back shall include the manufacturer’s logo.  The logo shall be centered on the standard headrest of the seat back and on the left side of a split headrest.
SEAT DRIVER

The driver's seat shall be an H.O. Bostrom 500 Series Firefighter Sierra model seat.  The seat shall feature eight-way electric positioning.  The eight positions shall include up and down, fore and aft with 8.00 inches of travel, back angle adjustment and seat rake adjustment.  The seat shall feature integral springs to isolate shock.

The seat shall feature an all belts to seat (ABTS) style of safety restraint.  The ABTS feature shall include a three-point shoulder harness with the lap belt, automatic retractor and buckle as an integral part of the seat assembly. The ABTS feature shall also include the RiteHite™ shoulder adjustment feature to provide enhanced comfort and safety by allowing customized seat belt fit.

The minimum vertical dimension from the seat H-point to the ceiling for this belted seating position shall be 35.00 inches measured with the seat height adjusted to the lowest position of travel.

This model of seat shall have successfully completed the static load tests set forth by FMVSS 207, 209, and 210 in effect at the time of manufacture.  This testing shall include a simultaneous forward load of 3000 pounds each on the lap and shoulder belts and twenty (20) times the weight through the center of gravity.  

The materials used in construction of the seat shall also have successfully completed testing with regard to the flammability of materials used in the occupant compartments of motor vehicles as outlined in FMVSS 302, of which dictates the allowable burning rate of materials in the occupant compartments of motor vehicles.
The driver’s seat shall include a standard seat back incorporating the all belts to seat feature (ABTS).  The seat back shall feature a contoured head rest.
The driver’s seat shall be installed in an ergonomic position in relation to the cab dash.
The power seat installed in the cab shall be wired directly to battery power.
OCCUPANT PROTECTION DRIVER

The driver’s position shall be equipped with the IMMI 4Front and RollTek™ Systems which shall secure belted occupants and increase the survivable space within the cab. The 4Front and RollTek™ Systems shall selectively deploy integrated systems to protect against injuries in qualifying frontal impact, and rollover events. 

The Driver’s seating area protection shall include:

· Drivers’ airbag DAB - inflates a steering wheel airbag to protect the head and neck of the driver.
· Driver’s knee airbag DKAB - inflating knee bolster airbags to protect the knees.

· Integrated roll sensor IRS - detects an imminent rollover, activates protective devices and records crash events.

· Integrated belt pretension IBP - device for mechanical and/or electrical seats tightens the seat belt, securing driver in seat and positions driver for contact with seat integrated head cushion side roll airbag.

Inflatable head cushion seat integrated side roll airbag SRA - protects driver's head/neck and shields driver from dangerous surfaces.  
SEAT OFFICER 
The officer's seat shall be an H.O. Bostrom 500 Series Sierra model seat.  The seat shall feature two-way manual adjustment and shall include a tapered and padded seat cushion.  The seat shall also feature integral springs to isolate shock.   

The seat shall feature an all belts to seat (ABTS) style of safety restraint.  The ABTS feature shall include a three-point shoulder harness with the lap belt, automatic retractor and buckle as an integral part of the seat assembly. The ABTS feature shall also include the RiteHite™ shoulder adjustment feature to provide enhanced comfort and safety by allowing customized seat belt fit.

The minimum vertical dimension from the seat H-point to the ceiling for this belted seating position shall be 35.00.

This model of seat shall have successfully completed the static load tests by FMVSS 207, 209, 210 and 302 in effect at the time of manufacture.  This testing shall include a simultaneous forward load of 3000 pounds each on the lap and shoulder belts and twenty (20) times the weight through the center of gravity.  The model of seats shall also have successfully completed the flammability of materials used in the occupant compartments of motor vehicles as outlined in FMVSS 302, of which decides the burning rate of materials in the occupant compartments of motor vehicles.
The officer’s seat shall be installed in an ergonomic position in relation to the cab dash.
OCCUPANT PROTECTION OFFICER

The officer’s position shall be equipped with the IMMI 4Front and RollTek™ Systems which shall secure belted occupants and increase the survivable space within the cab. The 4Front and RollTek™ Systems shall selectively deploy integrated systems to protect against injuries in qualifying frontal impact, and rollover events. 

The Officer’s seating area protection shall include:

· Officer’s knee airbag OKAB - inflating knee bolster airbags to protect the knees.

· Integrated roll sensor IRS - detects an imminent rollover, activates protective devices and records crash events.

· Integrated belt pretension IBP - device for mechanical and/or electrical seats tightens the seat belt, securing officer in seat and positioning officer for contact with seat integrated head cushion side roll airbag.

· Inflatable head cushion seat integrated side roll airbag SRA - protects officer’s head/neck and shields officer from dangerous surfaces.

SEAT BELT ORIENTATION CREW

The crew position seat belts shall follow the standard orientation which extends from the outboard shoulder extending to the inboard hip.
SEAT FORWARD FACING OUTER LOCATION

The crew area shall include two (2) forward facing outboard seats, which include one (1) located next to the outer wall of the cab on the left side of the cab and one (1) located next to the outer wall on the right side of the cab. 

The primary position designation per NFPA 1900 2024 edition, shall only declare the positioning in the cab offers a minimum width of 27.60 inches of shoulder clearance without overlap of any other primary seating position and a minimum of 10.80 inches each side of seat center line. Clear width may be offset from center of seat cushion by up to 3.00 inches. It shall also offer a minimum of 22.00 inches of shoulder width clearance without any overlap of any position.
SEAT CREW FORWARD FACING OUTER

The crew area shall include a seat in the forward-facing outer position which shall be a H.O. Bostrom 500 Series Firefighter model seat.  The seat shall feature a tapered and padded seat, and cushion.  The seat shall be mounted in a fixed position. The seat and cushion shall be hinged and compact in design for additional room.  The seat shall include a “Fold and Hold” feature so that the cushion shall remain in the seated position and simply touched to flip up.

The seat shall feature an all belts to seat (ABTS) style of safety restraint.  The ABTS feature shall include a three-point shoulder harness with the lap belt and automatic retractor as an integral part of the seat assembly.  The buckle portion of the seat belt shall extend from the seat base towards the driver position within easy reach of the occupant.  The ABTS feature shall also include the RiteHite™ shoulder adjustment feature to provide enhanced comfort and safety by allowing customized seat belt fit.

The minimum vertical dimension from the seat H-point to the ceiling for each belted seating position shall be 35.00 inches.

This model of seat shall have successfully completed the static load tests by FMVSS 207/210.  This testing shall include a simultaneous forward load of 3000 pounds each on the lap and shoulder belts and twenty (20) times the weight through the center of gravity.  This model of seat installed in the cab model, as specified, shall have successfully completed the dynamic sled testing using FMVSS 208 as a guide with the following accommodations.  In order to reflect the larger size outfitted firefighters, the test dummy used shall be a 95th percentile hybrid III male weighing 225 pounds rather than the 50th percentile male dummy weighing 165 pounds as referenced in FMVSS 208.  The model of seats shall also have successfully completed the flammability of materials used in the occupant compartments of motor vehicles as outlined in FMVSS 302, of which decides the burning rate of materials in the occupant compartments of motor vehicles.
The forward-facing outer seat shall be mounted inboard from the side wall for additional clearance facing the front of the cab.
OCCUPANT PROTECTION FFO

The forward-facing outer seat positions shall be equipped with the RollTek™ rollover occupant protection system which shall secure occupants, increase the survivable space within the cab and protect against head/neck injuries in the event of a rollover accident.

The system shall function using a microprocessor-controlled, solid-state sensing device which, when the system detects a side roll shall provide instantaneous occupant protection (less than 0.3 seconds from trigger to total deployment) by automatically initiating the following sequence:

1.  The seat belt shall tighten around the occupant.

System Components Shall Include:

· Integrated Roll Sensor IRS - detects an imminent rollover, activates protective devices and records crash events.

· Integrated Belt Pretension IBP with flip-up (non theatre) and fixed mechanical seats - tightens the seat belt around occupant, securing occupant in seat. 

· Integrated Gas Pretension IGP with flip-up theatre style seats - tightens the seat belt around occupant, securing occupant in seat. 
SEAT FORWARD FACING CENTER LOCATION

The crew area shall include one (1) forward facing center crew seat located directly behind the engine tunnel in the center of the cab.
SEAT CREW FORWARD FACING CENTER

The forward-facing center seat shall be a H.O. Bostrom 500 Series Firefighter model seat.  The seat shall feature a tapered and padded seat, and cushion.  The seat shall be mounted in a fixed position. The seat and cushion shall be hinged and compact in design for additional room.  The seat shall include a “Fold and Hold” feature so that the cushion shall remain in the seated position and simply touched to flip up.

The seat shall feature an all belts to seat (ABTS) style of safety restraint.  The ABTS feature shall include a three-point shoulder harness with the lap belt and automatic retractor as an integral part of the seat assembly.  The buckle portion of the seat belt shall extend from the seat base towards the driver position within easy reach of the occupant.  The ABTS feature shall also include the RiteHite™ shoulder adjustment feature to provide enhanced comfort and safety by allowing customized seat belt fit.

The minimum vertical dimension from the seat H-point to the ceiling for each belted seating position shall be 35.00 inches.

This model of seat shall have successfully completed the static load tests by FMVSS 207/210.  This testing shall include a simultaneous forward load of 3000 pounds each on the lap and shoulder belts and twenty (20) times the weight through the center of gravity.  This model of seat installed in the cab model, as specified, shall have successfully completed the dynamic sled testing using FMVSS 208 as a guide with the following accommodations.  In order to reflect the larger size outfitted firefighters, the test dummy used shall be a 95th percentile hybrid III male weighing 225 pounds rather than the 50th percentile male dummy weighing 165 pounds as referenced in FMVSS 208.  The model of seats shall also have successfully completed the flammability of materials used in the occupant compartments of motor vehicles as outlined in FMVSS 302, of which decides the burning rate of materials in the occupant compartments of motor vehicles.
OCCUPANT PROTECTION FFC

The forward facing center seat positions shall be equipped with the RollTek™ rollover occupant protection system which shall secure occupants, increase the survivable space within the cab and protect against head/neck injuries in the event of a rollover accident.

The system shall function using a microprocessor-controlled, solid-state sensing device which, when the system detects a side roll shall provide instantaneous occupant protection (less than 0.3 seconds from trigger to total deployment) by automatically initiating the following sequence:

1.  The seat belt shall tighten around the occupant.

System Components Shall Include:

Integrated Roll Sensor IRS - detects an imminent rollover, activates protective devices and records crash events.

Integrated Belt Pretension IBP with flip-up (non theatre) and fixed mechanical seats - tightens the seat belt around occupant, securing occupant in seat. 

Integrated Gas Pretension IGP with flip-up theatre style seats - tightens the seat belt around occupant, securing occupant in seat. 

SCBA SEAT BACKS
The officer’s seat and the three (3) crew area seats shall feature a SecureAll™ self-contained breathing apparatus (SCBA) locking system which shall be one bracket model and store most U.S. and International SCBA brands and sizes while in transit or for storage within the seat back.  The bracket shall be easily adjustable for all SCBA brands and cylinder diameters. All adjustment points shall utilize similar hardware and adjustments shall be made with one tool. 

The bracket shall be adjustable to compensate for different cylinder lengths without the use of tools.  The adjustment shall be made by raising a lever and moving the top clamp vertically.
The bracket system shall be free of straps and clamps that may interfere with auxiliary equipment on SCBA units.  The center guide fork shall keep the SCBA tank in place for a safe and comfortable fit in the seat back cavity.  The SCBA unit simply needs to be pushed against the pivot arm to engage the patented auto- locking system.  Once the lock is engaged, the top clamp shall surround the top of the SCBA tank for a secure fit in all directions.

The SecureAll™ shall include a release handle which shall be integrated into the seat cushion for quick and easy release.  This shall eliminate the need for straps or pull cords to interfere with other SCBA equipment. 

The seat back shall include a removable padded cover which shall be provided over the SCBA cavity.
SEAT FRAME FORWARD FACING

The forward-facing center seating positions shall include an enclosed style seat frame located and installed at the rear wall.  The seat frame shall measure 62.38 inches wide X 12.38 inches high X 22.00 inches deep.  The seat frame shall be constructed of Marine Grade 5052-H32 0.19-inch-thick aluminum plate.  The seat box shall be painted with the same color as the remaining interior.
SEAT FRAME FORWARD FACING STORAGE ACCESS

There shall be two (2) access points to the seat frame storage area, one (1) on each side of the seat frame.  Each access point shall be covered by a hinged door which measures 15.00 inches in width X 10.63 inches in height with an opening that measures 13.75 inches wide X 10.00 inches high. 
CAB FRONT UNDERSEAT STORAGE ACCESS

The left and right under seat storage areas shall have a solid aluminum hinged door with non-locking latch.
All underseat storage compartment access doors shall have a multi-tone onyx black texture finish.
WINDSHIELD WIPER SYSTEM 

The cab shall include a triple arm linkage wiper system which shall clear the windshield of water, ice and debris.  There shall be two (2) windshield wipers; each shall be affixed to a radial arm.  The wiper motor shall be activated by an intermittent wiper control located within easy reach of the driver’s position. The windshield wipers shall be interlocked with the park brake allowing activation only when the park brake is released.

There shall be virtual button on the vehicle display and control screen to override the park brake deactivation. This will reset when the park brake is cycled.
ELECTRONIC WINDSHIELD FLUID LEVEL INDICATOR

The windshield washer fluid level shall be monitored electronically. When the washer fluid level becomes low the yellow “Check Message Center” indicator light on the instrument panel shall illuminate and the message center in the dual air pressure gauge shall display a “Check Washer Fluid Level” message.
CAB DOOR HARDWARE

The cab entry doors shall be equipped with exterior pull handles, suitable for use while wearing firefighter gloves. The handles shall be made of a fiber reinforced plastic composite with a black matt finish.

The interior exit door handles shall be flush paddle type with a black finish, which are incorporated into the upper door panel.

All cab entry doors shall include locks which are keyed alike. The door locks shall be designed to prevent accidental lockout.
DOOR LOCKS

Each cab entry door shall include a manually operated door lock. Each door lock may be actuated from the inside of the cab by means of a red knob located on the paddle handle of the respective door or by using a TriMark key from the exterior. The door locks are designed to prevent accidental lock out.
The left and right-side EMS compartments shall feature a manual door lock.
GRAB HANDLES

The cab shall include one (1) 24.00-inch knurled, anti-slip, one-piece exterior assist handle behind each cab door.  The grab handle shall be made of SAE 304 stainless steel and be 1.25-inch diameter to enable non-slip assistance with a gloved hand.
Each assist handle shall include a polished stainless-steel turnout coat hook.
REARVIEW MIRRORS
Retrac Aerodynamic West Coast style dual vision mirror heads model 613315 shall be provided and installed each of the front cab doors. 

The mirrors shall be mounted via 1.00-inch diameter tubular stainless-steel arms to provide a rigid mounting to reduce vibration.

The mirrors shall measure 8.00 inches wide X 19.00 inches high and shall include an integral convex mirror in the mirror head below the flat glass to provide wider field of vision. The flat and convex mirrors shall be motorized with remote horizontal and vertical adjustment. The control switches shall be mounted within easy reach of the driver. The flat and convex mirrors shall be heated for defrosting in severe cold weather conditions.  

The mirror backs shall be constructed of vacuum formed chrome plated ABS plastic housings that are corrosion resistant and shall include an amber marker light. The mirrors shall be manufactured with the finest quality non-glare glass.
The heat for the rearview mirrors shall be controlled through a rocker switch in the mirror control panel on the left side dash.
EXTERIOR TRIM REAR CORNERS
There shall be an overlay of 3003-H22 aluminum tread plate which shall be 0.07 inches thick on the outside corners at the back of the cab.  The overlay shall wrap 1.00 inches forward on the sides of the cab and 12.00 inches inboard on the rear wall.
CAB FENDERS
Full width wheel well liners shall be installed on the extruded cab to limit road splash and enable easier cleaning. Fender shall consist of an inner liner 16.00 inches wide made of ABS composite and an outer fenderette 3.50 inches wide made of SAE 304 polished stainless steel.
MUD FLAPS FRONT
The front wheel wells shall have mud flaps installed on them. The mud flaps shall extend from the outer edge of the wheel well to the inner edge of the wheel well to provide additional protection from road spray.
CAB EXTERIOR FRONT & SIDE EMBLEMS

The cab shall include three (3) manufacturer’s emblems. There shall be one (1) installed on the front air intake grille and one (1) emblem on each of the cab sides.
The cab shall include model designated nameplates on the front driver and officer side doors.
IGNITION

A master battery system with a keyless start ignition system shall be provided. There shall be a three-position rocker switch with off, battery, and ignition positions as well as a stainless-steel etched engine start push-button. The engine start button shall include an illuminated LED halo ring. Both switches shall be mounted to the left of the steering wheel on the dash.  

The engine start switch shall only operate when the master battery and ignition switch is in the “ignition” position.
BATTERIES
The single start electrical system shall include six (6) Harris BCI 31 925 CCA batteries with a 210-minute reserve capacity and 4/0 welding type dual path starter cables per SAE J541.  

The batteries shall be held in place by non-conducting phenolic resin hold down boards.
BATTERY TRAYS
The batteries shall be installed within two (2) steel battery trays located on the left side and right side of the chassis, securely bolted to the frame rails. The battery trays shall be coated with the same material as the frame.  

The battery trays shall include drain holes in the bottom for sufficient drainage of water. A durable, non-conducting, interlocking mat made by Dri-Dek shall be installed in the bottom of the trays to allow for air flow and help prevent moisture build up.
BATTERY BOX COVERS 
Each battery box shall include a steel cover which protects the top of the batteries.  Each cover shall include flush latches which shall keep the cover secure as well as a black powder coated handle for convenience when opening.
BATTERY CABLES
The starting system shall include cables which shall be protected by 275-degree F. minimum high temperature flame retardant loom, sealed at the ends with heat shrink and sealant. 
BATTERY JUMPER STUDS
The starting system shall include battery jumper studs.  These studs shall be located in the forward most portion of the driver's side lower step, 8.00 inches apart. The studs shall allow the vehicle to be jump started, charged, or the cab to be raised in an emergency in the event of battery failure.
ALTERNATOR 
The charging system shall include a 320-amp Leece-Neville 12-volt alternator.  The alternator shall include a self-exciting integral regulator.
STARTER MOTOR

The single start electrical system shall include a Delco brand starter motor. 
BATTERY CONDITIONER 
A Kussmaul Auto Charge Chief 4012 battery conditioner shall be supplied. The battery conditioner shall provide a circuit protected 40-amp output for the chassis batteries and a 20-amp output circuit for accessory loads. The conditioner shall also include a battery temperature sensor.
The battery conditioner shall be mounted in the cab on the forward face of the left-hand mid EMS compartment.
A Kussmaul battery conditioner display with a Digital Status Center shall be integrated into the electrical inlet. The battery conditioner display shall be integrated into the electrical inlet and located via the electrical inlet location 5209 subcategory.
ELECTRICAL INLET

An electrical inlet shall be installed on the left-hand side of cab over the wheel well in the forward position rear of the grab handle in the upper position and above the door entry key pad and standard paint break lines if applicable
A Kussmaul 20 amp super auto-eject electrical receptacle shall be supplied.  It shall automatically eject the plug when the starter button is depressed.
The electrical inlet shall be connected to the battery conditioner.
The electrical inlet connection shall include a red cover.
HEADLIGHTS

The cab front shall include four (4) rectangular LED headlamps with separate high and low beams mounted in bright chrome bezels. Each lamp shall include a heating system that de-ices the headlight.
The headlights shall be located on the front fascia of the cab directly below the front warning lights.
FRONT TURN SIGNALS
The front fascia shall include two (2) Whelen model M6 4.00-inch X 6.00-inch amber LED with clear lens turn signals which shall be installed in a chrome radius mount housing above and outboard of the front warning and head lamps.
SIDE TURN/MARKER LIGHTS

The sides of the cab shall include two (2) Tecniq S170 LED side marker lights which shall be provided just behind the front cab radius corners. The lights shall be amber with chrome bezels.
MARKER AND ICC LIGHTS

In accordance with FMVSS, there shall be five (5) marker lamps on the front of the vehicle designating identification and clearance. There shall be five (5) face mounted lights integrated into the scene light.
HEADLIGHT AND MARKER LIGHT ACTIVATION

The headlights and marker lights shall be controlled via a virtual button on the Vista display. The headlamps and markers lamps shall illuminate to 100% brilliance when the ignition switch is in the "On" position.

The dash lights shall only have a dim setting for night and a bright setting for day which shall be controlled with a virtual dimmer control on the Vista display. The last button state selected before master power is turned off for the will be held in memory, and they will return to that state when master power is turned back on providing button selection was made at least fifteen (15) seconds prior to shut down.
INTERIOR OVERHEAD LIGHTS

The cab shall include a LED dome lamp located over each door. The lights shall include push switches on each lamp to activate both the clear and red portions of the light individually.
The clear portion of each lamp shall be activated by opening the respective door and via the multiplex display.
AUXILIARY DOME LIGHT FRONT CENTER

The cab shall include a LED dome lamp as an auxiliary dome light. The auxiliary dome light shall be located over the engine tunnel. The light shall include push switches to activate both the clear and red portions of the light individually.
FRONT SCENE LIGHT

The front of the cab shall include one (1) HiViz model FireTech FT-B-72-ML-B LED scene light installed on the brow of the cab. The light shall feature (5) five integrated marker lights. 

The housing shall be powder coated black.

The front scene lighting shall be activated by a virtual button on the vehicle display and control screen.
SIDE SCENE LIGHTS

The cab shall include two (2) Whelen model Pioneer PCH2 semi-recess mount lights installed one (1) on each side of the cab.

Each 150-watt lamp head shall incorporate a 12-volt DC Super-LED combination flood/spot light installed in a die-cast aluminum housing.  Each lamp head shall use a collimator/metalized redux spot/flood reflector assembly with Proclera™ silicone optics and a clear non-optic polycarbonate lens. The lens/reflector assembly shall utilize a liquid injected molded silicone gasket to be resistant to water, moisture, dust, and other environmental conditions. The PCH2 shall be vibration resistant. The Pioneer PC boards shall be conformal coated for additional protection. Each combination flood light lamp head shall draw 13.0 amps in spotlight mode and generate 17,750 lumens total. Each lamp head shall measure 4.25 inches in height X 14.00 inches in width. Each lamp head shall be mounted within a semi-recess housing featuring a chrome flange which shall measure 7.92 inches in height X 17.17 inches in width. The lamp heads and brackets shall be powder coated white. 
The scene lighting located on the left and right sides of the cab shall be mounted rearward of the cab “B” pillar in the 10.00 inch raised roof portion of the cab between the front and rear crew doors.
The scene lights shall be activated by two (2) virtual buttons on the vehicle display and control screen, one (1) for each light, and by opening the respective side cab doors.
REAR SCENE LIGHTS

The vehicle shall include multiplex activated rear scene lighting for body builder installed scene lights and body builder installed multiplex output.
The rear scene lighting shall be activated via individual virtual buttons on the Vista display and control screen.
GROUND LIGHTS

Each door shall include a Tecniq T44 LED ground light mounted to the underside of the cab step below each door. The lights shall include a polycarbonate lens, a housing which is vibration welded and LEDs which shall be shock mounted for extended life.
The ground lighting shall be activated when the parking brake is set, by the opening of the door on the respective cab side, and through a virtual button on the vehicle display and control screen.
UNDER BUMPER LIGHTS

There shall be two (2) 4.00-inch round LED ground lights mounted under the bumper. The lights shall include a polycarbonate lens, a housing which is vibration welded, and LEDs which shall be shock mounted for extended life.  The under bumper ground lighting shall be interlocked with the park brake.
LOWER CAB STEP LIGHTS

The middle step located at each door shall include a Tecniq T44 LED light which shall activate with the opening of the respective door. The lights shall include a polycarbonate lens, a housing which is vibration welded and LEDs which shall be shock mounted for extended life.
INTERMEDIATE STEP LIGHTS

The intermediate step well area at the front doors shall include a TecNiq D06 LED light within a chrome housing. The front egress step lights shall provide visibility to the step well area for the first step exiting the vehicle. The Egress step lights shall activate with entry step lighting. 
ENGINE COMPARTMENT LIGHT

There shall be a LED NFPA compliant light mounted under the engine tunnel for area work lighting on the engine. The light shall activate automatically when the cab is tilted.

DO NOT MOVE APPARATUS LIGHT

The front headliner of the cab shall include a flashing red TecNiq K50 LED light clearly labeled "Do Not Move Apparatus".  In addition to the flashing red light, an audible alarm shall be included which shall sound while the light is activated. 

The flashing red light shall be located centered left to right for greatest visibility.   

The light and alarm shall be interlocked for activation when either a cab door is not firmly closed, or an apparatus compartment door is not closed, and the parking brake is released.
MASTER WARNING SWITCH

A master switch shall be included, as a virtual button on the Vista display and control screen which shall be labeled “E Master” for identification. The button shall feature control over all devices wired through it. Any warning device switches left in the “ON” position when the master switch is activated shall automatically power up. 
LIGHTBAR PROVISION

There shall be one (1) light bar installed on the cab roof.  The light bar shall be provided and installed by the chassis manufacturer. The light bar installation shall include a lowered mounting that shall place the light bar just above the junction box and wiring to a control switch on the cab dash.
The lightbar provision shall be for one (1) Whelen brand Freedom IV LED lightbar mounted centered on the front of the cab roof.  The lightbar shall be 72.00 inches in length.  The lightbar shall feature twelve (12) red LED light modules and two (2) clear LED light modules.  The entire lightbar shall feature a clear lens. The clear lights shall be disabled with park brake engaged.  The cable shall exit the lightbar on the right side of the cab.
The light bar shall be controlled by a virtual button on the vehicle display and control screen. This button shall be clearly labeled for identification.
HEADLIGHT FLASHER

An alternating high beam headlight flashing system shall be installed into the high beam headlight circuit which shall allow the high beams to flash alternately from left to right.   

Deliberate operator selection of high beams will override the flashing function until low beams are again selected.  Per NFPA, these clear flashing lights will also be disabled “On Scene” when the park brake is applied.
The flashing headlights shall be activated through a virtual button on the Vista display and control screen.
INBOARD FRONT WARNING LIGHTS

The cab front fascia shall include two (2) Whelen M6 Super LED front warning lights in the left and right inboard positions. The lights shall feature multiple flash patterns including steady burn. The lights shall be mounted to the front fascia of the cab within a chrome bezel. The warning lights shall be set to emit the “TripleFlash 75” in/out flash pattern.
The warning lights mounted on the cab front fascia in the inboard positions shall be red with a clear lens.
OUTBOARD FRONT WARNING LIGHTS

The cab front fascia shall include two (2) Whelen M6 Super LED front warning lights in the left and right outboard positions. The lights shall feature multiple flash patterns including steady burn. The lights shall be mounted to the front fascia of the cab within a chrome bezel. The warning lights shall be set to emit the “TripleFlash 75” in/out flash pattern.
The warning lights mounted on the cab front fascia in the outboard position shall be red with a clear lens.
BUMPER FACE WARNING LIGHTS
The front bumper face shall include two (2) Whelen M6 series 4.31 inches tall X 6.75 inches wide Super LED® warning lights. The warning lights shall feature multiple flash patterns including steady burn.  The lights shall be surface mounted within a chrome bezel. The warning lights shall be set to flash “TripleFlash 75” in/out flash pattern.
The warning lights in the bumper shall be red with clear lenses.
The front bumper face warning lights shall be located between the frame rails in the right and left side outboard positions. 
FRONT WARNING SWITCH

The front warning lights shall be controlled through a virtual control on the vehicle display and control screen.  This switch shall be clearly labeled for identification. 
INTERSECTION WARNING LIGHTS

The chassis shall include two (2) Whelen M6 series Super LED intersection warning lights, one (1) each side. The lights shall feature multiple flash patterns including steady burn. The lights shall be set to flash “TripleFlash 75” I/O flash pattern.
The intersection lights shall be red with a clear lens.
The intersection lights shall be mounted centered front to rear on the flat portion of the side of the bumper tail.
SIDE WARNING LIGHTS

The cab sides shall include two (2) Whelen M6 Super LED warning lights, one (1) on each side. The lights shall feature multiple flash patterns including steady burn for solid colors and multiple flash patterns for split colors. The lights shall be mounted to the sides of the cab within a chrome bezel. The light shall be programmed to emit the "TripleFlash 75” in/out flash pattern.
The warning lights located on the side of the cab shall be red with clear lens.
The warning lights on the side of the cab shall be mounted above the “B” pillar in the highest available position.
AUXILIARY SIDE WARNING LIGHTS

The cab sides shall include two (2) Whelen series M6 Super LED warning lights, one (1) each side, which shall feature multiple flash patterns including steady burn. The warning lights shall be set to flash “TripleFlash 75” in/out flash pattern.
The auxiliary warning lights located on the side of the cab shall be red with clear lens.
The auxiliary warning lights on the side of the cab shall be mounted behind the rear crew doors. 
The auxiliary warning lights shall be centered approximately 36.25 inches up from the bottom of the cab.
SIDE AND INTERSECTION WARNING SWITCH

The side warning lights shall be controlled through a virtual button on the vehicle display and control screen.  This button shall be clearly labeled for identification.
TANK LEVEL LIGHTS

There shall be two (2) FRC MaxVision surface mount water level light strips. The light strips shall feature four (4) colors of LED lights to indicate the fluid level of a tank. The colors from top to bottom shall be green, blue, amber, and red. The tank level lights shall be pre-wired and coiled at rear of the cab for connection to the apparatus by the body builder. 
There shall be water level lights mounted on each side of the cab, centered between the rear cab doors and the rear corners of the cab. 
REAR TRAFFIC ADVISOR
The cab shall have a Whelen TACTL5 Traffic Advisor control head installed and wired in the center dash panel.  

The power to the control head shall be ignition switched and activation shall be dependent upon the state of the controllers switched position upon ignition.

Wiring provisions shall be provided routed to the rear of the frame for OEM installation of up to eight (8) individual traffic advisor warning lights rated at no more than one (1) amp each.
INTERIOR DOOR OPEN WARNING LIGHTS
The interior of each door shall include one (1) red 4.00-inch diameter Tecniq T40 LED warning light located on the door panel. Each light shall activate with a flashing pattern when the respective door is in the open position to serve as a warning to oncoming traffic.

Each door shall also include one (1) 15.87-inch-long X 0.73-inch-tall amber Weldon LED warning light. The light shall be located on the upper portion of the door frame to be visible when a person is standing in front of the door while entering or exiting the cab.  Each light shall activate with a scrolling directional flash pattern which moves from inside to outside when the door is in the open position. This shall serve as an additional warning to oncoming traffic.
SIREN CONTROL HEAD

A Whelen 295HFSC9 electronic siren control head shall be provided.  The siren head shall feature a 200-watt output, wail, yelp, manual siren, and hands-free operation which shall allow the operator to turn the siren on and off from the horn ring if a horn/siren selector switch option is also selected. The siren shall be installed in the switch panel with a location specific to the customer’s needs.
STEERING WHEEL HORN BUTTON SELECTOR SWITCH

A virtual button on the Vista display and control screen shall be provided to allow control of the electric horn or the air horn from the steering wheel horn button.  The horn button selection shall default to the air horn each time the Vista screen power is cycled off and on.
AUDIBLE WARNING LH FOOT SWITCH
A foot switch wired to actuate the mechanical siren shall be supplied for installation in the front section of the cab for driver actuation.
The mechanical siren foot switch shall be a Linemaster model 632-S.
The mechanical siren foot switch shall be located on the left-hand side accessible to the driver between the steering column and the door.
The mechanical siren foot switch shall be positioned outboard of any other foot switch, if applicable.
A 30.00-degree angled foot switch bracket, wide enough to accommodate (2) foot switches, shall be installed outboard of the steering column for specified driver accessible foot switch activations.  
AUDIBLE WARNING RH FOOT SWITCH
A foot switch wired to actuate the mechanical siren shall be supplied for installation in the front section of the cab for officer actuation.
The mechanical siren foot switch shall be a Linemaster model 632-S.
The mechanical siren foot switch shall be temporarily tied up with a coiled wire drop at the firewall inboard for installation by the customer on the right-hand side accessible to the officer. 
An individual 30.00-degree angled bracket shall be shipped loose with the chassis for installation of each officer accessible foot switch by the customer. 
AIR HORN ACTIVATION
The air horn activation shall be accomplished by two (2) lanyard cables, one (1) on the left-hand side accessible to the driver and one (1) on the right-hand side accessible to the officer. An air horn activation circuit shall be provided to the chassis harness pump panel harness connector.
MECHANICAL SIREN BRAKE CONTROLS
Two (2) red back lit push button type momentary type siren brakes shall be provided in the switch panel on the dash.
MECHANICAL SIREN INTERLOCK
The siren shall only be active when master warning switch is on to prevent accidental engagement.
BACK-UP ALARM 
An ECCO model 575 backup alarm shall be installed at the rear of the chassis with an output level of 107 dB. The alarm shall automatically activate when the transmission is placed in reverse.
INSTRUMENTATION 
An ergonomically designed instrument panel shall be provided. Each gauge shall be backlit with LED lamps.  Stepper motor movements shall drive all gauges.  The instrumentation system shall be multiplexed and shall receive ABS, engine, and transmission information over the J1939 data bus to reduce redundant sensors and wiring.

A twenty-eight (28) icon lightbar message center with integral LCD odometer/trip odometer shall be included.  The odometer shall display up to 999,999.9 miles.  The trip odometer shall display 9,999.9 miles. The LCD message center screen shall be capable of custom configuration by the users for displaying certain vehicle status and diagnostic functions.

The instrument panel shall contain the following gauges:

One (1) three-movement gauge displaying vehicle speed, fuel level, and Diesel Exhaust Fluid (DEF) level. The primary scale on the speedometer shall read from 0 to 100 MPH, and the secondary scale on the speedometer shall read from 0 to 160 KM/H. The scale on the fuel and DEF level gauges shall read from empty to full as a fraction of full tank capacity. Red indicator lights in the gauge and an audible alarm shall indicate low fuel or low DEF at 1/8th tank level.  

One (1) three-movement gauge displaying engine RPM, and primary and secondary air system pressures shall be included.  The scale on the tachometer shall read from 0 to 3000 RPM.  The scale on the air pressure gauges shall read from 0 to 150 pounds per square inch (PSI) with a red line zone indicating critical levels of air pressure.  Red indicator lights in the gauge and an audible alarm shall indicate low air pressure.

One (1) four-movement gauge displaying engine oil pressure, coolant temperature, voltmeter, and transmission temperature shall be included.  The scale on the engine oil pressure gauge shall read from 0 to 100 pounds PSI with a red line zone indicating critical levels of oil pressure.  A red indicator light in the gauge and audible alarm shall indicate low engine oil pressure.  The scale on the coolant temperature gauge shall read from 100 to 250 degrees Fahrenheit (°F) with a red line zone indicating critical coolant temperatures.  A red indicator light in the gauge and audible alarm shall indicate high coolant temperature.  The scale on the voltmeter shall read from 9 to 18 volts with a red line zone indicating critical levels of battery voltage.  A red indicator light in the gauge and an audible alarm shall indicate high or low system voltage.  The low voltage alarm shall indicate when the system voltage has dropped below 11.8 volts for more than 120 seconds in accordance with the requirements of NFPA 1901.  The scale on the transmission temperature gauge shall read from 100 to 300 degrees °F with a red line zone indicating critical temperatures.  A red indicator light in the gauge and an audible alarm shall indicate a high transmission temperature.

The light bar portion of the message center shall include twenty-eight (28) LED backlit indicators.  The lightbar shall be split with fourteen (14) indicators on each side of the LCD message screen. The lightbar shall contain the following indicators and produce the following audible alarms when supplied in conjunction with applicable configurations:

RED INDICATORS

Stop Engine - indicates critical engine fault

Air Filter Restricted - indicates excessive engine air intake restriction

Park Brake - indicates parking brake is set

Seat Belt - indicates a seat is occupied and corresponding seat belt remains unfastened

Low Coolant - indicates critically low engine coolant

Cab Tilt Lock - indicates the cab tilt system locks are not engaged.

AMBER INDICATORS

Malfunction Indicator Lamp (MIL) - indicates an engine emission control system fault

Check Engine - indicates engine fault

Check Transmission - indicates transmission fault

Anti-Lock Brake System (ABS) - indicates anti-lock brake system fault

High exhaust system temperature – indicates elevated exhaust temperatures

Water in Fuel - indicates presence of water in fuel filter

Wait to Start - indicates active engine air preheat cycle
Windshield Washer Fluid – indicates washer fluid is low

DPF restriction - indicates a restriction of the diesel particulate filter

Regen Inhibit-indicates regeneration of the DPF has been inhibited by the operator

Range Inhibit - indicates a transmission operation is prevented and requested shift request may not occur.

SRS - indicates a problem in the supplemental restraint system

Check Message - indicates a vehicle status or diagnostic message on the LCD display requiring attention.

GREEN INDICATORS
Left and Right turn signal indicators

ATC - indicates low wheel traction for automatic traction control equipped vehicles, also indicates mud/snow mode is active for ATC system

High Idle - indicates engine high idle is active.

Cruise Control - indicates cruise control is enabled

OK to Pump - indicates the pump is engaged and conditions have been met for pump operations

Pump Engaged - indicates the pump transmission is currently in pump gear

Auxiliary Brake - indicates secondary braking device is active

BLUE INDICATORS

High Beam indicator

AUDIBLE ALARMS

Air Filter Restriction

Cab Tilt Lock

Check Engine

Check Transmission

Open Door/Compartment

High Coolant Temperature

High or Low System Voltage

High Transmission Temperature

Low Air Pressure

Low Coolant Level 

Low DEF Level

Low Engine Oil Pressure

Low Fuel

Seatbelt Indicator

Stop Engine

Water in Fuel

Extended Left/Right Turn Signal On

ABS System Fault
BACKLIGHTING COLOR

The instrumentation gauges and the switch panel legends shall be backlit using red LED backlighting.
AUXILIARY SPEEDOMETER

The dash shall include an auxiliary speedometer which shall feature a digital readout. The primary scale shall match the primary scale of the instrumentation.
RADIO

A Jensen brand heavy-duty radio with weather band, AM/FM stereo receiver and Bluetooth capabilities shall be installed in a customer specified location. Radio shall be the current, commercially available heavy-duty single-DIN automotive model at time of vehicle manufacturing date.
The radio shall be installed in the left-hand overhead position above the driver.
A small antenna shall be located on the left hand side of the cab roof for AM/FM and weather band reception.
There shall be two (2) speakers installed in the front portion of the cab recessed overhead and two (2) speakers installed in the rear portion of the cab overhead. The speakers shall be provided for connection to the sound system.
The radio shall cut-out with the activation of the Master warn switch as well as by a virtual button on the vehicle display and control screen.
CAMERA RIGHT HAND
One (1) Audiovox Voyager heavy duty rearview teardrop shaped chrome plated housing camera shall be mounted on the officer side of the cab below the windshield ahead of the front door at approximately the same level as the cab door handles. The camera display shall activate when the right side turn signal is activated.
CAMERA REAR
One (1) Audiovox Voyager heavy duty box shaped HD camera shall be shipped loose for OEM installation in the body to afford the driver a clear view to the rear of the vehicle.  

The camera system shall include a one-way communication device that shall be an integral part of the rear camera for the use of voice commands directly to the driver. The rear camera display shall activate when the vehicle’s transmission is placed in reverse.
CAMERA DISPLAY
The camera system shall be wired to a single vehicle display and control screen located on the driver’s side dash. The camera system display can be activated through the vehicle display and control screen.
CAMERA SPEAKER
The rear camera shall be wired to speaker(s) in the cab and shall audible to the driver and officer. There shall be a virtual button provided on the Vista display and control panel to deactivate the speaker(s).
COMMUNICATION ANTENNA #1
An antenna base, for use with an NMO type antenna, shall be mounted on the left-hand front corner of the cab roof so not to interfere with light bars or other roof mounted equipment installed by chassis builder. The antenna base shall be an Antenex model MABVT8 made for either a 0.38 inch or 0.75 inch receiving hole in the antenna and shall include 17 foot of RG58 A/U cable with no connector at the radio end of the cable. The antenna base design provides the most corrosion resistance and best power transfer available from a high temper all brass construction and gold-plated contact design. The antenna base shall be chassis builder supplied.
The antenna cable shall be routed from the antenna base mounted on the roof to the area inside the center rocker switch console.
COMMUNICATION ANTENNA #2
An antenna base, for use with and NMO type antenna, shall be installed on the cab. The antenna base shall be an Antenex model MABVT8 and shall include 17.00 foot of RG58 A/U cable with no connector at the radio end of the cable. The antenna shall be mounted on the right-hand front corner of the cab roof so not to interfere with light bars or other roof mounted equipment installed by chassis builder. The antenna base shall be chassis builder supplied.
The antenna cable shall be routed from the antenna base mounted on the roof to the area inside the center rocker switch console.
FIRE EXTINGUISHER 
A 2.50-pound D.O.T approved fire extinguisher with BC rating shall be shipped loose with the cab.
ROAD SAFETY KIT

The cab and chassis shall include one (1) emergency road safety triangle kit.
DOOR KEYS

The cab and chassis shall include a total of four (4) door keys for the manual door locks.
CHASSIS OPERATION MANUAL
The chassis operation manual shall be contained in an on-board USB digital storage device. The chassis operation manual shall be accessible through a USB port provided in the OBD diagnostic panel.
ENGINE AND TRANSMISSION OPERATION MANUALS

The following manuals specific to the engine and transmission models ordered will be included with the chassis in the ship loose items:

(1) Hard copy of the Engine Operation and Maintenance manual with digital copy

(1) Digital copy of the Transmission Operator’s manual

(1) Digital copy of the Engine Owner’s manual
CAB/CHASSIS AS BUILT WIRING DIAGRAMS
The cab and chassis wiring schematics and option wiring diagrams shall be contained in an on-board USB digital storage device. The cab and chassis wiring schematics and option wiring diagrams shall be accessible through a USB port provided in the OBD diagnostic panel.
DRIVELINE LAYOUT CONFIRMATION 
During the design phase of the chassis the Spartan Chassis driveline engineer shall submit the driveline layout to an OEM engineer to review the chassis design for any potential problems integrating the OEM body to the chassis. The OEM engineer shall provide approval to the driveline engineer prior to driveline bills of materials being released.
CHASSIS REQUIRED LABELING
Signs that state "Occupants must be seated and belted when apparatus is in motion" shall be provided.

They shall be visible from each seating position.

There shall be a lubrication plate mounted inside the cab listing the type and grade of lubrication used in the following areas on the apparatus and chassis:

- Engine oil

- Engine Coolant

- Transmission Fluid

- Pump Transmission Lubrication Fluid

- Drive Axle Lubrication Fluid

- Tire Pressures

APPARATUS INFORMATION LABEL
There shall be a high-visibility label installed in a location clearly detectable to the driver while in the seated position.

The label shall indicate the following specified information.

Overall Height (feet and inches)

Overall Length (feet and inches)

Overall GVWR (tons or metric tons)

HELMET RESTRAINTS

All NFPA required helmet restraints will be supplied and installed by the Fire Department prior to the truck being placed into service.

MUD FLAPS
Heavy-duty rubber mud flaps shall be installed behind the rear wheels. The mud flaps shall be black rubber type and be bolted in place.

CAB TILT PENDANT CONTROL

There shall be a cab tilt pendant control provided and installed on the right side of the apparatus. The pendant shall be located directly behind the lower auxiliary pump access panel, accessible through a small hinged door secured with a push button style latch. A label shall be provided that states "CAB TILT".  The cab tilt door shall open towards the rear of body.

There shall also be a cab tilt instruction plate located as close as possible to the control pendant for ease of operation.

AIR TANK DRAIN LINES (extended)
There shall be manual pull air tank drain lines provided with the apparatus. The air drain lines shall be extended to the outer edge of the apparatus to facilitate draining moisture from the chassis air tanks to a single location for all drains and shall be actuated by a key ring. A label shall be affixed indicating “Air Tank Drain”.

HEAT EXCHANGER
A supplementary heat exchanger cooling system shall be provided with the chassis and shall be complete to the discharge side of the fire pump through the engine compartment, without intermixing, for absorption of excess heat. The heat exchanger shall be adequate in size to maintain the temperature of the coolant in the pump drive engine not in excess of the engine manufacturer's temperature rating under all pumping conditions.

Appropriate drains shall be provided to allow draining the heat exchanger to prevent damage from freezing. A manual shut-off valve shall be supplied at the pump operator's position.

FUEL FILL DOOR
There shall be an aluminum fuel fill assembly located on the apparatus body accessing the chassis supplied fuel tank. The assemblies shall be located in the upper area of the rear wheel on the left and right side.

The fuel fill assembly will have a brushed aluminum door. There shall be a drain in the fuel fill assembly to allow over flow to drain on the back side of the apparatus body. The fuel fill cap shall be removable, manufactured of plastic materials, green in color and equipped with a tether.

The fuel fill cap shall be labeled "DIESEL FUEL". The stainless-steel fuel fill neck shall have a 3/8" inside diameter vent line installed from the top of the fuel tank to the fill tube.

SIDE MOUNT PUMP CONTROL MODULE

The pump control module shall be a self-supported structure mounted independently from the body and chassis cab. The pump module frame shall be constructed entirely of 6061-T6 aluminum extrusions and 5052-H32 aluminum plate. The pump module design shall allow normal frame deflection through isolation mounts without imposing stress on the pump module structure or side running boards. The pump module support shall bolt directly to the chassis frame web.

VIBRA-TORQ™ PUMP MODULE MOUNTING SYSTEM
The entire pump module assembly shall be mounted so that it “floats” above the chassis frame rails exclusively with Vibra-Torq™ torsion isolator assemblies to reduce the vibration and stress providing an extremely durable pump module mounting system.

The pump module substructure shall be mounted above the frame to allow independent flexing to occur between the body and the chassis. Each assembly shall be mounted to the chassis frame rails with steel, gusseted mounting brackets. Each bracket shall be powder coated for corrosion resistance. Each pump compartment mount bracket shall be mounted to the side chassis frame flange with two 5/8”-UNC Grade 5 HHCS.

Each assembly shall have a two-part rubber vibration isolator. The isolator shall be of a specific durometer to carry the necessary loads of the pump module, apparatus body, equipment, tank, water, and hose. The quantity of mounts utilized shall correspond directly to the anticipated weight being supported. Certain assemblies shall also incorporate a torsion spring. Helical coil springs shall be incorporated into specific mounts in tandem with the rubber isolators to minimize the stress absorbed by the body caused from chassis frame rail flexing.

There shall be no welding to the chassis frame rail sides, web or flanges, or drilling of holes in the top or bottom frame flanges between axles. All pump module to chassis connections shall be bolted so that in the event of an accident, the body shall be easily removable from the truck chassis for repair or replacement.

Because of the constant vibration and twisting action that occurs in chassis frame rails and suspension, the torsion mounting system is required to minimize the possibility of premature pump module structural failures. The Vibra-Torq™ mounting system shall have a lifetime warranty.

PUMP COMPARTMENT WORK LIGHT

One (1) 24.00-inch OnScene Solutions "Access" series tube light shall be installed inside the pump compartment module to illuminate the plumbing and piping components. The light shall be activated by water proof toggle switch installed inside the pump compartment.

PUMP MODULE PANELS
The panels shall be an integral part of the pump module structure.

The driver’s side panels shall consist of a removable lower panel fastened with mechanical fasteners, a hinged operator's panel and a fixed diamond plate panel above the operator's panel.

The officer’s side panels shall consist of a fixed upper diamond plate panel and a lower full height removable access panel to provide ease of entrance for service and maintenance. The panel shall be attached to the module frame utilizing mechanical latching devices.

OPERATOR’S GAUGE PANEL
The operator gauge panel shall be located on the upper portion of the left (drivers) side main pump module. The gauge panel shall be hinged along the bottom edge and shall allow access to the components behind the panel. Cable hold-open straps shall be mounted on the gauge panel and module structure to prevent the panel from being completely detached from the pump module.

PUMP PANEL & OPERATOR'S PANEL FINISH
The pump module panels and the operator's panel shall be brushed stainless steel finish.

LEFT SOFT SUCTION HOSE STORAGE

There shall be a recessed cavity on the left side of the pump compartment module integrated into the side panel. The cavity shall be located rearward of the steamer inlet and be capable of storing a 25-foot roll of 5.00-inch suction hose. The cavity shall be approximately 12 inches wide x 17.5 inches high x 12.5 inches deep. The floor area shall have a light taper downward to assist in restraining the hose. Drain holes shall be provided in the rear corners.

RIGHT SOFT SUCTION HOSE STORAGE

There shall be a recessed cavity on the right side of the pump compartment module integrated into the side panel. The cavity shall be located rearward of the steamer inlet and be capable of storing a 25-foot roll of 5.00-inch suction hose. The cavity shall be approximately 12 inches wide x 17.5 inches high x 12.5 inches deep. The floor area shall have a light taper downward to assist in restraining the hose. Drain holes shall be provided in the rear corners.


SEATBELT HOLD DOWN STRAP

There shall be one (1) seat belt type hose hold down installed on each storage area. The hold down will be used to secure the stored hose in place during transit.

STORAGE CAVITY INTERIOR FINISH

The interior of the storage compartments shall feature a painted medium gray Speedliner finish.

PUMP PANEL LIGHTS

There shall be adequate illumination provided at the side pump panels with the installation of two (2) shielded light assemblies functioning as an intermediate step and installed on a stationary surface, one (1) on the left and one (1) on the right-side pump compartment.

There shall be up to three (3) handhold cutouts provided in the top step surface measuring approximately 2.50 inches deep. There shall be one (1) full length aluminum non lit handrail integrated into each side assembly.

Each shield shall contain two (2) 18-inch OnScene LED "Access" lights.

PUMP PANEL SWITCHING

There shall be a rocker switch located on the operator's pump panel to turn on or off all four (4) pump panel lights. 

One light on the driver's side and one light on the passenger's side pump module shall be activated by "pump engagement “and it is "OK TO PUMP".  

SWITCH HOUSING

All specified lighting fixtures and electrical components activated at the pump operator's panel shall be activated by Carling W-series rocker style switches. 

The switches shall be located on a separate matte black Innovative Controls 6-position electrical panel, complete with backlit name tags describing the function of each individual switch. 

The switches shall be laid out in the following order left to right: Panel Lights, Left Scene, Right Scene, Rear Scene, Blank Space, Air Horn.

VALVE CONTROL - T-HANDLE PULL ASSEMBLY

Unless specified otherwise, the discharge valves shall be controlled from an Innovative Controls side mount valve control assembly. The ergonomically designed handle shall be chrome-plated with recessed areas for name plate and color code. A .75-inch diameter hardcoat anodized aluminum control rod and housing shall, together with a stainless spring steel locking mechanism, eliminate valve drift. Teflon impregnated bronze bushings in both ends of the rod housing shall minimize rod deflection, never need lubrication, and ensure consistent long-term operation. The control assembly shall include a decorative chrome-plated panel-mounting bezel. The valve operating mechanism will indicate the position of the valve at all times.

RUNNING BOARDS

Running boards shall be installed on each side of the pump compartment module. The running boards shall be constructed of .188-inch embossed aluminum diamond plate. Each shall be a minimum of approximately 12.00 inches deep by the width of the module.

The running boards shall have a 1.25-inch upward bend on the inside edge to act as a kick plate.

The aluminum diamond plate shall meet recommendations for slip resistant surfaces at the time of proposal.

The running boards shall be attached to a frame mounted outrigger support structure. Each running board to have a 3.00-inch downward bend on the outboard face with a 1.50-inch underside return for superior strength.

APPARATUS PLUMBING LABELING
Innovative Controls verbiage tag bezels shall be installed. The bezel assemblies will be used to identify apparatus components. These tags shall be designed and manufactured to withstand the specified apparatus service environment and shall be backed by a warranty equal to that of the exterior paint and finish. The verbiage tag bezel assemblies shall include a chrome-plated panel-mount bezel with durable easy-to-read UV resistant polycarbonate inserts featuring the specified verbiage and color coding. These UV resistant polycarbonate verbiage and color inserts shall be subsurface screen printed to eliminate the possibility of wear and protect the inks from fading. Both the insert labels and bezel shall be backed with 3M permanent adhesive, which meets UL969 and NFPA standards.

PRESSURE GOVERNOR AND MONITORING DISPLAY
Fire Research "InControl 400" Series pressure governor and monitoring display kit shall be installed. The kit shall include a control module, intake pressure sensor, discharge pressure sensor, and cables. The control module case shall be waterproof and have dimensions not to exceed 5.50 inches high by 10.50 inches wide by 2.00 inches deep. The control knob shall be 2.00 inches in diameter with no mechanical stops, have a serrated grip, and a red idle push button in the center. It shall not extend more than 1.75 inches from the front of the control module. Inputs for monitored information shall be from a J1939 data bus or independent sensors. Outputs for engine control shall be on the J1939 data bus or engine specific wiring.

The following continuous displays shall be provided:

· Pump discharge; shown with four daylight bright LED digits more than 1/2" high

· Pump Intake; shown with four daylight bright LED digits more than 1/2" high

· Pressure / RPM setting; shown on a dot matrix message display

· Pressure and RPM operating mode LEDs

· Throttle ready LED

· Engine RPM; shown with four daylight bright LED digits more than 1/2" high

· Check engine and stop engine warning LEDs

· Oil pressure; shown on a dual color (green/red) LED bar graph display

· Engine coolant temperature; shown on a dual color (green/red) LED bar graph display

· Transmission Temperature: shown on a dual color (green/red) LED bar graph display

· Battery voltage; shown on a dual color (green/red) LED bar graph display.

The dot-matrix message display shall show diagnostic and warning messages as they occur. It shall show monitored apparatus information, stored data, and program options when selected by the operator. All LED intensity shall be automatically adjusted for day and nighttime operation.

The program shall store the accumulated operating hours for the pump and engine to be displayed with the push of a button. It shall monitor inputs and support audible and visual warning alarms for the following conditions:

· High Battery Voltage

· Low Battery Voltage (Engine Off)

· Low Battery Voltage (Engine Running)

· High Transmission Temperature

· Low Engine Oil Pressure

· High Engine Coolant Temperature

· Out of Water (visual alarm only)

· No Engine Response (visual alarm only).

The program features shall be accessed via push buttons and a control knob located on the front of the control panel. There shall be a USB port located at the rear of the control module to upload future firmware enhancements.

Inputs to the control panel from the pump discharge and intake pressure sensors shall be electrical. The discharge pressure display shall show pressures from 0 to 600 psi. The intake pressure display shall show pressures from -30 in. Hg to 600 psi.

The governor shall operate in two control modes, pressure and RPM. No discharge pressure or engine RPM variation shall occur when switching between modes. A throttle ready LED shall light when the interlock signal is recognized. The governor shall start in pressure mode and set the engine RPM to idle. In pressure mode the governor shall automatically regulate the discharge pressure at the level set by the operator. In RPM mode the governor shall maintain the engine RPM at the level set by the operator except in the event of a discharge pressure increase. The governor shall limit a discharge pressure increase in RPM mode to a maximum of 30 psi. Other safety features shall include recognition of no water conditions with an automatic programmed response and a push button to return the engine to idle. 

PRESSURE RELIEF VALVE
A Task Force Tips model #A18XX pressure relief valve shall be provided. The valve shall have an easy-to-read adjustment range from 90 to 300 PSI with 90, 125, 150, 200, 250 and 300 PSI adjustment settings and an "OFF" position. Pressure adjustments shall be made utilizing a 1/4" hex key, 9/16" socket or 14mm socket.

For corrosion resistance the cast aluminum valve shall be a hardcoat anodized with a powder coat interior and exterior finish. The valve shall meet current NFPA, Standard for Automotive Fire Apparatus, requirements for pump inlet relief valves. The unit shall be covered by a five-year warranty. The valve shall be preset at 125 suction inlet pressure. The valve shall be installed inside the pump compartment where it will be easily accessible for future adjustment. The excess water shall be plumbed to the atmosphere via the unloader pipe and shall dump on the opposite side of the pump operator.

For normal pumping operations, the relief valve shall not be capped and there shall be a placard stating "DO NOT CAP" installed.

UL TEST PORTS
One (1) set of UL testing ports with plugs shall be provided on the pump panel for testing of the vacuum and pump pressures.

WATER TANK LEVEL GAUGE

A Fire Research TankVision model WLA300-A00 tank indicator kit shall be installed on the operator's panel.

The kit shall include an electronic indicator module, a pressure sensor, and a 20' sensor cable. The indicator shall show the volume of water in the tank on nine (9) easy to see super bright LEDs. A wide view lens over the LEDs shall provide for a viewing angle of 180 degrees. The indicator case shall be waterproof, manufactured of aluminum, and have a distinctive blue label.

The program features shall be accessed from the front of the indicator module. The program shall support self-diagnostics capabilities, self-calibration, and a datalink to connect remote indicators. Low water warnings shall include flashing LEDs at 1/4 tank, down chasing LEDs when the tank is almost empty, and an output for an audio alarm.

The indicator shall receive an input signal from an electronic pressure sensor. The sensor shall be mounted from the outside of the water tank near the bottom. No probe shall place on the interior of the tank. Wiring shall be weather resistant and have automotive type plug-in connectors.

AIR HORN SWITCH
The air horn shall be activated by a red momentary rocker switch provided and installed on the pump operator's gauge panel. The air horn button shall be of weather resistance type and labeled “AIR HORN”.

PUMP COMPARTMENT TOP OVERLAY
The top of the pump compartment shall be overlaid with .188-inch embossed aluminum diamond plate.

MIDSHIP PUMP
The pump shall have a capacity of 1500 gallons per minute, measured in U.S. Gallons. The pump shall be a Waterous model CSU single stage midship pump.

The pumps impellers shall be bronze with double suction inlets, accurately balanced (mechanically and hydraulically), of mixed flow design with reverse-flow, labyrinth-type, wear rings that resist water bypass and loss of efficiency due to wear. The impeller shall have flame plated hub to assure maximum pump life and efficiency despite the presence of abrasive particles, such as fine sand, in the water being pumped. The wear rings shall be bronze and easily replaceable to restore original pump efficiency and eliminate the need for replacing the entire pump casing due to wear.

Pump casing shall be close grained gray iron, bronze fitted and horizontally split in two sections for easy removal of entire impeller assembly, including wear rings, without disturbing setting of pump in chassis or pump piping. The pump, for ease and rapid servicing in the future, shall have the separable impeller shaft which allows true separation of transmission or pump without disassembly or disturbing the other component. This shall be accomplished by using a two-piece shaft. This feature will allow field service to accomplish in much less time since each component (pump or transmission) can be repaired independently. The impeller shaft shall be stainless steel, accurately ground to size and polished. Shaft shall be supported at each end by ball type oil grease lubricated bearings. Sleeve bearings or bushings will not be acceptable. The bearings shall be protected from water at each end of the impeller shaft.

The discharge manifold shall be cast as an integral part of the pump body assembly and shall provide at least three full 3.50-inch openings for ultimate flexibility in providing various discharge outlets for maximum efficiency and shall be located as follows: one outlet on the right side of the pump body, one outlet on the left side of the pump body, and one outlet directly on top of the pump discharge manifold.

The entire pump shall be cast, manufactured and tested at the pump manufacturer's factory. The pump transmission housing shall be high strength aluminum, three pieces and horizontally split. Power transfer to the pump shall be through a Morse Hy-Vo drive chain. Chain shall be pressure lubricated through oil pump. Chain sprockets shall be cut from carbonized, hardened alloy steel. Spur gears will not be acceptable.

The drive shafts shall be 2.35" in diameter, made of hardened and ground alloy steel. All shafts shall be ball bearing supported. Case shall be designed to eliminate the need of water cooling.

The entire pump, both suction and discharge passages, shall be hydrostatically tested to a pressure of 600 PSI. A certificate documenting this test shall be provided with the completed apparatus. The pump shall be fully tested at the pump manufacturer's factory to the performance requirements as outlined by the latest NFPA, Standard for Automotive Fire Apparatus. Pump shall be free from objectionable pulsation and vibration.

The pump shall be the Class "A" type and shall deliver the percentage of rated discharge at pressures indicated below.

100% of rated capacity at 150 PSI net pump pressure.

100% of rated capacity at 165 PSI net pump pressure.

70% or rated capacity at 200 PSI net pump pressure.

50% of rated capacity at 250 PSI net pump pressure.

PUMP HOUSE WIDTH

The width of the pump house shall be 48.00 inches from front to back.

PUMP SEALS

The pump shall be equipped with maintenance free mechanical shaft seals that shall not require manual adjustment. The seal size, type, component materials, and housing configuration shall be specifically designed for the pump application and rated operating parameters as specified.

AIR PRIMER SYSTEM
The priming system shall be a Trident Emergency Products compressed air powered high efficiency, multi-stage, venturi-based Air Prime System.

All wetted metallic parts of the priming system are to be of brass and stainless-steel construction. A single panel mounted control will activate the priming pump and open the priming valve to the pump.

The primer shall be mounted above the pump impeller so that the priming line will automatically drain back to the pump. The primer shall also automatically drain when the panel control actuator is not in operation. The inlet side of the primer shall include a brass ‘wye’ type strainer with removable stainless steel fine mesh strainer to prevent entry of debris into the primer body.

The system shall employ an 80 PSI pressure protection valve, located on the chassis auxiliary air tank.

The primer shall be covered by a five (5) year parts warranty.

6.0" STEAMER INLETS

Two (2) 6.00-inch steamer inlets shall be provided, one (1) on the left side and one (1) on the right side.

Each inlet shall have a chrome plated long handle chrome vented caps and die cast zinc screens designed to provide cathodic protection for the pump. The caps shall be National Standard Thread with long handles.

PUMP COOLING LINE
There shall be a .375-inch line run from the pump to the water tank to assist in keeping the pump water from overheating. A manual Class 1 1/4 turn .25 inch on/off valve with a rectangular handle shall be supplied on the operator's panel.

PUMP ANODES
Two (2) pump anodes shall be installed in plumping system of the apparatus, to prevent damage from galvanic corrosion within the pump system. There shall be one (1) anode on the intake side and one (1) on the discharge side.

MASTER PUMP DRAIN

The pump shall be equipped with a Master Pump drain to allow draining of the lower pump cavities, volute and selected water carrying lines and accessories. The drain shall have an all brass body.

The drain valve control shall be panel mounted and identified as MASTER DRAIN.

DRAIN VALVES

All manual drains shall be Class 1 with .75-inch J-style lift handle kit.

Each drain shall have a 90-degree push lock fitting supply with a 90-degree poly elbow drain. Reinforced clear vinyl tubing shall be utilized to route the water to atmosphere.

VALVES
All valves shall be of a heavy-duty design capable of bi-directional flow and incorporate a self-locking ball feature and full flow optimizing characteristics that reduce the operational force required for actuation.

The valves shall be Akron 8000 series.

The valves shall be of a self-adjusting dual seat design requiring no lubrication or regular maintenance. The valve shall meet or exceed NFPA standard requirements.

PLUMBING

All plumbing and piping shall be of 304 stainless steel or flexible type piping. All inlet and outlet plumbing 3.00 inch and smaller shall be plumbed with either stainless steel piping or synthetic reinforced rubber hose blended with high tensile strength cord for maximum performance in tight bend applications.

Secondary plumbing such as small diameter drain lines shall be stainless steel, brass or hose. Where chassis and module flexing or vibration may damage or loosen piping or where a coupling is required for servicing, the piping shall be equipped with Victaulic or rubber type couplings.

All lines shall drain through the master drain valve or shall be equipped with individual drain valves. All individual drain lines for discharges shall be extended to the point where they shall drain below the chassis frame rails. All water carrying drain lines shall be of flexible polypropylene type tubing.

MANIFOLDS

Plumbing manifold bodies shall be ductile cast iron or stainless steel. The suction inlets shall include removable die cast zinc screens designed to provide cathodic protection for the pump, therefore reducing deterioration within the pump.

TANK FILL 

One (1) 2.00-inch pump to tank fill line shall be installed from the discharge manifold directly to the booster tank.

TANK TO PUMP
One (1) 3.50-inch Waterous valve shall be installed between the water tank and the pump with flow recommendations as set forth by (NFPA) 1901, Standard for Automotive Fire Apparatus, and shall be tested to those standards when the pump is being certified.

There shall be a tank to pump check valve, conforming to NFPA standard requirements to prevent water from back flowing at an excessive rate if the pump is being supplied from a pressurized source.

The check valve shall be mounted as an integral part of the pump suction extension. A hole up to .25 inch is allowable in the check valve to release steam or other pressure buildup so that the void between the valve and check valve may drain of water that could be subject to freezing.

2.5" LEFT SIDE INLET
There shall be one (1) 2.50-inch gated suction inlet with .75-inch bleeder installed on the left side of the apparatus, forward of the steamer.

The inlet shall be controlled at the operator's panel. 

The plumbing shall consist of 2.50-inch piping and shall incorporate a manual drain control installed below the pump area for ease of access.

The termination shall include the following components: one (1) 2.50-inch NST swivel female straight adapter with screen and one (1) 2.50-inch self-venting plug, secured by a cable

2.5" LEFT SIDE DISCHARGE
There shall be one (1) 2.50-inch gated discharge installed on the left side of the apparatus in the forward position.  The discharge shall be controlled with a rack & sector control with a rod bezel.

The plumbing shall consist of 2.50-inch piping and shall incorporate a manual drain control installed below the pump area for ease of access.

The discharge termination shall include the following components: one (1) 2.50-inch male NST adapter, one (1) 2.50-inch NST female swivel by male with 30-degree polished elbow and one (1) 2.50-inch female self-venting cap, secured by a cable

2.5" RIGHT SIDE DISCHARGE
There shall be one (1) 2.50-inch gated discharge installed on the right side of the apparatus.

The plumbing shall consist of 2.50-inch piping and shall incorporate a manual drain control installed below the pump area for ease of access.

The discharge termination shall include the following components: one (1) 2.50-inch male NST adapter, one (1) 2.50-inch NST female swivel by male with 30-degree polished elbow and one (1) 2.50-inch female self-venting cap, secured by a cable

4.0" RIGHT SIDE DISCHARGE
There shall be one (1) gated 4.00-inch discharge with a 3.00-inch ball valve installed on the right side of the apparatus.

The plumbing shall consist of 4.00-inch piping and shall incorporate a manual drain control installed below the pump area for ease of access.

The discharge termination shall include the following components: one (1) 4.00-inch NST adapter, one (1) 4.00-inch NST female by 5.00-inch Storz with 30-degree elbow and one (1) 5.00-inch Storz cap, secured by a cable

2.5"LEFT REAR DISCHARGE

There shall be one (1) 2.50-inch discharge located on the left side at the rear of the vehicle.

The plumbing shall consist of 2.50-inch piping and shall be plumbed from the right-hand discharge manifold, and shall incorporate a manual drain control installed below the pump area for ease of access.

The discharge termination shall include the following components: one (1) 2.50-inch male NST adapter, one (1) 2.50-inch NST female swivel by male with 30-degree polished elbow and one (1) 2.50-inch female self-venting cap, secured by a cable

3.0" DECK GUN DISCHARGE

There shall be a 3.00-inch deck gun discharge provided.

The deluge waterway shall be plumbed with 3.00-inch piping that terminates in the center location at the top of the pump compartment module.

The plumbing shall be drained with an auto-drain located at the lowest point of the waterway plumbing if required.

EXTEND-A-GUN
There will be a Task Force Tips 18.00-inch manual Extenda-Gun, model XG18VL-PL, installed on the deluge pipe. If the Extenda-Gun is not properly stowed and the parking brake is released, it shall activate the hazard light in the cab to alert the crew.

CROSSLAYS

Two (2) crosslays hose beds shall be located in the upper portion of the pump compartment, toward the front. The crosslay area shall span the entire width of the pump compartment module. Slotted aluminum flooring shall be provided for the hose bed area for drainage.

The two (2) crosslays shall each have capacity for 200 feet of 1.75-inch double jacket fire hose.

The ends of the crosslay dividers shall be cut at a 62-degree angle to provide room for the hand holes cut into the crosslay cover ends.

The plumbing shall consist of 2.00-inch piping and shall incorporate a manual drain control installed below the pump area for ease of access.

The discharge termination shall include the following components: one (1) 2.00-inch NPT x 1.50-inch NST chrome plated brass Chicksan swivel,
The use of a swivel shall allow hose payout to either side of the pump compartment.

2.5" CROSSLAY DISCHARGE

One (1) additional crosslay hose bed shall be provided.

The crosslay shall have capacity for 200 feet of 2.50-inch double jacket fire hose.

The plumbing shall consist of 2.50-inch piping and shall incorporate a manual drain control installed below the pump area for ease of access.

The discharge termination shall include the following components: one (1) 2.5" NPT x 2.5" NST chrome plated brass Chicksan swivel

The use of a swivel shall allow hose payout to either side of the pump compartment.

CROSSLAY COVER

The crosslay hose bed area shall have a .188-inch embossed aluminum diamond plate cover installed. The cover shall be installed to provide a solid surface over all bays. The cover shall have a hand hold slot on each end.  The cover shall be attached with a full-length piano style hinge.

When opened, the diamond plate cover shall rest upon rubber bumpers or an equivalent protective type stop to eliminate marring or scratching of other apparatus body work.

The crosslay hose bed area shall have a cover installed at each end of the crosslay area by the Fire Department or Dealership prior to the apparatus being placed into service.

CROSSLAY HOSE BED LIGHT
One (1) On Scene LED 60" Walkway series waterproof light shall be installed in an anodized aluminum housing on the front of the body to illuminate the crosslay area.

The crosslay light shall be activated with the pump "Panel Lights" switch.

FRONT BUMPER DISCHARGE
One (1) 1.50-inch front bumper discharge outlet shall be provided.

The front bumper discharge plumbing shall consist of 2.00-inch piping and shall incorporate a manual drain control installed below the pump area for ease of access.

Auto-drains shall be installed in the discharge piping at the lowest point of the plumbed system.

The discharge termination shall include the following components: one (1) 2.00-inch NPT x 1.50-inch NST polished stainless steel Chicksan swivel.

The use of a swivel shall allow hose payout to either side of the apparatus.

The front bumper discharge shall be mounted on top of the gravel shield of the front bumper extension. The discharge shall be placed to the right of the hose well.

The front bumper discharge Chicksan swivel shall include a guard installed on the front bumper gravel shield to prevent the swivel from hitting the cab. The guard shall be fabricated of smooth aluminum with a dual-action sanded finish on all sides. There shall be two (2) rubber bumper stops installed on the guard to protect the chrome swivel.

HOSE REEL VALVE PREPLUMB
The reel shall be plumbed to the pump with a 1.50-inch quarter turn Akron 8815 ball valve and 1.00-inch-high pressure hose and couplings. The valve shall be controlled from the operator's panel with a push-pull control.

A discharge gauge shall be included for the booster reel.

DISCHARGE GAUGES

An Innovative Controls TC 3010xxxx Series nominal 2.50-inch gauge shall be supplied for reading the pressure of each discharge greater than 1.50 inches in diameter, unless otherwise specified.

A KEM-X socket saver diaphragm, located in the stem, eliminates freeze-up by preventing water from entering and/or clogging the gauge internals while containing a low temperature instrument oil that fills and protects the socket and the bourdon tube.

The molded glass-filled Nylon 66 case will not corrode and includes a scratch-resistant molded polycarbonate lens with O-ring seal. The gauge shall withstand pressures up to 100psi over gauge range with operation from –40° F to +160°F.

Each gauge shall be marked for reading a discharge pressure of 0-400 PSI.

Each gauge shall have black markings on a white face.

BEZELS FOR 2.5" DISCHARGE GAUGES
There shall be a deluxe metal bezel supplied around each of the 2.50-inch discharge pressure gauges. The bezels shall be constructed from chrome-plated zinc with large, easily identifiable recessed labels for color-coding and verbiage.

FOAMPRO 1600

There shall be a fully automatic electronic direct injection foam proportioning system furnished and installed on the apparatus. The system shall be capable of Class A foam concentrate. The proportioning operation shall be based on an accurate direct measurement of water flows with no water flow restriction. The foam system shall be installed in accordance with the manufacturer's recommendations. The foam system shall have a 12-volt, 1/3 horsepower electric positive displacement foam concentrate pump with a rated capacity of .01 to 1.7 GPM with operating pressures up to 400 psi.

The system shall be manufactured by the Fire Research Corporation and be model FoamPro 1600.

The system shall be equipped with a control module. It shall be installed on the pump operators panel and enable the pump operator to perform the following functions:

- Activate the foam system

- Change foam concentrate proportioning rates from .1% to 1%

- Flash a "low concentrate" warning light when the foam concentrate tank runs low. In two (2) minutes if foam concentrate is not added to tank, the foam concentrate pump shall be deactivated.
The apparatus foam system shall be tested, and the Foam Flow meter shall be certified by the manufacturer prior to delivery.
The front bumper discharge and the two (2) 2.00-inch crosslay discharges shall be foam capable.

FOAM TANK LEVEL GAUGE

Fire Research TankVision Pro model WLA360-A00 tank indicator kit shall be installed. The kit shall include an electronic indicator module, a pressure sensor, a 20' sensor cable and a tank vent. The indicator shall show the volume of Class A foam concentrate in the tank on nine (9) easy to see super bright RGB LEDs. A wide view lens over the LEDs shall provide for a viewing angle of 180 degrees. The indicator case shall be waterproof, manufactured of Polycarbonate/Nylon material, and have a distinctive green label.

The program features shall be accessed from the front of the indicator module. The program shall support self-diagnostics capabilities, self-calibration, six (6) programmable colored light patterns to display tank volume, adjustable brightness control levels and a datalink to connect remote indicators. Low water warnings shall include flashing LEDs at 1/4 tank, down chasing LEDs when the tank is almost empty, and an output for an audio alarm.

The indicator shall receive an input signal from an electronic pressure sensor. The sensor shall be mounted from the outside of the foam tank near the bottom. No probe shall be placed on the interior of the tank. Wiring shall be weather resistant and have automotive type plug-in connectors.

FOAM TANK
There shall be a 20-gallon poly foam tank provided and installed with non-corrosive piping to the foam system. The tank shall be integral with the booster water tank.

A label shall be affixed to the foam tank fill indicating: “WARNING” Class A foam tank fill, do not mix brands or types of foam.

The system shall be supplied by a single foam tank.

There shall be a 1/4 turn valve installed at the foam tank to shut off the flow from the supply line.

There shall be a 1.00-inch quarter turn drain valve installed for drainage of the foam tank. The valve shall be installed in the pump house with a drain line extended to the side running board.

VIBRA-TORQ™ BODY MOUNTING SYSTEM
The entire body module assembly shall be mounted to the chassis frame rails exclusively with Vibra-Torq™ torsion isolator assemblies to reduce the vibration and stress providing an extremely durable body mount.

The body substructure shall be mounted above the frame to allow independent flexing to occur between the body and the chassis. Two (2) assemblies shall be mounted to the chassis frame rails with steel, gusseted mounting brackets. Each bracket shall be painted for corrosion resistance. Each body mount bracket shall be mounted to the side chassis frame flange with two 5/8”-UNC Grade 5 HHCS.

The rear assemblies shall have a two-part rubber vibration isolator. Certain assemblies shall also incorporate a torsion spring. Helical coil springs shall be incorporated into specific mounts in tandem with the rubber isolators to minimize the stress absorbed by the body caused from chassis frame rail flexing.

There shall be no welding to the chassis frame rail sides, web or flanges, or drilling of holes in the top or bottom frame flanges between axles. All body to chassis connections shall be bolted so that in the event of an accident, the body shall be easily removable from the truck chassis for repair or replacement.

Because of the constant vibration and twisting action that occurs in chassis frame rails and suspension, the torsion mounting system is required to minimize the possibility of premature body structural failure. The Vibra-Torq™ body mounting system shall have a lifetime warranty.  
COMPARTMENT VENTILATION
To allow for proper air circulation and flow, each compartment shall have a venting route. The venting locations shall be determined by "best-fit" locations for each body style configuration. The vents will be a chrome louvered and mounted appropriately on the compartment interior walls.

COMPARTMENTATION
The following compartments shall be supplied on the apparatus:

Compartment "L1"

There shall be one (1) full height compartment ahead of the rear wheels on the left side of the apparatus with interior dimensions of the following:

The upper portion shall be approximately 37.75 inches wide by 39.00 inches high by 12.50 inches deep. The lower portion shall be approximately 37.75 inches wide by 33.00 inches high by 26.00 inches deep.

Clear door opening dimensions shall be 29.70" wide by 63.10" high.

Compartment "L2"
There shall be one (1) compartment over the rear wheels on the left side of the apparatus with interior dimensions of the following:

The upper portion shall be approximately 68.00 inches wide by 39.00 inches high by 12.50 inches deep. The lower portion shall be approximately 68.00 inches wide by 8.00 inches high by 25.75 inches deep.

Clear door opening dimensions shall be 65.50" wide by 38.10" high.

Compartment "L3"

There shall be one (1) full height compartment behind the rear wheels on the left side of the apparatus with interior dimensions of the following:

The upper portion shall be approximately 46.75 inches wide by 39.00 inches high by 12.50 inches deep. The lower portion shall be approximately 46.75 inches wide by 33.00 inches high by 26.00 inches deep.

Clear door opening dimensions shall be 41.70" wide by 63.10" high.

Compartment "R1"

There shall be one (1) full height compartment ahead of the rear wheels on the right side of the apparatus with interior dimensions of the following:

The upper portion shall be approximately 37.75 inches wide by 39.00 inches high by 12.50 inches deep. The lower portion shall be approximately 37.75 inches wide by 33.00 inches high by 26.00 inches deep.

Clear door opening dimensions shall be 29.70" wide by 63.10" high.

In cases of chassis driven exhaust clearances issues, the rear lower back portion of the compartment shall be notched to accommodate.  This notch may be approx. 6.00 inches deep x 6.00 inches high x 6.00 inches long.  This may be reduced as required to allow for a minimal intrusion as possible. 

Compartment "R2"

There shall be one (1) compartment over the rear wheels on the right side of the apparatus with interior dimensions of the following:

The upper portion shall be approximately 68.00 inches wide by 39.00 inches high by 12.50 inches deep. The lower portion shall be approximately 68.00 inches wide by 8.00 inches high by 25.75 inches deep.

Clear door opening dimensions shall be 65.5" wide by 38.1" high.

Compartment "R3"

There shall be one (1) full height compartment behind the rear wheels on the right side of the apparatus with interior dimensions of the following:

The upper portion shall be approximately 46.75 inches wide by 39.00 inches high by 12.50 inches deep. The lower portion shall be approximately 46.75 inches wide by 33.00 inches high by 26.00 inches deep.

Clear door opening dimensions shall be 41.70" wide by 63.10" high.

Compartment "B1"
There shall be one (1) compartment installed at the rear of the apparatus.

The interior dimensions of this compartment shall be approximately 41.50 inches wide by 39.50 inches high by 33.25 inches deep.

Clear door opening dimensions shall be 33.50" (850.90 mm) wide by 31.80" (807.72 mm) high.

FORMED BODY DESIGN CONSTRUCTION
The apparatus body shall be a formed sheet metal design, which serves as the compartment enclosures and supporting substructure of the body. The substructure and enclosures shall work in unison to provide maximum storage that supports and protect the contents contained within.

BODY CONSTRUCTION
The body substructure and compartments shall utilize a combination of huck bolting and welding methods.

The huck bolt systems utilized in either body or substructure shall be .3125 inch or .375-inch diameter stainless steel fasteners for maximum shear and tension strength. Other system of fasteners that do not consist of stainless steel shall NOT be acceptable.

In combination with the huck bolt system, strictly monitored welding procedures shall be instituted.  To ensure maximum joint strength, any welding zones shall be welded together utilizing American Welding Standard (AWS), Certified welding procedures.

Due to the engineered combination of specifically chosen materials, no dissimilar metals shall be used in the body and its supporting substructure without being separated by a sufficient corrosion and electrolysis inhibitor. This shall consist of isolation pads and structural adhesives.

ECK® ANTI-CORROSION PROCESS
Absolutely no dissimilar metals shall be used in the body and its supporting substructure without being separated by Eck®, which prevents corrosion by providing a barrier between dissimilar metals, sealing out moisture and absorbing energy created by a dissimilar metal reaction.

BODY STRUCTURE
The supporting tank and compartment substructure shall be manufactured from corrosion resistant 304L stainless steel material. The supporting material shall be engineered from 7-gauge stainless steel material to provide both high strength and corrosion resistance for longevity of the apparatus body. The use of black carbon steel materials that have been painted or coated to try to prevent corrosion shall not be acceptable.  

BODY COMPARTMENTS
The formed sheet metal compartments shall utilize a 0.125 inch thick 5052-H32 aluminum alloy to provide maximum strength and durability. Each compartment sheet and enclosure shall be fabricated in a manor to provide proper sheet alignment and weld location application. The body shall consist of multiple pre-engineered compartment assemblies that shall be combined to create a series of body combinations. In the event of body damage, these assemblies shall allow for easier disassembly and assembly through the use of common tools and materials.

COMPARTMENT TOPS AND EXTERIOR HOSE BED WALL
The exterior compartment tops and outer hose bed walls shall consist of .125-inch embossed aluminum diamond plate material to provide both strength and pleasing appearance. The hose bed walls shall be embossed aluminum diamond plate to the outward face while incorporating an additional smooth aluminum interior wall sheet to form the hose bed area. The use of interior and exterior hose bed wall sheets shall provide an enclosed section for strength integrity, wire routing, etc. Single hose bed wall sheet construction shall NOT be acceptable.  

COMPARTMENT FLOORS
The body compartments shall be enclosed with aluminum sheet metal as specified above. The compartment floors shall have a 1.00-inch lip downward at the door opening side of the compartment. This lip shall integrate with a structural member on the bottom edge and form a “sweep-out” compartment. This design shall also allow for a structural flush fitting door frame and a complete door/weather seal.

COMPARTMENT LOAD CAPACITY
Each compartment shall have a minimum of one additional structural compartment floor support hat section centered on the underside of the compartment floor.  This additional member shall be integral with compartment assemblies of each area. Each compartment must be designed, and analyzed to carry a working load of:

Full depth side compartment: 500 pounds per compartment

Half depth side compartment: 375 pounds per compartment

Rear center compartment: 500 pounds 

ROLLUP COMPARTMENT DOORS
A R•O•M Corporation Series IV roll-up shutter door shall be installed on each compartment. Each shutter slat, track, bottom rail, and drip rail shall be constructed from anodized 6063 T6 aluminum.

Shutter slats shall feature a double wall extrusion 0.315 inches thick with a concave interior surface to minimize loose equipment jamming the shutter door closed. Shutter slats shall feature an interlocking end shoe to prevent side to side binding of the shutter door during operation. Slats must have interlocking joints with an inverted locking flange. Slat inner seal shall be a one piece PVC extrusion; seal design shall be such to prevent metal to metal contact while minimizing dirt and water from entering the compartment.

Shutter door track shall be one piece design with integral overlapping flange to provide a clean finished look without the need of caulk. Door track shall feature an extruded Santoprene rubber double lip low profile side seal with a silicone co-extruded back to reduce friction during shutter operation.

Shutter bottom rail shall be a one-piece double wall extrusion with integrated finger pull. Finger pull shall be curved upward with a linear striated surface to improve operator grip while operating the shutter door. Bottom rail shall have a smooth contoured interior surface to prevent loose equipment from jamming the shutter door. Bottom rail seal shall be made from Santoprene; it will be a double “V” seal to prevent water and debris from entering compartment. Bottom rail lift bar shall be a one piece “D” shaped aluminum extrusion with linear striations to improve operator grip during operation. Lift bar shall have a wall thickness of 0.125 inches. Lift bar shall be supported by no less than two pivot blocks; pivot blocks shall be constructed from Type 66 Glass filled reinforced nylon for superior strength. Bottom rail end blocks shall have incorporated drain holes which will allow any moisture that collects inside the extrusion to drain out.

Shutter door shall have an enclosed counterbalance system. Counterbalance system shall be 4.00 inches in diameter and held in place by 2 heavy duty 18-gauge zinc plated plates. Counterbalance system shall have 2 over-molded rubber guide wheels to provide a smooth transition from vertical track to counterbalance system.

SIDE COMPARTMENT DOOR/TRACK/TRIM WET PAINTED
The side compartment doors, track, and trim shall be aluminum finish and wet painted to color match the apparatus body.

REAR COMPARTMENT DOOR/TRACK/TRIM WET PAINTED
The rear compartment door, track and trim shall be aluminum finish and wet painted to color match the apparatus body.

ROLL-UP DOOR PROTECTORS
There shall be a protective cover installed under each body side compartment door roll to protect the door in the rolled-up position.

Each cover shall be fabricated of smooth aluminum and of natural finish.

ROLL-UP DOOR ASSIST STRAPS
There shall be nylon straps installed on both the left and right-side body side, 'high side' compartment doors, to assist in closing the door. The strap shall be attached to each door and shall be permanently mounted to the rearward wall with footman loops using Nutserts, halfway between the top and bottom of the compartment.

DOOR OPEN INDICATORS
Each roll up door shall have an integral door open indicator magnet in the lift bar. If the bar is not properly closed and the transmission is shifted out of neutral with the parking brake is released, it shall activate the hazard light in the cab to alert the crew.

COMPARTMENT LIGHTING
Two (2) OnScene Access LED strip lights shall be installed in each body compartment. The tube lights shall be centered vertically along each side of the door framing and shall be maximum length available to fit the opening. The lights in each compartment shall be on a separate circuit, turning on only those lights that have open compartment doors.  The lights shall not be tied into the park brake control.

HOSE STORAGE
A hose bed shall be provided that meets the minimum NFPA storage requirements. The hose bed shall have slotted .25-inch aluminum flooring installed to allow drainage through the tank cavity to the ground below. The aluminum flooring shall be manufactured in discrete sections to allow for easy removal and outstanding stability. The area shall be free of sharp edges to protect the hose when loaded or distributed.

The apparatus hose bed interior walls shall be incorporated with a brushed stainless steel overlay material.

HOSE BED DIVIDERS WITH HAND CUTOUT
There shall be two (2) full height adjustable hose bed dividers provided and installed in the hose bed area of the apparatus body.

Each divider shall be fabricated of .25 inch thick aluminum plate with a double sided reinforcement and attached to the adjustable slide rails. The rear of the divider shall have a radius to provide a smooth corner and a hand cut out to aid in access to the hose bed area. Hose payout shall be unobstructed by the divider.

HOSEBED RISER
A 15.00 inch hosebed riser made from the same material as the body shall be provided in order to increase the hosebed capacity. There shall be a red reflective stripe installed at the top of the hosebed riser sides. 

CATWALKS

Catwalks shall be provided over the top of the compartments. The catwalks shall be manufactured with .125-inch embossed aluminum diamond plate material.

The outboard edge shall be bent downward at a 90-degree angle and over the compartments on both sides.

Catwalks shall not be an approved stepping surface, "Do not walk" labels to be installed

D&S HYPALON HOSE BED COVER
There shall be a custom D&S hose bed cover provided and installed with the apparatus to cover the top of the hose bed area. The cover shall be manufactured from 22oz Hypalon material with a grab tensile strength of 500lbs. The cover shall be held in place by an elastic shock cord sewn into the tarp with brass grommets where the shock cord passes through the hose bed cover on the front and sides.

The cover shall be secured by extruded aluminum channel on the front and Velcro straps on the sides. The D&S hose bed cover shall be red.

REAR HOSE BED COVER
There shall be a custom D&S crosslay hose bed cover that extends down over the rear of the hose bed which shall be fastened by an elastic shock cord sewn into the cover with brass grommets where the shock cord passes through the hose bed cover.

Hooks shall be installed on the lower corners to provide a means of attaching the cover to the apparatus. The rear D&S hose bed flap shall be red.

LED HOSEBED FLOOD LIGHT
There shall be one (1) 46.00-inch OnScene "Access" LED strip lights installed on the hose bed dunnage wall.  The tube light shall be mounted at the top to shine into the hose load.

The hose bed light shall be activated when the park brake is set.

DUNNAGE AREA
A vertical bulkhead shall be installed at the front of the hose bed area, just behind the water tank fill tower, forming a storage area that is separated from the hose bed. The rear face of the bulkhead shall serve as a mounting surface for the hose bed dividers, resulting in the ability to move any hose bed divider across the entire width of the hose bed.

POLYPRENE TANK
The booster tank shall be constructed of .50-inch, .75 inch, and 1.00-inch-thick polypropylene sheet stock which is a non-corrosive stress relieved thermoplastic. It shall be designed to be completely independent of the body and compartments. All joints and seams are extrusion welded and/or contain the "Bent Edge" and tested for maximum strength and integrity. The top of the booster tank is fitted with lifting eyes designed with a 3 to 1 safety factor to facilitate tank removal.

The tank cover shall be constructed of .75-inch thick Polyprene and shall be recessed. A minimum of two lifting dowels shall be drilled and tapped .50-inch x 2.00 inch to accommodate the lifting eyes.

The swash partitions are manufactured of .50-inch Polyprene. All partitions are equipped with vent and air holes to permit movement of air and water between compartments to provide to provide maximum water flow. All swash partitions interlock and are welded to one another as well as to the walls of the tank.

The tank shall rest on the sub-frame cross members with an unsupported area not to exceed 530 square inches on tanks up to 40.00 inches in height. On tanks over 40.00 inches in height, an unsupported area of not more than 400 square inches must be maintained. 

All tanks shall be isolated from those cross members with a minimum of 2.00-inch x .25-inch hard rubber strips that are 60 durometers in hardness. The tank shall sit cradle mounted in the under body sub-frame and shall be completely removable without disturbing the body side panels.

The tank shall be 1000 gallons in capacity.

The fill tower base shall measure approximately 14.00 inches long x 14.00 inches wide and incorporate an "anti-surge" baffle inside the tower and the opening shall be approximately 14.00 inches x 14.00 inch. The tower will have a .25-inch thick removable Polyprene screen and a Polyprene hinged type cover that will open if the tank is filled at an excess rate. There shall be a removable .25-inch thick Polyprene screen to prevent debris from falling into the tank. The fill tower shall have a 6.00-inch overflow that will discharge underneath the tank, behind the rear axle, avoiding the chassis fuel tank and suspension components where applicable.  The overflow shall terminate above the tank water level when filled to the rated capacity.

DIRECT TANK FILL
There shall be one (1) external direct tank fill port installed on the right rear of the apparatus.

A 2.50-inch Akron Brass 8000 series swing-out valve with stainless steel ball shall be provided for the direct tank fill.

The valve shall be controlled with a 'swing-type' lever directly attached to the valve. The lever shall operate just over 90 degrees of travel to provide full open/full closed positioning of the valve.

The plumbing shall consist of 2.50-inch piping and shall incorporate a manual drain control installed below the pump area for ease of access.

The direct tank fill termination shall include the following components: one (1) 2.50-inch NST female swivel 30-degree elbow adapter and one (1) 2.50-inch NST male self-venting plug, secured by a cable to the outlet termination location.

LADDER STORAGE
The ground ladders shall be stored within a compartment installed on the right side of the apparatus booster tank, with ladders lying on their side. There shall be storage for two (2) pike poles and a slot for a back board integrated into the compartment.  There shall be a non-metal ladder stop to prevent metal to metal contact.  There shall be non-metallic guides installed for ladders to slide on. All items shall be stored in their own independent section to allow one item to be removed without disturbing another.

The compartment and door shall be fabricated of .125-inch smooth aluminum. The door shall be vertically hinged on the inboard edge (left side) and provided with two push button style latches and a chrome handle centered between the push button latches. The door shall have retro-reflective striping in a chevron pattern. If the door is not properly closed and the transmission is shifted out of neutral with the parking brake is released, it shall activate the hazard light in the cab to alert the crew.

GROUND LADDERS
The following ground ladders shall be provided by the manufacturer:

-One (1) Duo-Safety 24-foot two (2) section aluminum extension ladder, model 900A.

-One (1) Duo-Safety 14-foot aluminum roof ladder with folding hooks, model 775A.

-One (1) Duo-Safety 10-foot aluminum attic ladder, model 585A.
SUCTION HOSE STORAGE

The suction hoses shall be stored on a formed carrier racks sized to hold 6.00-inch x 10.00-foot hose. Each tray shall be constructed of anodized aluminum for a durable, long-lasting finish. Each rack shall have two (2) Velcro hold-down straps, one (1) at each end, which shall secure the suction hose to the tray.

There shall be two (2) carriers installed, one (1) on each side of the apparatus, above the compartments.

SUCTION HOSE & STRAINER

There shall be two (2) 10.00-foot lengths of 6.00-inch clear PVC suction hose with lightweight couplings supplied and installed on the apparatus by the manufacturer.

One (1) Kochek Model #BS60-KBR barrel strainer provided with the apparatus. The strainer shall be constructed from aluminum with K-Brite finish and include a tie off loop on the end plate. 

BODY OVERLAYS – FRONT/REAR
The entire front face of the apparatus body shall have aluminum diamond plate overlays installed. The entire rear face of the apparatus body shall have raw aluminum overlays installed for the installation of chevron striping.

All overlay materials shall be coated with 3M adhesive sealant on the back portion to provide an insulating barrier between dissimilar metals.

WHEEL WELL ROLL-OUT DRAWER
There shall be a roll-out drawer installed in the compartment located above the rear wheels on the left side of the body in the L-2 compartment. The slide assemblies shall incorporate cadmium plated ball bearing roller slides and a lock-in, lock-out front drawer release system (FDR).

The drawer shall be approximately 25.00-inch deep by 63.15 inch wide with 2.75-inch-tall sides and have a 300.00-pound capacity.

SCBA COMPARTMENT BIN
There shall be an eight (8) place air bottle compartment bin provided in the lower portion of the compartment located above the wheel well area on the right side in the R-2 compartment.

The interior surface of each SCBA storage tube shall be lined with a finish matching the compartment interior. The finish application shall aid to minimize any damage caused to the canisters while stored in the holders.

The NFPA required SCBA bottle straps shall be mounted deeper in the compartment, so the bottles do not hit the door when the door is closed.

OVERWHEEL SHELVING
One (1) shelf 66.25-inch-wide x 11.50-inch-deep x 2.00-inch high shall be provided in the left side wheel well compartment. The shelf shall be .19-inch smooth aluminum with a formed 2.00-inch lip on the front and back. The side mounting brackets shall be integral with the shelf to form the sides.

One (1) shelf 66.25-inch-wide x 11.50-inch-deep x 2.00-inch high shall be provided in the right-side wheel well compartment as part of the SCBA storage assembly. The shelf shall be .19-inch smooth aluminum with a formed 2.00-inch lip on the front and back. The side mounting brackets shall be integral with the shelf to form the sides.

COMPARTMENT UNISTRUT
Vertically mounted Unistrut shall be installed in ALL compartments of the apparatus body to accommodate mounting shelves, trays, and other miscellaneous equipment items as specified.

COMPARTMENT UNISTRUT
Two (2) horizontally mounted Unistrut tracks shall be provided on the back wall in each over wheel compartment.

SHELVING
The shelving shall be made out of .190-inch smooth aluminum sheet material with a formed 2.00-inch lip on the front and back.

The side mounting brackets shall be integral with the shelving to form the sides.  The shelving shall be vertically adjustable.

The following shelving shall be provided:

UPPER HALF DEPTH SHELVING

A full width x half depth shelf shall be provided and installed in the upper compartments specified:
One (1) shall be located in the L-3 compartment.

One (1) shall be located in the R-1 compartment.

One (1) shall be located in the R-3 compartment.

The shelves specified above shall be reduced in depth by 1.00 inch for installation of wall mount tool boards.

FULL DEPTH SHELVING

A full width x full depth shelf shall be provided and installed in the compartments as specified:
One (1) shall be located in the L-1 compartment.
One (1) shall be located in the L-3 compartment.

One (1) shall be located in the R-1 compartment.

One (1) shall be located in the R-3 compartment.

One (1) shall be located in the B-1 compartment.

FLOOR MOUNT ROLL-OUT DRAWERS
There shall be a roll-out drawer installed in the compartments as specified:

 The slide assemblies shall incorporate cadmium plated ball bearing roller slides and a lock-in, lock-out front drawer release system (FDR).

The drawer shall be approximately 25.00-inch deep by 63.15 inch wide with 2.75-inch-tall sides and have a 300.00-pound capacity.
One (1) shall be located in the L-3 compartment.

One (1) shall be located in the R-1 compartment.

One (1) shall be located in the R-3 compartment.

One (1) shall be located in the B-1 compartment.

SHELF AND TRAY FINISH

Any shelf or roll-out tray installed shall have a dual-action sanded finish applied on the front and side faces.

WHEEL WELL PANELS

The body panel area around the wheel well on each side of the body shall be painted the same color as the rest of the body

SIDE RUB RAILS
The bottom edge of the body compartments and pump compartment shall be protected with rub rails to absorb minor damage while protecting the body. The rear rub rails shall be full length to the end of the tailboard.

The rub rails shall be fabricated of brightly anodized aluminum channel. The rub rails shall be bolted in place with stainless steel bolts and shall be spaced away from the body with .50-inch nylon spacers to help prevent the collection of water and debris. Each rub rail section shall be easily removable and replaced should it become damaged.

REAR RUB RAILS
The rearward edge of the rear step shall be trimmed with rub rails to absorb minor damage while protecting the body.

The rub rails shall be fabricated of brightly anodized aluminum channel. The rub rails shall be bolted in place with stainless steel bolts and shall be spaced away from the body with .50-inch nylon spacers to help prevent the collection of water and debris. Each rub rail section shall be easily removable and replaced should it become damaged.

RUB RAIL RETRO-REFLECTIVE STRIPING
One inch retro-reflective Diamond Grade striping shall be applied to the length of each rub rail section making the perimeter of the apparatus more readily visible.

The reflective striping shall be red in color.

DOOR SILL TRIM PLATES

Brushed stainless steel door sill plates shall be installed at the bottom of each body compartment door opening.

VERTICAL OVERLAY TRIM PLATES

Full height brushed stainless steel vertical overlay trim plates shall be installed on the outer corners of the front and back body compartments.

FENDERETTES
Two (2) polished aluminum fenderettes shall be provided and installed on body rear wheel well openings, one (1) each side. Rubber welting shall be provided between the body and the crown to seal the seam and restrict moisture from entering. A dielectric barrier shall be provided between the fender crown fasteners (screws) and the fender sheet metal to resist deterioration.

REAR TAILBOARD
The rear tailboard shall be fabricated of the same materials as used in the apparatus body. The tailboard shall be an independent assembly fastened to the rear body structural framing to provide body protection and a solid rear stepping platform.

The rear of the apparatus body shall be vertical in design - otherwise known as a 'flat-back'. On the rear body surface, a sign shall be attached that states: "DO NOT RIDE ON REAR STEP, DEATH OR SERIOUS INJURY MAY RESULT."

The rear tailboard and body shall be constructed such that the angle of departure shall be no less than 8 degrees at the rear of the apparatus when fully loaded (Per NFPA, current edition).

The rear tailboard shall be approximately seventeen and one half (17.50) inches deep and shall incorporate a .125-inch embossed aluminum diamond plate overlay.

The stepping area shall span the width of the apparatus, overlapping the perimeter of the structural tailboard framework.

The embossed diamond plate material shall meet the minimum NFPA standard requirements for slip resistance.

INTERMEDIATE REAR STEP
The rear step shall be 53.38 inches wide by 10.00 inches in depth. There shall be four (4) handhold cutouts provided in the top step surface measuring approximately 3.00 inches deep. There shall be one (1) full length aluminum non lit handrail integrated into the assembly.

The step shall be mounted on the flat back of the apparatus with gusset-type mounting and 5/16" bolts to provide sufficient support for loading hose and gaining access to the hose bed area.

The platform stepping surface shall be constructed of .188-inch embossed aluminum diamond plate materials.

Two (2) Whelen OS lights shall be installed to illuminate the stepping area.  Additionally, there shall be one (1) On-Scene Access 38" LED tube light installed below the intermediate step to illuminate below the step.

The step lights shall be activated when the park brake is set.

FOLDING STEPS 

Innovative Controls Inc. model #3004234 steps, made of high strength die cast aluminum, conforming to current NFPA requirements, shall be provided and installed on the apparatus as specified. 

The steps shall include a molded gasket and drain at the bottom to allow any water to escape the assembly, preventing water ingress and keeping the mount from damaging painted surfaces.

The steps shall be mounted with 5/16" bolts. 

The steps shall have a minimum of 46 sq. inches of surface area capable of sustaining a 1200 lb. static load. The steps shall be mounted no more than 18" inches between each step.

ILLUMINATED FOLDING STEPS-LF
Three (3) illuminated folding steps shall be installed on the left front vertical face of the body.

The step lights shall be activated when the park brake is set.

One (1) handrail constructed of knurled Aluminum tubing shall be installed to assist in climbing the steps according to NFPA 1901, current edition. There shall be a 2.00-inch minimum clearance between the bracket and the body.

Location:  Front edge of catwalk, angled at approximately 30 degrees.

ILLUMINATED FOLDING STEPS-RF
Three (3) illuminated folding steps shall be installed on the right front vertical face of the body.

The step lights shall be activated when the park brake is set.

One (1) handrail constructed of knurled Aluminum tubing shall be installed to assist in climbing the steps according to NFPA 1901, current edition. There shall be a 2.00-inch minimum clearance between the bracket and the body.

Location:  Front edge of catwalk, angled at approximately 30 degrees.

ILLUMINATED FOLDING STEPS-RR
Three (3) illuminated folding steps shall be installed on the right rear vertical face of the body.

The step lights shall be activated when the park brake is set.

HANDRAILS
Two (2) full height vertical handrails shall be mounted, one (1) on each side of the rear center compartment area of the rear of the apparatus.   The vertical rear of body handrails shall be mounted with offset stanchions.

One (1) 50" horizontal hand rail shall be installed above the intermediate step.

Two (2) 20" handrails shall be installed parallel to the body on top the pump house, rear of the crosslays, one (1) each side.

REAR TOW EYES
There shall be two (2) rear tow eyes installed on the rear sub frame support structure, one each side. The location of the tow eyes shall be below the rear center compartment. The tow eyes shall be manufactured of 1.00-inch plate steel that is bolted to the chassis frame rail with a minimum of 6 grade 8 bolts.

PAINT SPECIFICATIONS

All bright metal fittings, if unavailable in stainless steel, shall be heavily chrome plated.

Critical body and sub-frame area which cannot be primed after assembly shall be pre-painted.

All welded metal surfaces shall be ground to a smooth surface prior to a degreasing and high pressure, high temperature phosphatizing process. The entire surface shall be sprayed with a non-chromate sealing compound to prevent formulation of stains or flash rust on previously phosphatized parts.

The paint applied to the apparatus shall be Akzo Nobel, Sikkens brand, LVBT650 basecoat, applied throughout a multi-step process including at least two coats of each color and clear coat finish.

The coating shall be an infra-red, baked air dried. The coatings shall provide full gloss finished suitable for application by high-pressure airless or conventional low pressure air atomizing spray.

The coatings shall not contain lead, cadmium or arsenic. The polyisocyanate component shall consist of only aliphatic isocyanates, with no portion being aromatic isocyanates in character. The solvents used in all components and products shall not contain ethylene glycol mono-ethyl ethers or their acetates (commercially recognized as cello solves), nor shall they contain any chlorinated hydrocarbons. The products shall have no adverse effects on the health or nor present any unusual hazard to personnel when used according to manufacturer's recommendations for handling and proper protective safety equipment, and for its intended use.

The coating system, as supplied and recommended for application, shall meet all applicable federal, state and local laws and regulations now in force or at any time during the courses of the bid.

The manufacturer shall supply (upon request) for each product and component of the system, a properly complete OSHA "Safety Data Sheet".

The following documents of the issue in effect on the date of the invitation to quote form a part of this document to the extent specified herein:

Federal Standards: Number 141A and 141B paint, varnish, lacquer and related material: methods of inspection, sampling, and testing.

Military Standard: MIL-C 83486B Coating, Urethane, Aliphatic Isocyanates, for Aerospace applications.

Industry Methods and Standards: ASTM Method of Analysis (American Society for testing and Materials). BMS 10-72A (Boeing Material Specifications).

The entire exterior body structure (excluding roll-up doors) shall receive the primer coats and the finish coats. The apparatus body will be painted in a down draft type paint booth to reduce dust, dirt or impurities in the finish paint. The painted surfaces shall have a finish with no runs, sags, craters, pinholes or other defects. The coating will meet the following test performance properties as a minimum standard.

The apparatus shall be painted Sikkens FLNA 32528 Red

SPEEDLINER COMPARTMENT FINISH
The compartment interiors shall be coated with bed liner type spray. The Superliner Color shall be Medium Gray.

LOW-VOLTAGE ELECTRICAL SYSTEM
The apparatus shall be equipped with a Weldon Logic Controlled, Low-Voltage (12v) Electrical System compliant with the latest revision of the NFPA 1901 guideline.

The system shall be capable of performing total load management, load management sequencing, and load shedding via continuous monitoring of the low-voltage electrical system.  In addition, the system shall be capable of switching loads (like operating as an emergency warning lamp flasher) eliminating the dependency on many archaic electrical components such as conventional flasher modules. The system shall also incorporate provisions for future expansion or modification.

The low-voltage electrical system shall be designed to distribute the placement of electrical system hardware throughout the apparatus thereby enabling a smaller, optimized wire harness.   The programmable, logic-controlled system shall eliminate redundant electrical hardware such as harnesses, circuit boards, relays, circuit breakers, and separate electrical or interlock subsystems and associated electronics for controlling various electrical loads and inputs. 

As-built electrical system drawings and a vehicle-specific reference of I/O shall be furnished in the delivery manuals. These drawings shall show the electrical system broken down into separate functions, or small groups of related functions. Drawings shall depict circuit numbers, electrical components and connectors from beginning to end. A single drawing for all electrical circuits installed by the apparatus builder shall not be accepted.

LED PERIMETER LIGHTS
There shall be six (6) LED TecNiq model T44 series, 4.00-inch round, 8 diode LED lights installed on the apparatus, one (1) under each side at the front of the body, one (1) under each side at the rear of the body and one (1) each side under the rear tailboard. The lights shall be positioned to provide illumination to the immediate ground area around the unit.

The underbody perimeter lights shall be activated with activation of the chassis ground lights.

LED DOT LIGHTING

There shall be seven (7) lights located on the rear of the vehicle. Three (3) of the lights shall be mounted on the upper rear face of the body just below the hosebed area in a cluster for use as identification lamps. Two (2) lights shall be located outboard on the upper rear, one each side for use as clearance lamps and two (2) lights in the rearmost position of the side rubrail on the tailboard facing the side for use as rear side marker lamps

The lights shall be Weldon brand 9186-1500 series LED red markers

DOT ADDITIONAL MARKER LIGHTS

There shall be two (2) amber LED intermediate marker lights/intermediate turn signals installed in the rub rail, forward of the rear wheel well, one (1) each side.

The lights shall be Weldon brand 9186-1500 series LED amber markers/turn.

The intermediate amber side marker lights installed in the rub rail, forward of the rear wheel well, shall flash when their respective side turn signal is activated. The lights shall return to steady burn when turn signal is deactivated.

UPPER LIGHTING PACKAGE
The following NFPA lighting package, manufactured by Whelen, shall be supplied and installed in the upper areas of the vehicle.

The body shall have the warning light dimming feature when the chassis is equipped with it. If the dimming feature is not provided on the chassis, the body WILL NOT be equipped. 

UPPER ZONE C:

There shall be two (2) Whelen beacons with 360 degree super LED lights with polished base, provided and installed at the rear upper outboard corners of the apparatus, one (1) each side.

The beacons shall be B63 Series with R316 Super-LED beacons with blue lights and clear lenses and two (2) M7 series LED amber flashing lights with clear lenses surface mounted below the beacons.

The upper ZONE C specified lights shall be mounted directly to the horizontal body surface as far rearward as possible.

UPPER REAR WARNING LIGHT SWITCH E-MASTER/VISTA

The upper rear warning lights shall be controlled through the master warning switch and a secondary rear warning switch located on the Vista display control screen. The switches shall be clearly labeled for ease of identification.

AUXILIARY UPPER ZONE WARNING LIGHTS

There shall be four (4) Whelen WION lights with chrome bezels installed, two (2) on each side.   One (1) light shall be located in the upper section of the side compartment header centered above the forward and rearmost compartments.

SIDE WARNING LIGHTS FLASH
The lights shall feature multiple flash patterns including steady burn for solid colors and multiple flash patterns for split colors.

The warning lights shall be red with clear lenses.

LOWER ZONE LED WARNING LIGHTING
The following NFPA lighting package, manufactured by Whelen, shall be supplied and installed in the lower areas of the vehicle.

The body shall have the warning light dimming feature when the chassis is equipped with it. If the dimming feature is not provided on the chassis, the body WILL NOT be equipped. 

LOWER ZONE B&D:
There shall be four (4) Whelen model M6 series LED lights with chrome bezels, two (2) each side, provided and installed with the apparatus.

The lower side lights shall feature multiple flash patterns including steady burn for solid colors and multiple flash patterns for split colors. The lights shall be programmed to emit the "TripleFlash 75 in/out" solid flash pattern.

The lower side warning lights mounted on the side positions shall be red with clear lenses.

The warning lights on the side of the apparatus shall be mounted at the mid height in the forward area of the rear wheel well panel and at the rear tailboard location.

LOWER ZONES B&D CAST ALUMINUM LIGHT HOUSING WITH PAINTED INSERT
A cast aluminum light housing with painted outward facing inserts, shall be installed for the rearmost warning light in zones B&D. The housing will ensure the light is mounted as far rearward as possible.

The inserts shall be painted to color match the body.

LOWER SIDE WARNING LIGHT SWITCH E-MASTER/VISTA

The lower side warning lights shall be controlled through the master warning switch and a secondary side warning switch located on the Vista display control screen. The switches shall be clearly labeled for ease of identification.

LOWER ZONE C:
There shall be two (2) Whelen model M9 series Super-LED lights with chrome bezels, one (1) each side, on provided and installed on the rear of the body.

The lower rear lights shall feature multiple flash patterns including steady burn for solid colors and multiple flash patterns for split colors. The lights shall be programmed to emit the "Triple Flash 75 in/out" flash pattern.

The lower rear warning lights mounted at the rear shall be red with a clear lens on the left side and blue with a clear lens on the right side.

REAR WARNING LIGHT ACTIVATION

The rear lower warning lights shall be activated by the master warning switch, and individually switched by a virtual switch on the vista screen in the cab by a "lower rear warning lights" switch.

LED REAR TAIL LIGHT ASSEMBLY
There shall be Whelen M9-Series Super LED rear tail light assemblies provided and installed with the apparatus, one (1) each side at the rear.

The following shall be installed in the order as specified from top to bottom:

One (1) Warning light called out above

One (1) #M9BTT LED red brake light

One (1) #M9T LED series amber turn signal light

One (1) #M9 BUW LED clear backup light

There shall be individual chrome bezels provided for each light of the taillight assembly.

REAR DIRECTIONAL LIGHTBAR
There shall be eight (8) rear directional lights provided and installed on the rear of the apparatus body.

The lights shall be Whelen model #WIONSMCA LED ION amber lights with clear lenses and chrome bezels. Each light shall be surface mounted and equally spaced, spanning horizontally across the rear of the apparatus.

The lights shall be controlled by a Whelen #TACTL5 control head.

The rear direction light bar controller shall be chassis provided and installed.

REAR VIEW CAMERA SYSTEM
The chassis provided camera shall be surface mounted under the intermediate step, on the center rear of the apparatus body for maximum viewing capability.

REAR SCENE LIGHTING

There shall be two (2) scene lights installed at the rear body panels, one (1) on each side.

The scene lights shall be Whelen model #M9LZC 12-volt scene lights with chrome bezels. The lights shall offer LED directional lighting from 2 to 40-degrees with internal and external optics.

The lights shall draw 6.0 amps and generate 6,500 lumens.

The rear scene lights shall be activated by one (1) virtual button on the Vista display control screen, one (1) switch on the officer switch panel, one (1) switch on the pump panel, and when the apparatus transmission is shifted into reverse.

The switch shall be labeled as follows: Rear Scene

REFLECTIVE STRIPING
The reflective stripe applied to the outside perimeter of the chassis and apparatus by the Fire Department or the Dealership prior to the truck being placed into service.

REAR RETRO-REFLECTIVE CHEVRON STRIPING
The rear of body (excluding rear door) shall be equipped with Diamond Grade, retro-reflective striping in a chevron pattern, sloping downward and away from the centerline of the vehicle at an angle of 45-degrees.

The stripe shall be 6.00-inch-wide alternating in colors in compliance with the current edition of (NFPA) 1901, Standard for Automotive Fire Apparatus.

Diamond Grade retro-reflective chevron striping shall also be applied to the front bumper.

The retro-reflective chevron striping shall be red and fluorescent yellow-green in color.

BODY LETTERING

The lettering shall be provided and installed on each side of the apparatus body by the Fire Department or the Dealership.

LICENSE PLATE BRACKET
A Cast Products, model LP0005-1-C, cast aluminum open bottom license plate bracket shall be installed on the apparatus.

The bracket shall incorporate a clear LED (WL0501) light to illuminate the license plate to meet DOT requirements.

The above specified license plate bracket shall be installed at the back of the apparatus on the right side. The bracket shall be mounted to meet all applicable DOT standards.

WHEEL CHOCKS
One (1) set of NFPA compliant wheel chocks shall be provided and installed by the Fire Department or Dealership before the truck is placed into service.

NFPA LOOSE EQUIPMENT

The Fire Department shall be responsible to provide all NFPA loose equipment.

PAGE  
1

